| 3GPP TS 38.340 V0.0.0 (2019-10) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Backhaul Adaptation Protocol (BAP) specification (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Abbreviations	6

4	General	6

4.1	Introduction	6

4.2	Architecture	6

4.2.1	BAP structure	6

4.2.2	BAP entities	6

4.3	Services	6

4.3.1	Services provided to upper layers	6

4.3.2	Services expected from lower layers	6

4.4	Functions	6

5	Procedures	7

5.1	BAP entity handling	7

5.1.1	BAP entity establishment	7

5.1.2	BAP entity release	7

5.2	Data transfer	7

5.2.1	Transmitting operation	7

5.2.1.1	General	7

5.2.1.2	Routing	7

5.2.1.3	Bearer mapping	7

5.2.1.4	Routing identity selection	7

5.2.2	Receiving operation	7

5.3	Flow control feedback	7

5.4	Backhaul RLF indication	7

5.4.1	Transmitting operation	7

5.4.2	Receiving operation	7

5.5	Handling of unknown, unforeseen, and erroneous protocol data	7

6	Protocol data units, formats, and parameters	7

6.1	Protocol data units	7

6.1.1	Data PDU	7

6.1.2	Control PDU	7

6.2	Formats	7

6.2.1	General	7

6.2.2	Data PDU	7

6.2.3	Control PDU	7

6.3	Parameters	7

Annex A (informative): Change history	8

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

1	Scope

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

4	General

4.1	Introduction

4.2	Architecture

4.2.1	BAP structure

4.2.2	BAP entities

4.3	Services

4.3.1	Services provided to upper layers

4.3.2	Services expected from lower layers

4.4	Functions

5	Procedures

5.1	BAP entity handling

5.1.1	BAP entity establishment

5.1.2	BAP entity release

5.2	Data transfer

5.2.1	Transmitting operation

5.2.1.1	General

5.2.1.2	Routing

5.2.1.3	Bearer mapping

5.2.1.4	Routing identity selection

5.2.2	Receiving operation

5.3	Flow control feedback

5.4	Backhaul RLF indication

5.4.1	Transmitting operation

5.4.2	Receiving operation

5.5	Handling of unknown, unforeseen, and erroneous protocol data

6	Protocol data units, formats, and parameters

6.1	Protocol data units

6.1.1	Data PDU

6.1.2	Control PDU

6.2	Formats

6.2.1	General

6.2.2	Data PDU

6.2.3	Control PDU

6.3	Parameters

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019.10 | RAN2#107bis | R2-1914006 |  |  |  | skeleton | 0.0.0 |