3GPP TSG-RAN WG2 Meeting #107bis	R2-1914064

Chongqing, China, 14-18 October 2019

Title:	LS on SFN LSB indication in msg2/msgB

# Release:	Rel-16

Work Item:	NR_unlic-Core, NR_2step_RACH-Core

Source:	RAN2

To:	RAN1

# Contact Person:

# Name:	Ozcan Ozturk

E-mail Address:	oozturk@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1. Overall Description:

In the previous RAN2 meeting of RAN2#107, it was agreed to “support extension of RAR window without modifying RA-RNTI” and to “include LSBs of SFN in msg2” for NR-U 4-step RACH.

The reason for extension of RAR window in NR-U was due to the consideration of LBT failures for msg2 transmission. This extension will cause the same RA-RNTI to be used for msg2 which corresponds to msg1 transmissions in different SFNs. The LSBs of SFN in msg2 indicates the SFN of time when msg1 is transmitted for this msg2. The benefit of using bits in the DCI compared to including them in the msg2 payload is UE power saving and reduced UE complexity by not having to decode msg2 payload which correspond to other SFNs where msg1 transmissions happened.

RAN2#107bis has further discussed the signaling of SFN LSB during the online session for NR-U and agreed on the following for NR-U:

Agreements:

-	From RAN2 point of view it is beneficial to include LSB of SFN in the DCI.  The same design is desirable to be used for 2-step RACH.   Write LS to RAN1 to ask if there is any feasibility issues.

-	For NR-U, 2 bits are enough for a maximum of 40ms response windows.

-	Multiplexing of responses for more than one SFN is not allowed.

RAN2 agreement on including the 2 LSBs of SFN in DCI was based on the assumption that the reserved bits available in the DCI addressed to RA-RNTI for msg2 transmission can be used for this purpose. In addition, RAN2 assumed that the DCI for msgB scheduling in 2-step RACH can also support this and this is the desirable option per the above agreements.

# RAN2 respectfully asks RAN1 to provide feedback on the inclusion of 2 LSBs of SFN in DCI used for scheduling of msg2 in 4-step RACH and msgB in 2-step RACH on the unlicensed carrier.

# 2. Actions:

To RAN1

ACTION: 	.

# RAN2 respectfully asks RAN1 to provide feedback on the inclusion of 2 LSBs of SFN in DCI used for scheduling of msg2 in 4-step RACH and msgB in 2-step RACH on the unlicensed carrier.

# 3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #108			18-22 November 2019	Reno, NV, USA

TSG RAN WG2 Meeting #109			24-28 February 2020	Athens, Greece