3GPP TSG-RAN WG2 #107bis	R2-1914149

Chongqing, P.R. China, 14th – 18th October 2019

Title:	LS on SL RLF handling

Release:	Rel-16

Work Item:	5G_V2X_NRSL-Core

Source:	RAN2

To:	SA2

Cc:	-

Contact Person:

#### Name:	Antonino Orsino

E-mail Address:	antonino.orsino@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 has discussed the NR sidelink RLF handling, comprising UE and network actions. According to this, RAN2 would like to inform SA2 that the following agreements have been reached:

- In case of SL RLC AM, RLF declaration is triggered by indication from RLC that the maximum number of retransmissions has been reached.

- RLF triggering condition based on indication by physical layer is supported (pending RAN1/RAN4 progresses on the topic).

- The RLM/RLF procedure only apply to NR SL unicast.

- In case of RRC_CONNECTED/INACTIVE/RRC_IDLE/Out-of-coverage UEs, upon SL RLF declaration (e.g., expiring of timer T310) the UE releases the PC5-RRC connection immediately and sends an indication to upper layers.

- For RRC_CONNECTED UEs, upon SL RLF declaration (e.g., expiring of T310), the UE informs NW via Sidelink UE Information. FFS if we need explicit failure indication in Sidelink UE information or if it’s enough for the UE to inform it by excluding the corresponding destination L2 id.

- Measurement results are not included in Sidelink UE Information at RLF.

- A new timer (e.g., similar to T310) is specified for SL RLF handling (pending RAN1/RAN4 progresses on the topic).

- No need to specify a release procedure over the PC5-RRC at least at RLF.

2. Actions:

To: SA2

ACTION: 	 RAN2 kindly ask SA2 to take the abovementioned agreements into account.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #108	2019-11-18 to 2019-11-22	Reno, Nevada, US

TSG-RAN WG2 Meeting #109	2020-02-24 to 2020-02-28	Athens, Greece