3GPP TSG RAN WG2#107bis	R2-1914191

Chongqing, China, 14 – 18 October, 2019

Title:	LS on differentiating between MSG2 and MSGB

Response to:

Release:	Rel-16

Work Item:	NR_2step_RACH-Core

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

#### Name: 	Eswar Vutukuri

Tel. Number:	+44 7722129788

E-mail Address:	eswar.vutukuri@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

As part of the 2-step RACH work item, RAN2 has made a number of agreements that result in the MSGB format to be different to the format of the Rel-15 RAR (MSG2). Some of the relevant agreements are copied below:

- The following fields can be included in the successRAR when CCCH message is included in msgA. a. Contention resolution ID b. C-RNTI c. TA command - FallbackRAR should contain the following fields a. RAPID b. UL grant (to retransmit the msgA payload). FFS on restrictions on the grant and UE behavior if different grant and rebuilding c. TC-RNTI d. TA command => SuccessRAR and fallbackRAR can be multiplexed => The fallback RAR shall be included in the general MsgB format, ie., be able to be multiplexed with the successRAR for 2-step RACH.

As a result, RAN2 agreed that MSGB should be transmitted in such a way that the legacy UEs shall not decode MSGB and to achieve this, the following alternatives are considered by RAN2:

Alt 1: MSGB using a different RNTI than MSG2 (note this is a not a UE-specific RNTI)

Alt 2: MSGB using a different search-space and/or coreset than MSG2 (i.e. the formula to calculate the RNTI for MSGB will be same as that of MSG2)

RAN2 notes that Alt1 will consume more of the available RNTI space for random access response. Hence, RAN2 would like to understand whether Alt2 is feasible from RAN1 perspective to be able to make the final decision on the way forward for this issue.

2. Actions:

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to take the above into account and answer the following question:

Question to RAN1: Is the specification of Alt2 above feasible from RAN1 perspective?

3. Date of Next TSG-RAN2 Meetings:

RAN2#108 		18-22 November 2019 		Reno, Nevada, US

RAN2#109		24-28 Feb 2020			Athens, GR