3GPP TSG-RAN WG2 Meeting #107bis	R2-1913220

Chongqing,  China, 14 – 18 October 2019	Revision of R2-1910619

Agenda item:	6.13.3

Source:	Nokia, Nokia Shanghai Bell

Title:	New TAC MAC CE for 2-step RACH

WID/SID:	NR_2step_RACH-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In the RAN2#106 meeting, the following details for contention resolution in case of CONNECTED mode UE attempting 2-step RA was agreed:

Agreements - For MsgA with C-RNTI, the UE shall monitor the PDCCH addressed to C-RNTI for success response and msgB-RNTI (e.g. RA-RNTI or new RNTI) - Contention resolution: - If the PDU PDCCH addressed to the C-RNTI (i.e. C-RNTI included in MsgA) containing the 12 bit TA command is received, the UE should consider the contention resolution to be successful and stop the reception of MsgB or with UL grant if the UE is synchronized already. - If the corresponding fallback RAR is detected, the UE should stop the monitoring of PDCCH addressed to the corresponding C-RNTI for success response and process the fallback operation accordingly. - If neither corresponding fallback RAR nor PDCCH addressed C-RNTI is detected within the response window, the UE should consider the msgA attempt failed and do back off operation based on the backoff indicator if received in MsgB. - FFS if a new MAC CE with 12bits Timing Advanced Command shall be introduced

In this contribution we discuss the remaining FFS point above.

# 2	FFS on new MAC CE with 12bits TAC

The FFS on defining a new MAC CE for the 12 bits TAC was left considering whether it would be possible to include the TAC into the DCI scheduling the DL assignment or UL grant or whether it would be possible to rely on the current MAC CE with 6 bits TAC.

The former option seems like an overkill reserving 12 bits from DCIs used for both, DL assignments and UL grants, just for the purpose of CONNECTED mode UE performing 2-step Random Access when it is not synchronized – when the synchronization exists, the TAC is not necessary as agreed in the previous meeting. Besides, when such TAC is sent along with the DL assignment, as well the TAC could be provided with a MAC CE. Besides, looking at the DCI formats for PUSCH and PDSCH scheduling defined in [1], there are no reserved bits to use for the purpose.

Observation 1: Reserving 12 bits from DCIs used for both, DL assignments and UL grants, just for the purpose of CONNECTED mode UE performing 2-step Random Access when it is not synchronized is infeasible.

The latter option seems not possible given the TAC provides adjustment relative to the previous timing and would allow adjusting the timing only 32 steps to either direction of the previous timing the UE had, ie., NTA. In case the timing adjustment would be required to be higher, network would have to issue PDCCH order defeating the whole purpose of performing 2-step RA in the first place.

Observation 2: Relying solely to the current TAC MAC CE with 6 bits TAC is infeasible.

On the contrary, there seems to be no issue in defining a new DL MAC CE with 12 bits TAC which can be applied by the NW when the C-RNTI is included into the MsgA by the CONNECTED mode UE. A reserved LCID value can be used to indicate this MAC CE to the UE.

Proposal 1: Define a new DL MAC CE with 12 bits TAC field, reserved LCID can be used to indicate this to the UE.

Proposal 2: Agree on the TP provided in the section 3.

# 3	TP into TS 38.321

First Modified Subclause

### 6.1.3	MAC Control Elements (CEs)

(…)

#### 6.1.3.21	Long Timing Advance Command MAC CE

The Long Timing Advance Command MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-1.

It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.21-1):

-	R: Reserved bit, set to 0;

-	Timing Advance Command: This field indicates the index value TA (0, 1, 2… 4095) used to control the amount of timing adjustment that MAC entity has to apply (as specified in TS 38.213 [6]). The length of the field is 12 bits.

Figure 6.1.3.21-1: Long Timing Advance Command MAC CE

Next Modified Subclause

## 6.2	Formats and parameters

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel |
| 33-45 | Reserved |
| 46 | Long Timing Advance Command |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octets) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) |
| 1–32 | Identity of the logical channel |
| 33–51 | Reserved |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

End of Modified Subclauses

# 4	Conclusion

Observation 1: Reserving 12 bits from DCIs used for both, DL assignments and UL grants, just for the purpose of CONNECTED mode UE performing 2-step Random Access when it is not synchronized is infeasible.

Observation 2: Relying solely to the current TAC MAC CE with 6 bits TAC is infeasible.

Proposal 1: Define a new DL MAC CE with 12 bits TAC field, reserved LCID can be used to indicate this to the UE.

Proposal 2: Agree on the TP provided in the section 3.

# References

[1]		TS 38.212 v15.6.0, Multiplexing and channel coding