3GPP TSG-RAN WG2 Meeting #107bis	R2-1913635

Chongqing, China, 14 – 18 October 2019

Agenda item:	6.18.2

Source:	Qualcomm Incorporated

Title:	Support for NPN HRNN broadcast

WID/SID:	Private Network Support for NG-RAN

Document for:	Discussion

# 1	Introduction

Following agreements were made regarding Human Readable Network Name (HRNN) for SNPN and CAG in RAN2#107:

5	If HRNN are broadcast then the HRNN should a be broadcasted in a separate SIB (i.e. different from SIB1).

4	If HRNN are broadcast then the HRNN should a be broadcasted in a separate SIB (i.e. different from SIB1).

Note that it was not agreed whether HRNNs are broadcast and this issue is discussed further in this paper.

# 2	Discussion

TS 23.501 includes following excerpts about expectation that SNPNs and CAG broadcast HRNN:

#### 5.30.2.2 Broadcast system information NG-RAN nodes which provide access to SNPNs broadcast the following information: - One or multiple PLMN IDs - List of NIDs per PLMN ID identifying the non-public networks NG-RAN provides access to NOTE 1: It is assumed that an NG-RAN node supports broadcasting a total of twelve NIDs. Further details are defined in TS 38.331 [28]. NOTE°2: The presence of a list of NIDs for a PLMN ID indicates that the related PLMN ID and NIDs identify SNPNs. - Optionally a human-readable network name per NID. NOTE 3: The human-readable network name per NID is only used for manual SNPN selection. The mechanism how human-readable network name is provided (i.e. whether it is broadcasted or unicasted) to the UE is specified in TS 38.331 [28]. - Optionally information, as described in TS 38.300 [27], TS 38.331 [28] and in TS 38.304 [50], to prevent UEs not supporting SNPNs from accessing the cell, e.g. in case the cell only provides access to non-public networks.

#### 5.30.3.2 Identifiers The following is required for identification: - A CAG is identified by a CAG Identifier which is unique within the scope of a PLMN ID; - A CAG cell broadcasts one or multiple CAG Identifiers per PLMN; NOTE 1: It is assumed that an NG-RAN node supports broadcasting a total of twelve CAG Identifiers. Further details are defined in TS 38.331 [28]. - A CAG cell may in addition broadcast a human-readable network name per CAG Identifier: NOTE 2: The human-readable network name per CAG Identifier is only used for presentation to user when user requests a manual CAG selection.

Observation 1: SA2 specification states that an SNPN is able to broadcast human-readable network name per NID and a CAG cell is able to broadcast a human-readable network name per CAG Identifier.

[1] notes that broadcasting of HRNN introduces additional overhead in system information. However, HRNN broadcast can be left as optional and overhead is not significant in NPNs broadcasting few NIDs/CAG-IDs which if absent impacts manual selection.

Observation 2: Overhead associated with broadcast of HRNNs is not significant in NPNs broadcasting few NIDs/CAG-IDs. Also, HRNNs should be optional IEs which if absent impacts manual selection.

Hence, we propose the following:

Proposal 1: RAN2 should define a new SIB to broadcast HRNNs for SNPNs and CAGs.

Proposal 2: It should be possible to broadcast HRNN of each SNPN and CAG associated with a cell.

# 5	Conclusions

The proposals and observations are re-iterated below:

Observation 1: SA2 specification states that an SNPN is able to broadcast human-readable network name per NID and a CAG cell is able to broadcast a human-readable network name per CAG Identifier.

Observation 2: Overhead associated with broadcast of HRNNs is not significant in NPNs broadcasting few NIDs/CAG-IDs. Also, HRNNs should be optional IEs which if absent impacts manual selection.

Proposal 1: RAN2 should define a new SIB to broadcast HRNNs for SNPNs and CAGs.

Proposal 2: It should be possible to broadcast HRNN of each SNPN and CAG associated with a cell.

# 6	References

- R2-1908981	Discussion on the need for Human Readable Network Name (HRNN) in broadcast, Ericsson