3GPP TSG-RAN WG2 Meeting #107bis			     R2-1913875

Chongqing, China, 14th - 18th October 2019               (Revision of R2-1909825)

Agenda Item	: 6.13.3 (NR_2step_RACH-Core)

Source	: LG Electronics Inc.

Title	: Need for RAPID MAC CE in payload of msgA

Document for	: Discussion and Decision

1.	Introduction

At RAN1 meeting, the following agreements were separately made:

Agreements @RAN1 #96bis:

- PUSCH resource unit for 2-step RACH is defined as

  - The PUSCH occasion and DMRS port / DMRS sequence used for an msgA payload transmission.

    - FFS support only one or both of DMRS port / DMRS sequence

    - The DMRS sequence generation mechanism should follow Rel.15.

Working assumption:

- At least support one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

  - Configurable number of preambles (including one or multiple) mapped to one PUSCH resource unit

  - FFS one-to-multiple mapping

- Companies are strongly encouraged to perform additional evaluations/analysis

Agreements @RAN1 #98:

- Confirm the working assumption that both one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit (PRU) are supported

  - Configurable number of preambles (including one or multiple) mapped to one PRU, explicitly or implicitly

  - FFS 1-to-multiple mapping

In this contribution, we’d like to discuss the need for pairing between preamble and payload of msgA, and suggest to include RAPID MAC CE in MsgA.

2.	Discussion

According to the agreements at RAN2#105bis, msgA of the 2-step RACH should contain the equivalent contents of msg3 of 4-step RACH consisting of the CCCH MAC SDU, or MAC SDU and C-RNTI MAC CE depending on whether or not the transmission of msgA is made for CCCH logical channel. However, we should additionally consider the point that the UE in 2-step RACH sequentially transmits the preamble and payload of msgA on each contention-based resource region. According to the agreements at RAN1 #98, RAN1 confirms the working assumption that both one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit are supported.

Observation 1. At RAN1 #98 meeting, RAN1 has decided to support both one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

If the network configures a PUSCH resource unit per preamble, there is no pairing issue because the network knows the payload received on the PUSCH resource unit associated with each preamble. However, if the network configures association with multiple preambles for one PUSCH resource unit, we should take into account the case where the network receives only one payload for multiple preambles. If UEs selecting the same PUSCH resource unit are located far from each other and/or one of them transmits the payload with higher power than others, it is possible for a gNB to successfully receive the payload only for the UE. Other UEs, which select the same PUSCH resource unit associated with different preamble index but successfully transmits preamble only, can quickly fall back to 4-step RACH by receiving the fallbackRAR. The best solution is for gNB to provide a PUSCH resource unit per preamble for 2-step RACH, but we should consider the case where the multiple-to-one mapping between preamble and PUSCH resource unit is supported, since there may be no sufficient PUSCH resources especially for cells of large coverage as well as RAN1 has already decided to support the multiple-to-one mapping.

Observation 2. If the network configures association with multiple preambles for one PUSCH resource unit, the gNB may receive multiple preambles on a RO but only one payload on a PUSCH resource unit associated with the multiple preambles.

If gNB configures multiple-to-one mapping between preamble and PUSCH resource unit, one or more UEs selecting different preamble index may transmit each payload on the same PUSCH resource unit. Figure 1 shows an example where the preamble index and PUSCH resource mapping is N:1. In this case, gNB receives two different preamble indexes on a PRACH occasion (RO), but only one payload on the PUSCH2 due to PUSCH collision. Based on the association between the preamble index and PUSCH resource unit, the gNB recognizes that two UEs transmit each payload on PUSCH2 but one of UEs fails to transmit its payload through receiving two preamble indexes. However, the gNB doesn’t know the exact preamble associated with the received payload even if there is the association between the preamble index and PUSCH resource unit. Consequently, the gNB cannot include the accurate TA command of the UE in the successRAR and cannot distinguish the preamble index for the fall back procedure.

Observation 3. If two UEs selecting different preamble index transmits each payload on the same PUSCH resource unit and gNB receives only one payload for two preambles, the gNB cannot include the accurate TA command of the UE in the successRAR and cannot distinguish the preamble index for the fallback procedure.

Figure 1. A collision case for PUSCH resource of msgA

Based on the above observations, for explicit pairing between them in case of multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit, we propose that MsgA additionally contain the RAPID MAC CE in the payload of msgA to provide the association between the preamble index and payload of msgA when the mapping between preambles and PUSCH resource unit is multiple-to-one.

Proposal. The payload of msgA should contain RAPID MAC CE including the randomly selected RACH Preamble Index of msgA when the mapping between preambles and PUSCH resource unit is multiple-to-one.

3.	Conclusion

In this contribution, we discuss contents of msgA and msgB required from the MAC point of view, and our proposals are as follows.

Observation 1. At RAN1 #98 meeting, RAN1 has decided to support both one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

Observation 2. If the network configures association with multiple preambles for one PUSCH resource unit, the gNB may receive multiple preambles on a RO but only one payload on a PUSCH resource unit associated with the multiple preambles.

Observation 3. If two UEs selecting different preamble index transmits each payload on the same PUSCH resource unit and gNB receives only one payload for two preambles, the gNB cannot include the accurate TA command of the UE in the successRAR and cannot distinguish the preamble index for the fallback procedure.

Proposal. The payload of msgA should contain RAPID MAC CE including the randomly selected RACH Preamble Index of msgA when the mapping between preambles and PUSCH resource unit is multiple-to-one.