3GPP TSG-RAN WG2 Meeting #108	R2-1914604

Reno,  USA, 18 – 22 November 2019

Agenda item:	6.4.6

Source:	Nokia, Nokia Shanghai Bell

Title:	Notification for Alternative QoS profiles

WID/SID:	5G_V2X_NRSL-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

Alternative QoS profiles have been agreed by SA2. The related mechanisms had been so far contained within the network but recent additions will now trigger NAS signalling towards the UE [S2-1910015]. This contribution discusses the impacts this would have on RAN.

# 2	Discussion

The recent additions to the notification mechanisms for alternative QoS profiles now involve the UE [S2-1910015]:

Upon receiving a notification from the NG-RAN that the GFBR can no longer be guaranteed, the SMF may forward the notification to the PCF, see TS 23.503 [45]. Unless the PCF indicates differently, the SMF uses NAS signalling (that is sent transparently through the RAN) to inform the UE about changes in the QoS parameters (i.e., 5QI, GFBR, MFBR) that the NG-RAN is currently fulfilling for the QoS Flow.

Although such NAS signalling can indeed be sent transparently through the RAN, it is not transparent to the RAN as such since as it requires RAN resources to be sent over Uu. And when the lack of resources triggers such signalling in the first place, consuming more resources to convey it can only worsen the problem.

Observation 1: the notification reduces the useful throughput further.

As a matter of fact, because each notification consumes critical resources (especially in bad radio conditions), each notification might in turn trigger another one, resulting in a cascade of notifications. The more limited the resources, the more serious the impacts on useful throughput. One can imagine a scenario where the notifications might end up using all the useful throughput.

Observation 2: in bad radio conditions, one notification might set off a series of notifications.

Since applications have been dealing with variable bit rates for decades, it is fair to say that they can naturally cope with varying radio conditions and throughputs. Thus, it is not clear to us what benefits the new notification would bring to the end user. The available throughput is directly visible to the UE (through grants) and TCP window management can efficiently manage variations. Besides, since in congested situations the network will not be able to deal with the plethora of notifications for all UEs having GBR services, applications would still need to quickly adapt without this new information.

Observation 3: applications have been and will still have to quickly adapt to variable bit rates.

Based on those three observations, it is suggested to not notify the UE.

Proposal 1: the SMF does not inform the UE about changes in the QoS parameters that the NG-RAN is currently fulfilling for a QoS Flow.

# 3	Conclusion

Regarding the possibility of the SMF informing the UE about changes in the QoS parameters that the NG-RAN is currently fulfilling for a QoS Flow, this paper has observed the following:

Observation 1: the notification reduces the useful throughput further.

Observation 2: in bad radio conditions, one notification might set off a series of notifications.

Observation 3: applications have been able to and will still have to quickly adapt to variable bit rates.

Based on those three observations, the following proposal was made:

Proposal 1: the SMF does not inform the UE about changes in the QoS parameters that the NG-RAN is currently fulfilling for a QoS Flow.

Corresponding LS is available in R2-1914605.