3GPP TSG-RAN WG2 Meeting #108	DRAFT R2-1914605

Reno, USA, 18 – 22 November 2019

Title:	[DRAFT] LS on Notification for Alternative QoS profiles

Response to:	-

Release:	Release 16

Work Item:	5G_V2X_NRSL-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG SA WG2

Cc:	TSG RAN WG3

Contact Person:

#### Name:	Benoist Sébire

E-mail Address:	benoist.sebire@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 has discussed the recent addition for V2X by SA2 of a notification mechanism involving the UE about changes in the QoS parameters that the NG-RAN is currently fulfilling for a QoS Flow.

RAN2 has observed that, although such NAS signalling could be sent transparently through the RAN, it would not be transparent to the RAN since it would require RAN resources to be sent over Uu. And when the lack of resources would trigger such signalling in the first place, consuming more resources to convey it would actually worsen the problem. In bad radio conditions, one notification might set off a series of notifications, impairing the useful throughput.

In RAN2’s understanding, applications have been dealing with varying radio conditions and throughputs. Also, because the available throughput is directly visible to the UE (through grants), and because TCP window management can efficiently manage variations, doubts were expressed on the gains the notification would bring. After all, why notifying the UE of an event it has already observed at first hand?

RAN2 would also like to point out that in congested situation, the network will not be able to notify all UEs and that applications will still have to be able to deal with varying radio conditions without the notifications.

2. Actions:

To SA2 group.

ACTION: 	Given the serious drawbacks and doubts on possible gains, RAN2 respectfully asks SA2 to reconsider notifying the UE about changes in the QoS parameters that the NG-RAN is currently fulfilling for a QoS Flow.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN2#109		24 – 28 February 2020		Athens

3GPP RAN2#109bis		04 – 20 April 2020			Japan