3GPP TSG-RAN2 Meeting #108	    	 					 	    R2-1914797

Reno, USA, 18 - 22 November, 2019

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the MsgB RA-RNTI calculation

Agenda item:	6.13.2

Document for: 	Discussion and Decision

# Introduction

It has been agreed in the previous meeting that:

- RAN2 will work on specifying a new RA-RNTI design for msgB

The intention of this contribution is to share our views on the new RA-RNTI design for MsgB.

# new RA-RNTI design for msgB

- According to current specs, the RA-RNTI design used for 4-step RACH can be found as follows:

- ------------------------------------ RA-RNTI design for 4-step RACH in 38.321 ----------------------------------------

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted, is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8], f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

- ------------------------------------ RA-RNTI design for 4-step RACH  in 38.321 ----------------------------------------

Based on the agreement made in the previous meeting that “Legacy UEs are not required to decode msgB”, the RA-RNTI will be used to distinguish the 2-step RACH from 4-step RACH. To avoid the overlap between new MsgB RA-RNTI and legacy 4-step RACH RA-RNTI, the most straight forward way is to add an offset (e.g. RA_type) in the formula for MsgB RA-RNTI. One example for the change of RA-RNTI can be found as follows:

-------------------------------------- Example for the RA-RNTI for 2-step RACH start ------------------------------------RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 ×2×RA_type

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8], f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier), and RA_type is the RACH type selected (0 for 4-step RACH, and 1 for 2-step RACH).

-------------------------------------- Example for the RA-RNTI for 2-step RACH end -------------------------------------

- Considering the limited time left, we propose to adopt the most straight forward way for MsgB RA-RNTI calculation, and introduce the RA_Type in the formula for MsgB RA-RNTI.

- Proposal 1: The formula below shall be used for MsgB RA-RNTI calculation:

- RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 ×2×RA_type

- Where RA_type is the RACH type selected (0 for 4-step RACH, and 1 for 2-step RACH).

# Conclusion and proposals

Based on the discussion above, the following observations and proposals are provided:

- Proposal 1: The formula below shall be used for MsgB RA-RNTI calculation:

- RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 ×2×RA_type

- Where RA_type is the RACH type selected (0 for 4-step RACH, and 1 for 2-step RACH).

# References

- R2-1914134	Session minutes for NR-U, Power Savings, NTN and 2-step RACH	Session chair (InterDigital)	report