3GPP TSG-RAN WG2 Meeting #108	R2-1914864

Reno, NV, USA, 18-22 November 2019

Title:	Draft LS on Uplink LBT failure indication

# Release:	Rel-16

Work Item:	NR_unlic-Core

Source:	Qualcomm [To be RAN2]

To:	RAN1

# Contact Person:

# Name:	Ozcan Ozturk

E-mail Address:	oozturk@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1. Overall Description:

RAN2 has agreed a mechanism for detection of consistent uplink LBT failure and subsequent recovery. In particular, the following were agreed:

RAN2#107 agreements:

L2 LBT failure mechanism take into account any LBT failure regardless UL transmission type. The UL LBT failure mechanism will have the same recovery mechanism for all failures regardless UL transmission type UL LBT failures are detected per BWP The UE will report the occurrence of consistent UL LBT failures on PSCell and SCells. The assumption is to reuse SCell failure reporting for BF Baseline Mechanism, further enhancements not precluded: A “threshold” for the maximum number of LBT failures which triggers the “consistent” LBT failure event will be used. Both a timer and a counter are introduced, the counter is reset when timer expires and incremented when UL LBT failure happens The timer is started/restarted when UL LBT failure occur.

RAN2#107bis agreements:

Agreements:

- MAC relies on reception of a notification of UL LBT failure from the physical layer to detect a consistent UL LBT failure.

- The UE switches to another BWP and initiates RACH upon declaration of consistent LBT failure on PCell or PSCell if there is another BWP with configured RACH resources.

- The UE shall perform RLF recovery if the consistent UL LBT failure was detected on the PCell and UL LBT failure was detected on “N” possible BWP.   “

- When consistent uplink LBT failures are detected on the PSCell, the UE informs MN via the SCG failure information procedure after detecting a consistent UL LBT failure on “N” BWPs.

- “N” is the number of configured BWPs with configured PRACH resources.   If N is larger than one it is up to the UE implementation which BWP the UE selects.

- When consistent uplink LBT failures are detected on an SCell, a new MAC CE to report this to the node where SCell belongs to is used.  FFS whether the MAC CE can be used to report failure on PCell

As it can be seen from the above agreements, for the operation of this mechanism, an LBT failure indication needs to be sent from physical layer to the MAC layer for all uplink transmissions. It should be emphasized that the indication is needed for also the uplink transmissions which are not triggered by MAC (e.g. ACK/NACK, SRS). Furthermore, all failures, irrespective of channel, CAPC, and LBT type, will be considered equivalent for the detection procedure.

RAN2 would like to respectfully request RAN1 to take the above RAN2 agreements into account and include the necessary uplink LBT failure indication in their specification if found to be feasible from RAN1 perspective.

# 2. Actions:

To RAN1

ACTION: 	.

RAN2 would like to respectfully request RAN1 to take the above RAN2 agreements into account and include the necessary uplink LBT failure indication in their specification if found to be feasible from RAN1 perspective.

# 3. Date of Next TSG-RAN WG2 Meetings:

TSG RAN WG2 Meeting #109			24-28 February 2020	Athens, Greece

TSG RAN WG2 Meeting #109bis			20-24 April 2020	TBD, Japan