# 3GPP TSG-RAN WG2 Meeting #108						  R2-1914903

# Reno, NV, USA, November 18 – November 22, 2019		 resubmission of R2-1913806

Agenda item:		6.20.2 (TEI16)

Source:	LG Electronics Inc., LG Uplus, Vivo

Title: 	ON Duration adaptation

Document for:	Discussion and Decision

1.	Introduction

This document discusses potential enhancement of DRX operation considering different purpose of Short DRX and Long DRX.

2.	Discussion

When DRX is configured, the UE can monitor PDCCH discontinuously using the DRX operation. Currently, two DRX cycles are used, i.e. short DRX cycle and long DRX cycle. In each DRX cycle, there is an ON duration per one DRX cycle, and during the ON duration the UE must monitor PDCCH.

The length of ON duration is ranged from 1/31ms to 1600ms, and signalled from the network. The length of ON duration is same for both short DRX cycle and long DRX cycle. The length of ON duration provides scheduling flexibility to the network.

The Figure 1 shows an example of DRX operation according to the current specification.

Figure 1. DRX operation according to current specification

The short DRX cycle is used when the probability of the DL transmission is expected to be high. As the DL transmission is expected, the UE wakes up frequently and monitors PDCCH frequently. On the other hand, if the probability of the DL transmission is expected to be low, the UE does not have to monitor PDCCH frequently. In this case, it would be better for the UE to monitor PDCCH occasionally using the long DRX cycle.

We think the legacy behavior is not efficient in that the same length of ON duration is used for both short DRX cycle and long DRX cycle. As the probability of DL transmission for UE in long DRX cycle is low compared to that for UE in short DRX cycle, monitoring PDCCH for the same length of ON duration in the long DRX cycle would waste the UE battery.

To save the UE battery in long DRX cycle, it would be better to configure short value of ON duration for long DRX cycle. However, if the short value is configured for the ON duration, the DL transmission would be delayed for the UE in short DRX cycle because the same length of ON duration is applied for both long DRX cycle and short DRX cycle.

To reduce battery consumption in long DRX cycle while not increasing transmission delay in short DRX cycle, it is proposed that different lengths of ON duration are configured for short DRX cycle and long DRX cycle. To meet different purpose of different DRX cycle, it is preferred that longer value is used for the ON duration in short DRX cycle and shorter value is used for the ON duration in long DRX cycle. An example of the proposed method is shown in Figure 2.

Figure 2. Applying different ON duration for Short DRX and Long DRX

We think that introducing different ON duration has minimal impact on the current specifications. In RRC, only one additional signaling, e.g. drx-onDurationTimerLong, is needed, and, in MAC, the changes shown in Annex would be sufficient.

Therefore, we propose that RAN2 consider to use different ON duration for Short DRX and Long DRX.

Proposal: Apply different ON duration for Short DRX and Long DRX.

3.	Proposal

In this document, we discuss potential enhancement of DRX operation, and propose followings:

Proposal: Apply different ON duration for Short DRX and Long DRX.

Annex: Proposed changes to 38.321

5.7 Discontinuous Reception (DRX) The MAC entity may be configured by RRC with a DRX functionality that controls the UE's PDCCH monitoring activity for the MAC entity's C-RNTI, CS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, and TPC-SRS-RNTI. When using DRX operation, the MAC entity shall also monitor PDCCH according to requirements found in other clauses of this specification. When in RRC_CONNECTED, if DRX is configured, for all the activated Serving Cells, the MAC entity may monitor the PDCCH discontinuously using the DRX operation specified in this clause; otherwise the MAC entity shall monitor the PDCCH as specified in TS 38.213 [6]. RRC controls DRX operation by configuring the following parameters: - drx-onDurationTimer: the duration at the beginning of a Short DRX Cycle; - drx-onDurationTimerLong: the duration at the beginning of a Long DRX Cycle; - drx-SlotOffset: the delay before starting the drx-onDurationTimer and drx-onDurationTimerLong; - drx-InactivityTimer: the duration after the PDCCH occasion in which a PDCCH indicates a new UL or DL transmission for the MAC entity; - drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received; - drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received; - drx-LongCycleStartOffset: the Long DRX cycle and drx-StartOffset which defines the subframe where the Long and Short DRX Cycle starts; - drx-ShortCycle (optional): the Short DRX cycle; - drx-ShortCycleTimer (optional): the duration the UE shall follow the Short DRX cycle; - drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity; - drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity. When a DRX cycle is configured, the Active Time includes the time while: - drx-onDurationTimer or drx-onDurationTimerLong or drx-InactivityTimer or drx-RetransmissionTimerDL or drx-RetransmissionTimerUL or ra-ContentionResolutionTimer (as described in clause 5.1.5) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in clause 5.4.4); or - a PDCCH indicating a new transmission addressed to the C-RNTI of the MAC entity has not been received after successful reception of a Random Access Response for the Random Access Preamble not selected by the MAC entity among the contention-based Random Access Preamble (as described in clause 5.1.4). When DRX is configured, the MAC entity shall: 1> if a MAC PDU is received in a configured downlink assignment: 2> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; 2> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. 1> if a MAC PDU is transmitted in a configured uplink grant: 2> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission; 2> stop the drx-RetransmissionTimerUL for the corresponding HARQ process. 1> if a drx-HARQ-RTT-TimerDL expires: 2> if the data of the corresponding HARQ process was not successfully decoded: 3> start the drx-RetransmissionTimerDL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerDL. 1> if a drx-HARQ-RTT-TimerUL expires: 2> start the drx-RetransmissionTimerUL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerUL. 1> if a DRX Command MAC CE or a Long DRX Command MAC CE is received: 2> stop drx-onDurationTimer or drx-onDurationTimerLong; 2> stop drx-InactivityTimer. 1> if drx-InactivityTimer expires or a DRX Command MAC CE is received: 2> if the Short DRX cycle is configured: 3> start or restart drx-ShortCycleTimer in the first symbol after the expiry of drx-InactivityTimer or in the first symbol after the end of DRX Command MAC CE reception; 3> use the Short DRX Cycle. 2> else: 3> use the Long DRX cycle. 1> if drx-ShortCycleTimer expires: 2> use the Long DRX cycle. 1> if a Long DRX Command MAC CE is received: 2> stop drx-ShortCycleTimer; 2> use the Long DRX cycle. 1> if the Short DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-ShortCycle) = (drx-StartOffset) modulo (drx-ShortCycle): 2> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. 1> if the Long DRX Cycle is used, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset: 2> start drx-onDurationTimerLong after drx-SlotOffset from the beginning of the subframe. 1> if the MAC entity is in Active Time: 2> monitor the PDCCH as specified in TS 38.213 [6]; 2> if the PDCCH indicates a DL transmission: 3> start the drx-HARQ-RTT-TimerDL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the DL HARQ feedback; 3> stop the drx-RetransmissionTimerDL for the corresponding HARQ process. 2> if the PDCCH indicates a UL transmission: 3> start the drx-HARQ-RTT-TimerUL for the corresponding HARQ process in the first symbol after the end of the first repetition of the corresponding PUSCH transmission; 3> stop the drx-RetransmissionTimerUL for the corresponding HARQ process. 2> if the PDCCH indicates a new transmission (DL or UL): 3> start or restart drx-InactivityTimer in the first symbol after the end of the PDCCH reception. 1> in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause: 2> not transmit periodic SRS and semi-persistent SRS defined in TS 38.214 [7]. 1> if CSI masking (csi-Mask) is setup by upper layers: 2> in current symbol n, if drx-onDurationTimer would not be running considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause: 3> not report CSI on PUCCH. 1> else: 2> in current symbol n, if the MAC entity would not be in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to symbol n when evaluating all DRX Active Time conditions as specified in this clause: 3> not report CSI on PUCCH and semi-persistent CSI on PUSCH. Regardless of whether the MAC entity is monitoring PDCCH or not, the MAC entity transmits HARQ feedback, aperiodic CSI on PUSCH, and aperiodic SRS defined in TS 38.214 [7] when such is expected. The MAC entity needs not to monitor the PDCCH if it is not a complete PDCCH occasion (e.g. the Active Time starts or ends in the middle of a PDCCH occasion).