3GPP TSG-RAN WG2 Meeting#108          							   R2-1915091

Reno, USA, 18th – 22th Nov 2019

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the HRNN and Access Control

Agenda item:		6.18.4

Document for: 	Discussion and Decision

- Introduction

In the previous meetings, there are many discussion on the SIB1 design, in this paper we focus on the HRNN and Access control, which also have impact on the SIB1 structure.

- Discussions

2.1 Human Readable Network Name

In the previous meetings, it has been agreed that

If HRNN are broadcast then the HRNN should be broadcasted in a separate SIB (i.e. different from SIB1).

As described in [1], the HRNN is optionally broadcasted for the NPN network, then the problems is how does the UE knows which HRRNS are present. In the LTE, a cell only belongs to one CSG, thus the cell only need to broadcast one HeNB name in SIB9. If there is no HeNB name, the network may not broadcast the SIB9. For the SNPN/CAG, considering the network sharing scenario, a cell may be shared by the different SNPN/CAG IDs and the HRNN per NID is optionally present, thus it’s better to add an indication to indicate whether the HRNN present for each SNPN/CAG ID in the SIB1, then the UE can decide whether to receive the corresponding SIB based on this indication.

Proposal 1: In SIB1, an indication shall be added for each SNPN/CAG ID to indicate whether the HRNN present.

Based on proposal 1, as an example the SNPN/CAG parameters can be expressed as follow:

snpn-IdentityInfoList SEQUENCE (SIZE (1..maxNPN)) OF SNPN-IdentityInfo, -- Cond SNPN cag-IdentityInfoList SEQUENCE (SIZE (1..maxNPN)) OF CAG-IdentityInfo, -- Cond CAG SNPN-IdentityInfo ::= SEQUENCE { plmn-Identity PLMN-Identity, snpn-IDInfoList SEQUENCE (SIZE (1..maxNPN))OF SNPNInfo, } SNPNInfo ::= SEQUENCE { snpn-ID SNPN-ID, RedableNamePresent ENUMERATED {TRUE} OPTIONAL -- Need R } CAG-IdentityInfo ::= SEQUENCE { plmn-Identity PLMN-Identity, cag-IDInfoList SEQUENCE (SIZE (1..maxNPN))OF CAGInfo, } CAGInfo ::= SEQUENCE { cag-ID CAG-ID, redableNamePresent ENUMERATED {TRUE} OPTIONAL -- Need R } Note: The format of the SNPN and CAG identity list can be modified based on the latest email discussion result.

Furthermore, to indicate associated network ID of the HRNN, an Index shall be added to the newly added SIB as follow:

SIB10 information element -- ASN1START -- TAG-SIB10-START SIB10 ::= SEQUENCE { readableNameList SEQUENCE (SIZE (1..maxSNPN))OF ReadableName, OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } ReadableName::= SEQUENCE { readableName-index INTEGER (1..maxNPN), readableName OCTET STRING } -- TAG-SIB10-STOP -- ASN1STOP

Proposal 2: An Index shall be added to the newly added SIB to indicate associated network ID of this HRNN.

2.2 Access control

In this chapter, we discuss the Unified Access Control for the SNPN network and CAG respectively. For the PLMN network, the unified access control is adopted and the uac-BarringInfo is defined per PLMN in SIB1 as follow:

uac-BarringInfo SEQUENCE { uac-BarringForCommon UAC-BarringPerCatList OPTIONAL, -- Need S uac-BarringPerPLMN-List UAC-BarringPerPLMN-List OPTIONAL, -- Need S uac-BarringInfoSetList UAC-BarringInfoSetList, uac-AccessCategory1-SelectionAssistanceInfo CHOICE { plmnCommon UAC-AccessCategory1-SelectionAssistanceInfo, individualPLMNList SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AccessCategory1-SelectionAssistanceInfo } OPTIONAL -- Need S } OPTIONAL, -- Need R

For the SNPN network, the network is identified by the SNPN ID. Considering that the SNPN network is deployed by the operator individually, and that the NG-RAN may be shared by multiple SNPNs, the uac-BarringInfo for the SNPN shall also be defined per SNPN ID.

Proposal 3: The UAC of the SNPN shall be defined per SNPN ID for the network sharing scenario.

For the PLMN Network, the Access Category and the Access Identity are mainly determined by the NAS layer based on the UE access attempt and the USIM Information (or the network configuration). To make it clear, in [1] a separate chapter was added to introduce the determination of Access Identity and the Access Category for the SNPN network. When it comes to the CAG, according to [1], a CAG cell broadcasts one or multiple CAG Identifiers per PLMN, then the question is whether per CAG ID Access control is needed or not. Different from the SNPN, the different CAG IDs under the same PLMN are deployed by the same operator. Furthermore, as noted in [1], the CAG is not used for the AMF or the Network Slice selection.

NOTE 2: CAG is used for authorization at network/cell selection and configured in the subscription as part of the Mobility Restrictions i.e. independent from any S-NSSAI. CAG is not used as input to AMF selection nor Network Slice selection.

Besides in [1], it also said in the Annex D.2 that

In order to prevent access to NPNs for authorized UE(s) in case of network congestion/overload and if a dedicated S-NSSAI has been allocated for an NPN, the Unified Access Control can be used using the operator-defined access categories with access category criteria type (as defined in TS 24.501) set to the S-NSSAI used for an NPN.

Thus, there is no need to support CAG ID level access control.

Proposal 4: CAG ID level access control is not supported, the UE execute the access control for the CAG cell based on the related PLMN.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: In SIB1, an indication shall be added for each SNPN/CAG ID to indicate whether the HRNN present.

Proposal 2: An Index shall be added to the newly added SIB to indicate associated network ID of this HRNN.

Proposal 3: The UAC of the SNPN shall be defined per SNPN ID for the network sharing scenario.

Proposal 4: CAG ID level access control is not supported, the UE execute the access control for the CAG cell based on the related PLMN.

- Reference

3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".