| 3GPP TS 38.314 V0.0.3 (2019-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Layer 2 Measurements (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	6

4	Layer 2 measurements	6

4.1	NR measurements performed by the gNB	6

4.1.1	Measurements valid for all gNB deployment scenarios	6

4.1.1.1	 Received Random Access Preambles	6

4.1.1.1.1	Received Random Access Preambles per cell	6

4.1.1.1.2	Received Random Access Preambles per SSB	6

4.1.1.2	 Packet delay	6

4.1.1.2.1	Average over-the-air interface packet delay in the UL per QCI	6

4.1.1.2.2	Average over-the-air interface packet delay in the UL per mapped 5QI	7

4.1.1.3	Number of active UEs in RRC_CONNECTED	7

4.1.1.3.1	Mean number of Active UEs in the DL per mapped 5QI per cell	8

4.1.1.3.2	Max number of Active UEs in the DL per mapped 5QI per cell	8

4.1.1.3.3	Mean number of Active UEs in the UL per mapped 5QI per cell	10

4.1.1.3.4	Max number of Active UEs in the UL per mapped 5QI per cell	10

4.1.1.3.5	Mean number of Active UEs per cell	11

4.1.1.3.6	Max number of Active UEs per cell	12

4.1.1.3.7	Mean number of Active UEs per mapped 5QI per cell	13

4.1.1.3.8	Max number of Active UEs per mapped 5QI per cell	14

4.1.1.4	Number of stored inactive UE contexts	15

4.1.1.4.1	 Mean number of stored inactive UE contexts	15

4.1.1.4.2	 Max number of stored inactive UE contexts	16

4.1.2	Measurements valid for split gNB deployment scenario	16

4.2	NR measurements performed by the UE	16

Annex <X> (informative): Change history	17

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document contains the description and definition of the measurements performed by NR or the UE that are transferred over the standardised interfaces in order to support NR radio link operations, radio resource management (RRM), network operations and maintenance (OAM), minimization of drive tests (MDT) and self-organising networks (SON).

Only the differences relative to TS 28.552 v16.2.0 [2] are specified in this specification.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.552 v16.2.0: "5G performance measurements".

3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

4	Layer 2 measurements

4.1	NR measurements performed by the gNB

4.1.1	Measurements valid for all gNB deployment scenarios

4.1.1.1	 Received Random Access Preambles

4.1.1.1.1	Received Random Access Preambles per cell

A use case for this measurement is RACH configuration optimization, where Received Random Access Preambles is signalled across an OAM interface.

Protocol Layer: MAC

| Definition | Received Random Access Preambles per cell. This measurement is applicable to PRACH. The reference point is the Service Access Point between MAC and L1. The measured quantity is the number of received Random Access preambles during a time period over all PRACHs configured in a cell. The measurement is done separately for: - Dedicated preambles - Randomly selected preambles in the low range - Randomly selected preambles in the high range. The unit of the measured value is [/s]. |
| --- | --- |

4.1.1.1.2	Received Random Access Preambles per SSB

A use case for this measurement is RACH configuration optimization, where Received Random Access Preambles is signalled across an OAM interface.

Protocol Layer: MAC

| Definition | Received Random Access Preambles per SSB. This measurement is applicable to PRACH. The reference point is the Service Access Point between MAC and L1. The measured quantity is the number of received Random Access preambles during a time period over all PRACHs configured in a cell. The measurement is done separately for: - Dedicated preambles - Randomly selected preambles in the low range - Randomly selected preambles in the high range. The unit of the measured value is [/s]. |
| --- | --- |

4.1.1.2	 Packet delay

4.1.1.2.1	Average over-the-air interface packet delay in the UL per QCI

The objective of this measurement is to measure air interface UL packet delay for OAM performance observability or for QoS verification of MDT.

Protocol Layer: MAC, RLC

| Definition | Average over-the-air packet delay in the UL per QCI. This measurement is applicable for EN-DC. This measurement refers to packet delay for DRBs. This measurement provides the average (arithmetic mean) time it takes to successfully receive a transport block from the time of UL transmission indicated in scheduling grant. Detailed Definition: M(T,qci)=∀itSucc(i)-tSched(i)I(T),where explanations can be found in the table 4.1.1.2.1-1 below. |
| --- | --- |

Table 4.1.1.2.1-1

| M(T,qci) | Over-the-air packet delay in the UL per QCI, averaged during time period T. Unit: Integer ms. |
| --- | --- |
| tSched(i) | The point in time when the UL RLC SDU i is scheduled as per the scheduling grant provided. |
| tSucc(i) | The point in time when the RLC SDU i was received successfully by the network. |
| i | A MAC SDU that arrives at the RLC during time period T. |
| I(T) | Total number of RLC SDUs i. |
| T | Time Period during which the measurement is performed |

4.1.1.2.2	Average over-the-air interface packet delay in the UL per mapped 5QI

The objective of this measurement is to measure air interface UL packet delay for OAM performance observability or for QoS verification of MDT.

Protocol Layer: MAC, RLC

| Definition | Average over-the-air packet delay in the UL per mapped 5QI. This measurement is applicable for SA, NR-DC, NE-DC, and NGEN-DC. This measurement refers to packet delay for DRBs. This measurement provides the average (arithmetic mean) time it takes to successfully receive a transport block from the time of UL transmission indicated in scheduling grant. Detailed Definition: M(T,m5qi)=∀itSucc(i)-tSched(i)I(T),where explanations can be found in the table 4.1.1.2.1-1 below. |
| --- | --- |

Table 4.1.1.2.1-1

| M(T,qci/m5qi) | Over-the-air packet delay in the UL per mapped 5QI, averaged during time period T. Unit: Integer ms. |
| --- | --- |
| tSched(i) | The point in time when the UL RLC SDU i is scheduled as per the scheduling grant provided. |
| tSucc(i) | The point in time when the RLC SDU i was received successfully by the network. |
| i | A MAC SDU that arrives at the RLC during time period T. |
| I(T) | Total number of RLC SDUs i. |
| T | Time Period during which the measurement is performed |

4.1.1.3	Number of active UEs in RRC_CONNECTED

The objective of the measurement is to measure number of active UEs per mapped 5QI for OAM performance observability. It is intended to be part of a calculation to determine the bitrate UEs achieve when they are active, i.e. when applications are transmitting and receiving data. The measurements are applicable for both non-split gNB and split gNB deployment scenario.

4.1.1.3.1	Mean number of Active UEs in the DL per mapped 5QI per cell

Protocol Layer: MAC, RLC

| Definition | Mean number of Active UEs in the DL per mapped 5QI per cell. This measurement refers to UEs for which there is buffered data for the DL for DRBs. The measurement is done separately per mapped 5QI. Detailed Definition: M(T,m5qi,p)=∀iN(i,5qi)I(T,p),where explanations can be found in the table 4.1.1.3.1-1 below. |
| --- | --- |

Table 4.1.1.3.1-1

| M(T,m5qi,p) | Mean number of Active UEs in the DL per mapped 5QI, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i,m5qi) | Number of UEs for which there is buffered data for the DL in MAC or RLC protocol layers for a Data Radio Bearer of traffic class with mapped 5QI = m5qi at sampling occasioni. In RLC and MAC layers, buffered data corresponds to data available for transmission according to the definitions in TS 38.322 and TS 38.321. Buffered data includes data for which HARQ transmission has not yet terminated. |
| i | Sampling occasion during time periodT. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |

4.1.1.3.2	Max number of Active UEs in the DL per mapped 5QI per cell

Protocol Layer: MAC, RLC

| Definition | Maximum number of Active UEs in the DL per mapped 5QI per cell. This measurement refers to UEs for which there is buffered data for the DL for DRBs. The measurement is done separately per mapped 5QI. Detailed Definition: M(T,m5qi,p)=maxTNi,m5qi,where explanations can be found in the table 4.1.1.3.2-1 below. |
| --- | --- |

Table 4.1.1.3.2-1

| M(T,m5qi,p) | Maximum number of Active UEs in the DL per mapped 5QI, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i,m5qi) | Number of UEs for which there is buffered data for the DL in MAC, RLC or PDCP protocol layers for a Data Radio Bearer of traffic class with mapped 5QI = m5qi at sampling occasioni. In RLC and MAC layers, buffered data corresponds to data available for transmission according to the definitions in TS 38.322 and TS 38.321. Buffered data includes data for which HARQ transmission has not yet terminated. |
| i | Sampling occasion during time periodT. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |

4.1.1.3.3	Mean number of Active UEs in the UL per mapped 5QI per cell

Protocol Layer: MAC, RLC

| Definition | Mean number of Active UEs in the UL per mapped 5QI per cell. This measurement refers to UEs for which there is buffered data for the UL for DRBs. The measurement is done separately per mapped 5QI. Detailed Definition: MT,m5qi,p=∀iNi,m5qiIT,p, where explanations can be found in the table 4.1.1.3.3-1 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load UE configuration and how DRBs are distributed over logical channel groups.

Table 4.1.1.3.3-1

| M(T,m5qi,p) | Mean number of Active UEs in the UL per mapped 5QI, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i,m5qi) | Number of UEs for which there is buffered data for the UL in MAC or RLC protocol layers for a Data Radio Bearer of traffic class with mapped 5QI = m5qi at sampling occasion.i This is a gNB estimation that is expected to be based on Buffer Status Reporting, provided semi-persistent grants and progress of ongoing HARQ transmissions (by including buffered data for which HARQ transmission has not yet terminated in buffered data). In addition, the gNB can use the analysis of received data in the estimation. In such case, when mapped 5QI cannot be determined at the time of the sampling occasion, gNB can determine mapped 5QI after successful reception of data. |
| i | Sampling occasion during time periodT. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |

4.1.1.3.4	Max number of Active UEs in the UL per mapped 5QI per cell

Protocol Layer: MAC, RLC

| Definition | Maximum number of Active UEs in the UL per mapped 5QI per cell. This measurement refers to UEs for which there is buffered data for the UL for DRBs. The measurement is done separately per mapped 5QI. Detailed Definition: MT,m5qi,p=maxTNi,m5qi, where explanations can be found in the table 4.1.1.3.4-1 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load UE configuration and how DRBs are distributed over logical channel groups.

Table 4.1.1.3.4-1

| M(T,m5qi,p) | Maximum number of Active UEs in the UL per mapped 5QI, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i,m5qi) | Number of UEs for which there is buffered data for the UL in MAC or RLC protocol layers for a Data Radio Bearer of traffic class with mapped 5QI = m5qi at sampling occasion.i This is a gNB estimation that is expected to be based on Buffer Status Reporting, provided semi-persistent grants and progress of ongoing HARQ transmissions (by including buffered data for which HARQ transmission has not yet terminated in buffered data). In addition, the gNB can use the analysis of received data in the estimation. In such case, when mapped 5QI cannot be determined at the time of the sampling occasion, gNB can determine mapped 5QI after successful reception of data. |
| i | Sampling occasion during time periodT. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |

4.1.1.3.5	Mean number of Active UEs per cell

Protocol Layer: MAC, RLC

| Definition | Mean number of Active UEs per cell. This measurement refers to UEs for which there is buffered data for the UL for DRBs, or there is buffered data for the DL for DRBs, or both. Detailed Definition: MT,p=∀iNiIT,p, where explanations can be found in the table 4.1.1.3.5-1 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load UE configuration and how DRBs are distributed over logical channel groups.

Table 4.1.1.3.5-1

| M(T,p) | Mean number of Active UEs, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i) | Number of UEs for which there is buffered data for the UL or for the DL or for both in MAC or RLC protocol layers at sampling occasion.i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided semi-persistent grants and progress of ongoing HARQ transmissions (by including buffered data for which HARQ transmission has not yet terminated in buffered data). In addition, the gNB can use the analysis of received data in the estimation. For DL, in RLC and MAC layers, buffered data corresponds to data available for transmission according to the definitions in TS 38.322 and TS 38.321. Buffered data includes data for which HARQ transmission has not yet terminated. |
| i | Sampling occasion during time periodT. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |

4.1.1.3.6	Max number of Active UEs per cell

Protocol Layer: MAC, RLC

| Definition | Maximum number of Active UEs per cell. This measurement refers to UEs for which there is buffered data for the UL for DRBs, or there is buffered data for the DL for DRBs, or both. Detailed Definition: M(T,p)=maxTNi, where explanations can be found in the table 4.1.1.3.6-1 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load UE configuration and how DRBs are distributed over logical channel groups.

Table 4.1.1.3.6-1

| M(T,p) | Maximum number of Active UEs, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i) | Number of UEs for which there is buffered data for the UL or for the DL or for both in MAC or RLC protocol layers at sampling occasion.i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided semi-persistent grants and progress of ongoing HARQ transmissions (by including buffered data for which HARQ transmission has not yet terminated in buffered data). In addition, the gNB can use the analysis of received data in the estimation. For DL, in RLC and MAC layers, buffered data corresponds to data available for transmission according to the definitions in TS 38.322 and TS 38.321. Buffered data includes data for which HARQ transmission has not yet terminated. |
| i | Sampling occasion during time periodT. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |

4.1.1.3.7	Mean number of Active UEs per mapped 5QI per cell

Protocol Layer: MAC, RLC

| Definition | Mean number of Active UEs per mapped 5QI per cell. This measurement refers to UEs for which there is buffered data for the UL for DRBs, or there is buffered data for the DL for DRBs, or both. The measurement is done separately per mapped 5QI. Detailed Definition: MT,m5qi,p=∀iNi,m5qiIT,p, where explanations can be found in the table 4.1.1.3.7-1 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load UE configuration and how DRBs are distributed over logical channel groups.

Table 4.1.1.3.7-1

| M(T,5qi,p) | Number of Active UEs per mapped 5QI, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i,5qi) | Number of UEs for which there is buffered data for the UL or for the DL or for both in MAC or RLC protocol layers for a Data Radio Bearer of traffic class with 5QI = 5qi at sampling occasion.i For UL, This is a gNB estimation that is expected to be based on Buffer Status Reporting, provided semi-persistent grants and progress of ongoing HARQ transmissions (by including buffered data for which HARQ transmission has not yet terminated in buffered data). In addition, the gNB can use the analysis of received data in the estimation. In such case, when mapped 5QI cannot be determined at the time of the sampling occasion, gNB can determine mapped 5QI after successful reception of data. For DL, in RLC and MAC layers, buffered data corresponds to data available for transmission according to the definitions in TS 38.322 and TS 38.321. Buffered data includes data for which HARQ transmission has not yet terminated. |
| i | Sampling occasion during time periodT. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |

4.1.1.3.8	Max number of Active UEs per mapped 5QI per cell

Protocol Layer: MAC, RLC

| Definition | Maximum number of Active UEs per mapped 5QI per cell. This measurement refers to UEs for which there is buffered data for the UL for DRBs, or there is buffered data for the DL for DRBs, or both. The measurement is done separately per mapped 5QI. Detailed Definition: M(T,m5qi,p)=maxTNi,m5qi,where explanations can be found in the table 4.1.1.3.8-1 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load UE configuration and how DRBs are distributed over logical channel groups.

Table 4.1.1.3.8-1

| M(T,5qi,p) | Maximum number of Active UEs per mapped 5QI, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i,5qi) | Number of UEs for which there is buffered data for the UL or for the DL or for both in MAC or RLC protocol layers for a Data Radio Bearer of traffic class with 5QI = 5qi at sampling occasion.i For UL, This is a gNB estimation that is expected to be based on Buffer Status Reporting, provided semi-persistent grants and progress of ongoing HARQ transmissions (by including buffered data for which HARQ transmission has not yet terminated in buffered data). In addition, the gNB can use the analysis of received data in the estimation. In such case, when mapped 5QI cannot be determined at the time of the sampling occasion, gNB can determine mapped 5QI after successful reception of data. For DL, in RLC and MAC layers, buffered data corresponds to data available for transmission according to the definitions in TS 38.322 and TS 38.321. Buffered data includes data for which HARQ transmission has not yet terminated. |
| i | Sampling occasion during time periodT. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |

4.1.1.4	Number of stored inactive UE contexts

The objective of the measurement is to measure number of stored inactive UE contexts for OAM performance observability. It is intended to be part of indication about the memory consumption in a RAN node.

The measurement is obtained by sampling at a pre-defined interval, the number of inactive UE contexts for each NR gNB and then taking the arithmetic mean or maximum value over pre-defined time duration.

4.1.1.4.1	 Mean number of stored inactive UE contexts

| Definition | Mean number of inactive UE contexts. Detailed Definition: M(T,p)=∀iN(i)I(T,p),where explanations can be found in the table 4.1.1.4.1-1 below. |
| --- | --- |

Table 4.1.1.4.1-1

| M(T,p) | Mean number of Inactive UE contexts, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i) | Number of inactive UE contexts stored in the gNB at sampling occasioni. |
| i | Sampling occasion during time periodT. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |

4.1.1.4.2	 Max number of stored inactive UE contexts

| Definition | Maximum number of inactive UE contexts. Detailed Definition: MT,p= maxTNi,where explanations can be found in the table 4.1.1.4.2-1 below. |
| --- | --- |

Table 4.1.1.4.2-1

| M(T,p) | Maximum number of Inactive UE contexts sampled during time period T. Unit: Integer. |
| --- | --- |
| N(i) | Number of inactive UE contexts stored in the gNB at sampling occasioni. |
| i | Sampling occasion during time periodT. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |

4.1.2	Measurements valid for split gNB deployment scenario

4.2	NR measurements performed by the UE

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-08 | RAN2#107 | R2-1909426 | - | - | - | Draft skeleton | 0.0.1 |
| 2019-10 | RAN2#107bis | R2-1912957 | - | - | - | Agreements from RAN2#107 on: - including the general reference to SA2 spec | 0.0.2 |
| 2019-11 | RAN2#108 | R2-1915203 | - | - | - | Capture agreements from RAN2#107bis on received random access preambles, UL over-the-air transmission delay and number of UEs. | 0.0.3 |