3GPP TSG-RAN WG2 Meeting #108	R2-1915221

# Reno, NV, USA, 18–22 November 2019

Agenda item:	6.2.2.5

Source:	Samsung

Title:	Support of dynamic HARQ process ID sharing in NR-U

Document for:	Discussion and Agreement

# 1	Introduction

The contribution discusses whether to support dynamic HARQ process ID sharing when the configured UL grant with cg-RetransmissionTimer is configured on NR-U cell.

# 2	Discussion

For both LTE and Rel-15 NR, the HARQ process IDs can be shared between dynamic UL grant and configured UL grant dynamically. That is, in NR, network configures the number of HARQ process ID for the configured UL grant by using nrofHARQ-Processes (can be from 1 and 16). Then the HARQ process IDs for the configured UL grant has the value 0 to nrofHARQ-Processes – 1. But the HARQ process IDs for the configured UL grant above can still be used for the dynamic UL grant. For instance, if the MAC entity receives UL grant with C-RNTI, and the HARQ process ID is part of the configured grant, it considers as a new transmission, and obtains a new data i.e. the previous data will be removed in such case.

2> if the uplink grant is for MAC entity's C-RNTI and if the previous uplink grant delivered to the HARQ entity for the same HARQ process was either an uplink grant received for the MAC entity's CS-RNTI or a configured uplink grant: 3> consider the NDI to have been toggled for the corresponding HARQ process regardless of the value of the NDI.

The operations above work in the legacy configured grant, as both network and UE know the HARQ process ID for each periodic PUSCH occasion according to the following equation.

For configured uplink grants, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation: HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes where CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8].

That is, network knows the status (e.g. successfully received or retransmission required) of the specific HARQ process ID configured for the HARQ process ID based on the equation above, and decides to use the successfully completed one for the dynamic grant without losing the data.

However for the configured UL grants in NR-U cells (i.e. configured with cg-RetransmissionTimer), the HARQ process ID for each periodic occasion is determined by the UE which network does not know, which is captured in the running CR for NR-U [3] below.

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation select an HARQ Process ID among the HARQ process IDs available for configured grants. The UE shall prioritize retransmissions before initial transmissions.

Since UE determines the HARQ process ID for the new transmission which network does not know, to allow the dynamic HARQ process ID sharing in the NR-U cells may result the data loss. That is, after UE determines the specific HARQ process ID for the new transmission, UE may not be able to transmit the generated packet due to LBT operation. From the network side, since network does not receive anything for the HARQ process ID, it may send the dynamic UL grant using the HARQ process ID. Then, the UE has to replace the existing packet with a new packet, which is not the intended behaviour. Hence, the dynamic HARQ process ID sharing should not be used if the configured UL grant is configured with cg-RetransmissionTimer.

Proposal 1: The dynamic HARQ process ID sharing should not be used if the configured UL grant is configured with cg-RetransmissionTimer.

This can be captured to the running CR for NR-U e.g. as follows:

If the configured uplink grants are configured with cg-RetransmissionTimer for a Serving Cell, the network does not use the HARQ Process ID(s) configured for the configured uplink grant to the dynamic uplink grant in the Serving Cell.

Proposal 2: To capture the following sentence to the running CR:
If the configured uplink grants are configured with cg-RetransmissionTimer for a Serving Cell, the network does not use the HARQ Process ID(s) configured for the configured uplink grant to the dynamic uplink grant in the Serving Cell.

# 3	Conclusion

Proposal 1: The dynamic HARQ process ID sharing should not be used if the configured UL grant is configured with cg-RetransmissionTimer.

Proposal 2: To capture the following sentence to the running CR:
If the configured uplink grants are configured with cg-RetransmissionTimer for a Serving Cell, the network does not use the HARQ Process ID(s) configured for the configured uplink grant to the dynamic uplink grant in the Serving Cell.

# 4	References

[1]	3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".

[2]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[3]	R2-1914026	Running MAC CR for NR-U	Ericsson