3GPP TSG-RAN WG2 Meeting #108	R2-1915253

# Reno, US, 18th – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1365 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification regarding conditions of setting omitEN-DC for mrdc-Request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is currently unclear whether a RAN node can request UE capabilities for MR-DC with filtering of omitEN-DC alone, the intended setting for mrdc-Request is that omitEN-DC always come together with the includeNE-DC if NE-DC only capablities are required. The expected UE operation if the mrdc-Request only includes the omitEN-DC without includeNE-DC, according to TS 38.331 subclause 5.6.1.4, would be below: - UE receives RAT-type of UECapabilityEnquiry as ‘nr’ and ‘eutra-nr’. - UE compiles a list of “candidate band combinations” associated with the consistently set in frequencyBandListFilter. - After checking the omitEN-DC in capabilityRequestFilterCommon, UE removes the band combinations for MR-DC from the list of "candidate band combinations" - UE could provide the empty UE-CapabilityRAT-Container for MR-DC because there are no supportedBandCombinationList and featureSetCombinations. Above operation seems not used for any operation scenarios. The network could set the rat type ‘nr’ alone if the network only needs to get the NR SA capabilities from UE. In addition, it requires redundant UE operation which will not be required for the results of UE capability report. If we see the field desctiption of TS 38.331 v15.60, network implicitly can know the purpose of this omitEN-DC field is to request the NE-DC only BCs. omitEN-DC. Only if this field is present, the UE supporting NE-DC shall indicate support for NE-DC in band combinations and feature set combinations which are applicable to EN-DC. Band combinations supporting both NE-DC and (NG)EN-DC shall be included in supportedBandCombinationList, band combinations supporting only NE-DC shall be included in supportedBandCombinationListNEDC-Only.. However, after changing field description as a results of the CR#1178 (R2-1911737), the restriction/condition of this field is removed. omitEN-DC. Only if this field is present, the UE shall omit band combinations and feature set combinations which are only applicable to EN-DC.. |
| Summary of change: |  | This CR includes the following changes: - Clarify that the network indicates omitEN-DC fields only if the NE-DC only band combinations and the associated feature set combinations are required. Impact analysis Impacted 5G architecture options: (NG)EN-DC, NE-DC Impacted functionality: Enquriy UE capability with MR-DC filtering Inter-operability: No interoperability issue. This is clarification of the network restriction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE receiving mrdc-Request with only omitEN-DC field conduct the redundant operation i.e. create candidate band combination and remove them. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

#### –	UE-CapabilityRequestFilterCommon

The IE UE-CapabilityRequestFilterCommon is used to request filtered UE capabilities. The filter is common for all capability containers that are requested.

UE-CapabilityRequestFilterCommon information element

-- ASN1START

-- TAG-UE-CAPABILITYREQUESTFILTERCOMMON-START

UE-CapabilityRequestFilterCommon ::=            SEQUENCE {

mrdc-Request                                SEQUENCE {

omitEN-DC                                   ENUMERATED {true}                      OPTIONAL,    -- Need N

includeNR-DC                                ENUMERATED {true}                      OPTIONAL,    -- Need N

includeNE-DC                                ENUMERATED {true}                      OPTIONAL     -- Need N

}                                                                                  OPTIONAL,        -- Need N

...

}

-- TAG-UE-CAPABILITYREQUESTFILTERCOMMON-STOP

-- ASN1STOP

UE-CapabilityRequestFilterCommon field descriptions

includeNE-DC Only if this field is present, the UE supporting NE-DC shall indicate support for NE-DC in band combinations and include feature set combinations which are applicable to NE-DC. Band combinations supporting both NE-DC and (NG)EN-DC shall be included in supportedBandCombinationList, band combinations supporting only NE-DC shall be included in supportedBandCombinationListNEDC-Only.

includeNR-DC Only if this field is present, the UE supporting NR-DC shall indicate support for NR-DC in band combinations and include feature set combinations which are applicable to NR-DC.

omitEN-DC Only if this field is present, the UE shall omit band combinations and feature set combinations which are only applicable to EN-DC. This field is optionally present if includeNE-DC is set to true.

END OF FIRST CHANGE