3GPP TSG-RAN WG2 Meeting #108	R2-1915301

Reno, Nevada, USA, 18th – 22nd November 2019	Revision of R2-1912604

Agenda Item:	7.2.6

Source:		Huawei, HiSilicon

Title:	Discussion on SON RACH and RLF remaining open issues

Document for:	Discussion and Decision

# Introduction

This document is a revision of R2-1912604, with the following changes:

- RLF report in 5GC, update the proposal to align with eLTE

At RAN#80, the Rel-16 work item on additional enhancements for NB-IoT was approved [1]. One of the objectives in this work item is to support ANR reporting for network management.

Network management tool enhancement: - SON support for reporting of [RAN2, RAN3] - Cell Global Identity and strongest measured cell(s) (ANR) - Random access performance - Radio link failure (RLF), if needed

SON RACH and RLF have been discussed now in RAN2 for a number of meetings. Good progress has been achieved and most aspects have been captured in the running CRs [3], [4].

In this document, we discussed the last remaining aspects with the objective to finalise the feature.

# Discussion

## Common issues

Security for UE information procedure

In LTE, the UE information procedure can only be triggered when AS security has been activated. In NB-IoT, there is no AS security for the CP solution.

SON relies on UEs inputs thus these inputs should be reliable, which means they should be secured (i.e. integrity protected), otherwise any one could pretend to be other UE and report its wrong information, which may cause SON to output a wrong result.

Proposal 1: The UE information procedure can only be initiated after AS security has been activated.

Proposal 2: SON (RACH and RLF) is not supported when UE is using the control plane solution.

Processing delay

In the running CR [4], the value for the UE information procedure processing delay is set to TBD in Table 11.2-2: UE performance requirements for RRC procedures for NB-IoT UEs.

Per analogy with other procedures, we propose to set the value to 45 ms.

Proposal 3: The processing delay for the UE information procedure is 45 ms.

## RACH report

Mandatory vs Optional

RACH report involves very low complexity in the UE as the report only includes information that is available at the UE and is reported in a single procedure. Thus we propose the feature be mandatory w/o capability reporting.

Proposal 4: Support of RACH report is mandatory for rel-16 UEs.

Applicability to 5GC

RACH report is related to the usage of the radio resources and independent of connection to EPC or 5GC. Thus there should be no difference.

Proposal 5: RACH report applies to UE connected to EPC or 5GC. There is no difference in the reporting.

## RLF report

When to indicate availability and when to report

In LTE, the UE only indicates availability of the report and includes the report in UEInformationResponse message if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report. The same should apply to NB-IoT.

Proposal 6: The UE only indicates availability of the RLF report and only include the report in UEInformationResponse message if the RPLMN is included in the list of EPLMNs stored by the UE when RLF occurs.

When to discard the report

In LTE, the RLF report may be deleted after successful transmission has been confirmed, 48 hours after the failure is detected, upon power off or upon detach. The same should apply to NB-IoT.

Proposal 7: The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

In LTE, the UE keeps the RLF report during state transitions and RAT changes, and indicates RLF report availability again after it returns to LTE.

In NB-IoT, inter-RAT mobility is not supported and for SON-ANR, RAN2 has agreed that the UE discards the ANR report if it is not fetched by the eNB when the UE is in RRC_CONNECTED mode. The same approach could be apply to the RLF report.

Proposal 8: The UE discards the RLF report when returning to RRC_IDLE after having reported rlf-InfoAvailable.

Reporting in 5GC

One aspect to consider is which information the UE should report when the UE supports both EPC and 5GC and when the serving cell is connected to both EPC and 5GC.

In eLTE, no change has been introduced to the RLF report or to the reporting procedure for 5GC. The EPC cell identity in EPC is included in the report. The same approach should be followed in NB-IoT.

Proposal 9: RLF report applies to UE connected to EPC or 5GC. There is no difference in the reporting.

Other parameters in the report

In the RRC running CR [4], there is an editor’s note as below:

Editor’s Note: FFS further information for RLF report.

In LTE, the Global Cell Identity of the re-establishment cell is also included in the RLF report. However, this is only useful if the report can be fetched in a different cell from the one where the re-establishment occurred, which will not happen in NB-IoT if proposal 8 is agreed

Proposal 10: There is no need to include the global cell identity of the re-establishment cell in the RLF report if proposal 8 is agreed.

Mandatory vs Optional

RLF report has some complexity at the UE, it impacts several procedures and requires the UE to keep the data across connections, so we propose the feature to be optional at the UE. There is no need for capability reporting as the eNB asks for the report only if the UE has indicated the availability of a report.

Proposal 11: Support of RLF report is optional at the UE without capability reporting.

# Conclusion

In this document, we have discussed some remaining issues for SON RACH and RLF reports and made the following proposals:

Proposal 1: The UE information procedure can only be initiated after AS security has been activated.

Proposal 2: SON (RACH and RLF) is not supported when UE is using the control plane solution.

Proposal 3: The processing delay for the UE information procedure is 45 ms.

Proposal 4: Support of RACH report is mandatory for rel-16 UEs.

Proposal 5: RACH report applies to UE connected to EPC or to 5GC. There is no difference in the reporting.

Proposal 6: The UE only indicates availability of the RLF report and only include the report in UEInformationResponse message if the RPLMN is included in the list of EPLMNs stored by the UE when RLF occurs.

Proposal 7: The UE may discard the radio link failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

Proposal 8: The UE discards the RLF report when returning to RRC_IDLE after having reported rlf-InfoAvailable.

Proposal 9: RLF report applies to UE connected to EPC or 5GC. There is no difference in the reporting.

Proposal 10: There is no need to include the global cell identity of the re-establishment cell in the RLF report if proposal 8 is agreed.

Proposal 11: Support of RLF report is optional at the UE without capability reporting.

# References

RP-192313 “WID revision: Additional enhancements for NB-IoT”, Futurewei, RAN#85, Newport Beach, USA, September 2019

R2-1914102 “RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC”, BlackBerry, RAN2#107bis, Chongqing, China, October 2019

R2-1914094 “Introduction of additional enhancements for NB-IoT in TS 36.300”, Huawei, RAN2#107bis, Chongqing, China, October 2019

R2-1914101 “Introduction of additional enhancements for NB-IoT in TS 36.331”, Huawei, RAN2#107bis, Chongqing, China, October 2019