3GPP TSG-RAN WG2 #108	Tdoc R2-1915405

Reno, Nevada, US, 18 – 22 November 2019	Revision of R2-1913119

Agenda Item:	7.2.4

Source:	Ericsson

Title:	PUR periodicity and UE multiplexing

Document for:	Discussion, Decision

# 1	Introduction

The Rel-16 Work Item Descriptions for LTE-M [1] and NB-IoT [2] contain a common objective on improving the uplink transmission efficiency and/or UE power consumption by means of transmission in preconfigured resources:

Improved UL transmission efficiency and/or UE power consumption:

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance [RAN1, RAN2, RAN4]

  - Both shared resources and dedicated resources can be discussed

  - Note: This is limited to orthogonal (multi) access schemes

In this revision we continue to discuss how it can be ensured that the periodic resources of different PUR UEs do not overlap, and new evaluation results have been added to give input to the discussion of the PUR periodicity is configured (apart from the update of the ‘PUR start position’ with the FFS from RAN2#107bis, this paper is a resubmission).

# 2	Discussion

In the following, dedicated PUR in Idle mode is discussed unless mentioned otherwise.

## 2.1 SFN-based PUR

The PUR start position and the PUR periodicity can either be based on clock time or SFN. The benefit of clock time is that for longer periodicities it may be more natural for the application layer, e.g. to transmit a sensor report every 24h. An SFN based solution would be limited to ~3h (SFN and H-SFN numbering 1048576 radio frames), but additional H-SFN bits could be kept track of in the network and communicated to the UE as part of the PUR configuration if longer periodicities are desired. Furthermore, an SFN based periodicity would drift over the clock hours such that it would not be possible to consistently configure a report e.g. at midnight every day. This is not a severe drawback and it could easily be avoided by the one-shot PUR if needed. The problem with the clock-based solution is that it is difficult to unambiguously define the subframe for transmission. That is, due to clock drift, the lack of SFN to clock time mapping, etc. the UE and NW may end up having different understanding of in which subframe the PUR transmission should take place. In other clock-based solutions the UE or the network triggers a transmission at the expiration of a timer. For example, TAU is triggered in the UE for Rel-12 PSM. However, for these solutions it is not required that the UE and eNB have a common understanding of which subframe the transmission will take place in. For the unambiguous definition of the PUR transmission subframe, the same principle as for paging must be applied.

PUR periodicity and start position are based on legacy counters: subframe, SFN, H-SFN.

## 2.2 PUR periodicity range

Regarding having PUR periodicities longer than the 3h enabled by H-SFN, there seem to be little need for that according to our PUR performance evaluations. The UE battery life gain from using PUR compared to Rel-15 EDT is shown in Figure 1 (eDRX cycles of 43 min, 50 byte payload, PTW of 2.56s with DRX cycle 1.28s are applied. See [2] for more detail on the simulation procedure).

Figure 1: Battery life gain as a function of the inter-arrival time of the uplink reports.

It is seen that there is little motivation to support PUR periodicities longer than H-SFN wrap-around of 3h since PUR gains will be less than 5% at 164 dB MCL or less than 1% in good coverage.

PUR periodicity of up to at most 3h is supported.

The exact range for the PUR periodicity is still an open issue. For the minimum value, since PUR transmission is in Idle mode, it does not make sense to have periodicities shorter that the network would keep the UE in RRC Connected, e.g., on the order of a couple of seconds. (For SPS in connected the longest semiPersistSchedInterval is 640 ms but extended to 1 s in UL in Rel-14). For the maximum value, for very long PUR periodicities the UE gain becomes insignificant as shown above. (This since the UE power consumption will be dominated by the monitoring of paging and other Idle mode procedures for very infrequent transmissions). For the ‘Improved UL transmission efficiency’ of this WI-objective and the system performance impact there may still be gains however. For the discussion of the determination of the PUR periodicity, an example of 2n PUR periodicities over the H-SFN is given in Table 1.

Table 1: Example of PUR periodicity radio frame to time conversion.

| SFN: | 2 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | 512 | 1024 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time: | 0.02s | 0.04s | 0.08s | 0.16s | 0.32s | 0.64s | 1.28s | 2.5s | 5s | 10s |
| SFN: | 2048 | 4096 | 8192 | 16384 | 32768 | 65536 | 131072 | 262144 | 524288 | 1048576 |
| Time: | 20s | 41s | 1.4min | 2.7min | 5.5min | 11min | 22min | 43min | 1.5h | 3h |

Based on the above analysis we propose the following range for the PUR periodicity.

3-bits or 4-bits is used for signaling PUR periodicity in the range from SFN=256 (2.5s) to SFN=1048576 (3h).

## 2.3 PUR start position

For the PUR start position, the UE must know the starting subframe. The remaining open issue is whether the UE should be able to request a certain time offset to the PUR configuration time to be used. This was listed as an FFS in RAN2#107bis:

FFS:

- Whether Requested Time Offset can be included in the PUR request.

For example, 20 SFN bits could be used to point out the radio frame, and 3-4 bits to point out the starting subframe, depending on if all 10 subframes are viable candidates or if some non-valid subframes can be excluded (note that a bitmap is not required since there is only one starting point). However, 24 bits adds a significant amount to the signaling overhead and it could be questioned if full flexibility is required. I.e. the PUR start position could instead be related the to the time of the configuration. E.g. if a sensor transmits reports every 2 hours the first report, in which PUR is initially configured, will be 2h ahead of the first PUR transmission.

PUR start position is configured relative to the time of configuration to reduce the signaling overhead.

## 2.4 Multiplexing of periodic PUR UEs

For both LTE-M and NB-IoT it is desirable to be able to multiplex PUR UEs in the same narrowband or carrier. For one-shot PUR this is straight forward since, as for legacy dynamic transmissions, the scheduler would need to check that no other UEs has been assigned to the radio resource before granting the UE a PUR resource. For periodic PUR, however, without any requirement on the PUR periodicity UEs may eventually collide. This is schematically illustrated for transmission without CE in Figure 2. First, obviously UEs must be provided with a unique PUR start position (either on radio frame or subframe level). Doing that ensures that UEs with the same periodicity will not collide (UE1 and UE2). If the periodicity is a factor of 2 for a third UE3, it is also ensured that there will be no collisions. However, for an arbitrary UE periodicity there will eventually be collisions (UE4 with a non-2n period).

Figure 2: Illustration of the need for requirements on the PUR periodicity.

Therefore, to enable multiplexing of periodic PUR UEs we propose that the granularity for the PUR periodicity is based on a power of 2. For example, as 10*2n subframes (or radio frames). That is, to avoid collision it must be ensured that:

- PUR starting positions are unique

- PUR periodicity is defined as a power of 2.

- The total time for PUR transmission (and the associated SS Window) for all multiplexed UEs is shorter than the minimum PUR period (See Figure 3).

The PUR periodicity parameter is quantized as a power of 2 to allow for multiplexing of PUR UEs.

It is then straight-forward to multiplex UEs with the same periodicity, as indicated for UE1 and UE3 in Figure 3. Using a PUR periodicity which is a power of 2 would enable the scheduler to also multiplex UE with other periodicities, e.g. UE2 with double the periodicity in Figure 3.

Figure 3: Illustration of PUR UE multiplexing.

Note that the proposed SFN values in Table 1 are a given as 2n so any of these values automatically satisfies the conditions of Proposal 5.

Also note that only the NW (scheduler) is concerned with the relative start positions for the different UEs, the UE itself would only need to know its own PUR start position and PUR periodicity.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	PUR periodicity and start position are based on legacy counters: subframe, SFN, H-SFN.

Proposal 2	PUR periodicity of up to at most 3h is supported.

Proposal 3	3-bits or 4-bits is used for signaling PUR periodicity in the range from SFN=256 (2.5s) to SFN=1048576 (3h).

Proposal 4	PUR start position is configured relative to the time of configuration to reduce the signaling overhead.

Proposal 5	The PUR periodicity parameter is quantized as a power of 2 to allow for multiplexing of PUR UEs.

# References

RP-191356, “Revised WID for Additional MTC enhancements for LTE”, Ericsson, RAN#84, Newport Beach, USA, June 2019.

RP-192313, “WID revision: Additional enhancements for NB-IoT”, Futurewei, RAN#85, Newport Beach, USA, Sept 2019.

R2-1906940, “PUR L1 or L2/L3 acknowledgement”, Ericsson, RAN2#106, Reno, Nevada, US, May 2019