3GPP TSG-RAN WG2 meeting #108	R2-1915456

Reno, NV, USA, 18– 22 November, 2019	Revision of R2-1912365

Source: 	Huawei, HiSilicon

Title: 		Remaining issues for routing

Agenda Item:	6.1.3.1

Document for:		Discussion and Decision

- Introduction

In RAN2 #107 meeting, the following agreements about routing in IAB network are achieved[1]:

For routing and bearer mapping of a packet retrieved from RLC layer, the IAB-node needs to be configurable with the following mappings:

BAP routing ID in BAP header  Egress link (routing table)

Ingress RLC channel Egress RLC channel (bearer mapping)

For the selection/addition of a BAP routing ID as well as routing and bearer mapping for a packet retrieved from upper layers, the IAB-node and IAB donor needs to be configurable with the following mappings:

(FFS) Upper layer information  BAP Routing ID to be added in BAP header

BAP routing ID in BAP header  Egress link

Upper layer information (FFS)   Egress RLC channel

In RAN3 #105 meeting, we have the following agreements about routing related configuration for DL[2]

On the DL, the IAB-donor DU is configurable with information that allows deriving the BAP routing ID from IP header information for F1-U, F1-C and non-F1 traffic.

On the DL, the IAB-donor is configurable with information that allows deriving the BAP address from the destination IP address.

In RAN3 #105bis meeting, the following agreement is achieved for DL routing[3]

configuration of downlink bearer mapping and routing should be performed by F1-AP

path id is derived from IP header and mapping provided by CU

In the DL, for BAP path id derivation on the donor DU: IP address, IPv6 flow level and/or DS/DSCP can be used; all of these fields are optional in F1AP message to configure routing

In this contribution, we are going to analyse more details about routing for wireless backhaul links in an IAB network, based on the above agreements.

- Discussion

RAN2 agrees that “BAP has a DU part configured by F1-AP and a MT part configured by RRC” in RAN2 #107 meeting, and RAN3 confirms that the DL bearer mapping and routing should be performed by F1-AP. Thus, we suggest to clarify that the BAP parameters for DU part should be defined in F1AP spec, while the BAP parameters for MT part are defined in RRC spec.

Proposal 1: In principle, the BAP parameters for DU part are defined in F1AP ASN.1, and the BAP parameters for MT part are defined in RRC ASN.1.

Based on the discussion progress about IAB routing, there are still several issues to be addressed, and we will discuss them in the following part.

Issue 1: Which BAP routing ID to be added in BAP header for UL

For the UL transmission, the access IAB node should decide which BAP routing ID will be added to a given BAP SDU according to configurations received from the IAB-donor-CU. Each BAP routing ID includes a BAP address, and may also include a path ID. Therefore, the IAB-donor-CU should configure one or more BAP routing ID for upstream transmission to the access IAB node. To limit the occupied bits of BAP path ID, we assume that the path ID is unique to a given destination BAP address. Thus, the configuration of available upstream BAP routing ID for access IAB node can be split into two parts, one is to configure the destination BAP address, and another is to configure how to choose the BAP path ID with a given destination BAP address.

- BAP address to be added in BAP header at access IAB node

The agreements about the definition of BAP address is “Each BAP address defines a unique destination (unique for IAB network of one Donor, either an IAB access node, or the IAB donor)” [4]. For upstream destination, it has not been clarified whether the BAP address should identify an IAB donor CU or the IAB donor DU.

We think using the BAP address to identify the IAB donor DU is reasonable since the BAP layer based routing will be performed across the wireless BH links, IP based routing will be used for the intra donor routing. Otherwise, if the BAP address is used to identify the IAB donor CU, each IAB donor DU will be responsible to forward packets with different BAP address carried in upstream packets, and multiple IAB donor DUs will share same BAP address only if these DUs connects to a same IAB donor CU-UP. In such case, it is meaningless for the IAB donor DU to check whether the BAP address of upstream packet is same as the one configured to it or not. To be honest, this is workable, but will defy the BAP layer receiving procedure of IAB node, and will cause split receiving operation for BAP layer specification. In addition, such way will prevent the IAB donor CU to do load balancing among different IAB donor DUs.

Observation 1: If assuming the BAP address in the BAP header is to identify the destination donor CU, BAP PDU with different BAP addresses may be transmitted via one donor DU.

Therefore, in the following analysis of this paper, we assume that the BAP address in upstream packets is used to identify the IAB donor DU. However, RAN2 still needs to ask RAN3 to confirm such assumption.

Proposal 2：For upstream, the BAP address in BAP header should be used to identify the destination IAB donor DU, rather than the IAB donor CU.

Proposal 3: RAN2 sends LS to RAN3 to confirm that the BAP address in BAP header is to identify the destination IAB donor DU for upstream.

Accordingly, similar to the BAP address configuration to each access IAB node, the BAP address of IAB donor used for upstream transmission should be configured to the IAB donor DU, which is used by the IAB donor DU to identify packets to be delivered to upper layers.

Proposal 4: For upstream, the BAP address needs to be configured to the IAB donor DU, to identify the traffic to be delivered to upper layers.

The following two options are proposed for the upstream BAP address configuration.

Option 1: Configure destination BAP address for upstream transmission per access IAB node. This is a simple way, where IAB donor CU only configures one destination BAP address for upstream transmission, with the assumption that the appointed IAB donor DU of the destination BAP address has the connection to all the CU-UPs the access IAB node connects to. In such a way, the IAB donor CU can do global load balancing for uplink transmission among multiple IAB donor DUs, through assigning different destination BAP address to different access IAB nodes. The granularity of such global load balancing is per IAB node.

Option 2: Allow the access IAB node to choose different destination BAP addresses in BAP layer according to upper layer information, e.g. the destination IP address.  In this way, the IAB donor CU can configure more than one UL destination BAP address for an access IAB node. To an access IAB node, each UL BAP address is assigned with one or more destination IP addresses of upstream packet. IAB donor CU will ensure that a destination IP address only be mapped to one UL BAP address in the configuration for an access IAB node. This option enables the IAB donor CU to do load balancing in a more flexible way, compared to option 1.

Proposal 5: For upstream, the BAP address to be added in BAP header is configured based on the destination IP address in the BAP SDU.

- BAP path ID to be added in BAP header at access IAB node

In addition to the BAP address configuration, the IAB donor CU may optionally configure one or multiple BAP path IDs towards each BAP address. If more than one path IDs are configured to a given BAP address, the access IAB node needs to decide which path ID to be added in an upstream packets. IAB donor CU may control such decisions for in access IAB nodes in a centralized way, through e.g. assigning a priority level value to each allocated path ID, or configure a mapping relationship from upper layer information to the path ID. The priority level based solution is not as flexible as the upper layer info based solution for the access IAB node, since the access IAB node will always select the path ID with the highest priority unless this path suffers from BH RLF, while the upper layer info based solution enables the access IAB node choose different paths for different packets towards a same destination BAP address. The upper layer information can be the information of UE DRB (i.e. identified by the Destination IP address + GTP-TEID). As to the CP signalling, the similar information can be used as in bearer mapping configuration, e.g. F1-AP message type.

If no BAP path ID is configured for a packet towards a BAP address, the default value will be added to BAP header by the access IAB node and the IAB node can select any next hop to transmit this packet. In some scenarios, if multiple BAP path IDs are configured towards same BAP address in the routing configuration information, and no preference is configured by the IAB-donor-CU, then the access IAB node can determine a preferred one by itself.

Proposal 6: For upstream, the path ID to be added in BAP header is configured based on Destination IP address + GTP-TEID or F1-C message type in the BAP SDU, in case multiple paths towards the same destination are configured.

Issue 2: How to handle the RLF case

According to section 2.1, the access IAB node will add the BAP routing ID for UL packets and IAB-donor-DU will add BAP routing IDs for DL packets, and thus, it is possible that the access IAB node/IAB-donor-DU adds a preferred BAP path ID to a packet, but some BH links along this path may suffer from RLF. To ensure lossless transmission, upon RLF, the intermediate IAB node can forward the packets through other candidate paths. For example, as shown in Figure 1, the link between IAB node 3 and IAB node 1 is failed, but there are still some packets with BAP path ID 2 in IAB node 3, IAB node 3 can send those packets with BAP path ID 2 to the IAB donor via IAB node 2.

When the intermediate IAB node needs to choose another next hop link different from the one indicated by the entry of the BAP path ID and BAP address, the IAB node may look up the routing table and find another entry with same BAP address by implementation or take the priority level into consideration if the priority level of each path is configured.

Furthermore, it is also possible for the access IAB node or the IAB donor to select other paths if it knows that some link along the path with the highest priority level is suffering from RLF. For example, the access IAB node or the IAB donor DU may detect that the first hop of the path is failure, or receive RLF notification from intermediate IAB nodes. In such case, the selection of BAP path ID from other inferior candidate paths to same BAP address can also be based on implementation or according to the configured priority level of each path.

.

Figure 1. An example of BH link RLF

Proposal 7: If the path with highest priority (or indicated by path ID in BAP header) suffers BH RLF, BAP layer selects the other alternative path belonging to the same BAP address by implementation or based on the priority if configured.

Issue 3: How to handle the load balancing

In RAN2 #106 meeting, the agreements about routing show that “For load balancing still FFS what is decided locally and/or decided by the Donor.”  In fact, both the global load balancing and local balancing are easy to be achieved through implementation.

First, it is easy for the IAB-donor-CU to do global load balancing through configuring how to choose the path ID according to upper layer information or assigning preferred path ID with highest priority to the IAB-donor-DU and IAB nodes.

In addition, for the source node, i.e. the access IAB node or the IAB-donor-DU, if it is aware that the CU preferred path suffers from congestion, it can choose an alternative path with second priority level if configured or totally by the local decision of source node through implementation, and add the alternative BAP path ID to the BAP SDU.

Moreover, for intermediate IAB nodes, if the path indicated by the BAP path ID is congested or will overload, an intermediate IAB node can choose an alternative path towards a same BAP address also, either according to the configured priority level of each path or local decision by implementation.

Proposal 8: It is up to the IAB donor implementation whether the path selection for local load balancing is allowed, i.e. the donor decides whether to configure the preferred path in the redundant paths case.

Issue 4: Whether the BAP path ID can be changed by intermediate IAB node

Based on the analysis in issue 2 and issue 3, it is obvious that re-routing may occurs in intermediate IAB node for some special reasons, e.g. BH link suffers RLF, load balancing. It means that an intermediate IAB node may select another candidate path which is different from the path indicated by the BAP path ID carried in BAP header of a received packet towards the same destination BAP address. A new issue for the re-routing case is that whether the intermediate IAB node changes BAP path ID carried in BAP header or not when it select another path. There are three options to handle such case, as listed in the following.

Option 1. Keep the original BAP path ID

In case of path switching in RLF, if the BAP path ID is not changed, all the following other intermediate IAB nodes in the new path may be confused about the BAP path ID in BAP header, since this BAP path ID is different from their already configured path IDs towards the corresponding BAP address. There are two possible cases for subsequent IAB nodes in the new forwarding path after re-routing. To be frank, both of them are abnormal cases.

Figure 2. An example of re-routing case

The first one is that some subsequent IAB nodes may be configured with BAP path ID which is conflicted with the unchanged BAP path ID carried in rerouted packets. For example, as shown in Figure 2, the upstream path IAB7IAB6IAB5IAB1IAB donor DU is identified by BAP path ID 1, and the UL routing configuration to IAB node 6 will including the routing entry of this BAP path ID 1. For IAB 4, we can see there are two UL paths to the IAB donor DU, the first one IAB4IAB2IAB donor DU is named as BAP path ID 1, while the second one IAB4IAB3IAB donor DU is named as BAP path ID 2. Such naming case is possible because the red dashed path and the greed dashed path are not overlapped except the same destination node. If some link in the red dashed path, e.g. the BH link between IAB 5 and IAB 6 suffers RLF, the IAB 6 can do rerouting for packets received from IAB 7 if the BAP path ID=1. Obviously, the IAB 6 will reroute these packets to IAB4 without change the BAP path ID. When IAB 4 receives these rerouted packets, IAB 4 will forward these packet to IAB2 after checking routing table, because IAB 4 is configured with a routing entry of BAP address=IAB donor DU and BAP path ID=1.  Such case is still workable for upstream data forwarding, but has disadvantage of causing overburden to the green dashed path, although the orange dashed path can also be used by IAB 4 to forward the re-routed packets received from IAB 6. Otherwise, more restrictions should be considered when allocating the BAP path ID to avoid such confliction when re-routing occurs.

The second abnormal case is that some subsequent IAB nodes is not able to select an entry for the unchanged BAP path ID carried in rerouted packets. To tackle with such abnormal case, some particular BAP behavior should be specified. For example, the BAP layer should support ignoring the BAP path ID if no routing entry is matched, and chooses any configured next hop node among the routing entries which have same destination BAP address as the BAP address carried in the packets.

Option 2a. The intermediate IAB node changes the BAP path ID to identify the actual path, which is selected when perform re-routing.

For example, the upstream paths from IAB 6 to the IAB donor DU are named as

BAP path ID=1: IAB6IAB5IAB1IAB donor DU

BAP path ID=2: IAB6IAB5IAB2IAB donor DU

BAP path ID=3: IAB6IAB4IAB2IAB donor DU

BAP path ID=4: IAB6IAB4IAB3IAB donor DU

When the BH link between IAB 6 and IAB 5 suffers RLF, the IAB 6 do rerouting for packets with BAP path ID=1, it can choose either the path identified by BAP path ID=3 or 4, and change the BAP path ID carried in the BAP header of the re-routed packets accordingly. This option will avoid the confliction case caused by option 1.

Option 2b. The intermediate IAB node changes the BAP path ID to the default value, e.g. all zero value.

Besides, another simple solution is enable the intermediate IAB node reset the BAP path ID to default value, e.g. all zero value, if re-routing is performed. And such option also need to specify the particular behaviour of BAP layer to handle with the default BAP path ID value, e.g. choose any configured entry towards the destination BAP address.

Based on the above analysis, option 1 may cause some confliction or require more restriction for allocating BAP path ID, thus we prefer option 2a and option 2b.

Proposal 9: If the path indicated by the path ID in the BAP header is not available (e.g. BH RLF), BAP resets the path ID in BAP header as one of the actual path to be re-routed, or resets the path ID to the default value, e.g. all zero values.

Issue 5: Size of BAP routing ID

As agreed in previous RAN 2 meeting, the BAP routing ID consists of BAP address and BAP path ID, the BAP address uniquely indicates a destination node. In last RAN2 107bis meeting, the following agreements are achieved for the BAP header [7].

Routing ID is 13bits

There is a C/D bit

Length of the BAP address and BAP path ID sub-fields of the BAP routing ID to be fixed/predefined

For the DL, BAP address is 10bits and BAP path ID is 3bits

For the UL, BAP address is FFS bits and BAP path ID is FFS bits

R2 expects that there will be no restrictions in the TS to restrict configuration of routing ID and its components. The network has to ensure that e.g. there is no path confusion.

For upstream transmission, the destination node is the IAB donor, and should be the IAB-donor-DU if IAB donor supports CU-DU split. For the downstream transmission, the destination node is access IAB node. It is straightforward that the number of downstream destination nodes is remarkably larger than the number of upstream destination nodes.

Observation 2: The amount of upstream destinations (i.e. the donor DU) is much smaller than that of downstream destinations (i.e. the access IAB node) in one typical IAB topology.

Figure 3. An example of IAB topology

The BAP path ID should indicate a unique path between the IAB-donor-DU and the access IAB node. For example, as shown in Figure 3, there are four UL paths between the IAB-donor-DU and the IAB node 6, i.e.

Path 1: IAB node 6IAB node 5IAB node 1  IAB-donor-DU

Path 2: IAB node 6IAB node 5IAB node 2  IAB-donor-DU

Path 3: IAB node 6IAB node 4IAB node 2  IAB-donor-DU

Path 4: IAB node 6IAB node 4IAB node 3  IAB-donor-DU

In addition, there are four UL paths from the IAB node 7 to the IAB-donor-DU:

Path 5: IAB node 7IAB node 5IAB node 2  IAB-donor-DU

Path 6: IAB node 7IAB node 5IAB node 1  IAB-donor-DU

Path 7: IAB node 7IAB node 4IAB node 2  IAB-donor-DU

Path 8: IAB node 7IAB node 4IAB node 3  IAB-donor-DU

If the global BAP path ID is configured by the IAB-donor-CU for each source node (i.e. the IAB-donor-DU for downstream, or access IAB node for upstream), the IAB-donor-CU can ensure that each configured path ID for a given source node indicates a unique path between the source node and a given destination node.

However, if the BAP path ID is only locally unique within one source node, there is a risk of the path ID collision at the intermediate IAB nodes, in case of forwarding the packets from multiple source node to a same destination node. For example, the IAB-donor-CU may assign “00”, “01”, “10”, and “11” to Path 1- Path 4 respectively, and configure the assigned 4 BAP path IDs to the IAB node 7. Similarly, it may also assign the “00”, “01”, “10”, and “11” to the Path 5-Path 8. For the intermediate IAB node 5, when it receives a packet, the BAP address shows that the destination node is IAB donor DU,  and the BAP path ID is “01”, then IAB node 5 may be confused when do routing selection, because both path 6 and path 2 have same destination node and same BAP path ID “01”, but the next hop node of the two paths are different (the next hop node should be IAB node 2 for path 2, but IAB node 1 for path 6).

Observation 3: BAP path ID collision may happen at intermediate IAB nodes, if the path ID is only locally unique within each source node.

The above example is about UL transmission, the problem is resulted from that an intermediate IAB node may be shared by multiple paths originated from different source IAB nodes. In fact, the DL transmission may also have similar problem if an IAB node can connect to multiple IAB donor DUs. To avoid such confusing problem of intermediate IAB nodes, there are two solutions, as listed in what follows.

Solution 1. The IAB-donor-CU ensures that each assigned BAP path ID is globally unique for each destination.

Solution 2. Adding information in the BAP address to indicate the source node ID in addition to the destination node ID.

The above two solutions are essentially similar. Both of them suggest to take the source node ID into consideration in the BAP routing ID. Solution 1 will somehow include the source ID info in the BAP path ID field, while the solution 2 uses BAP address to carry the source node ID information.

Based on observation 2, the number of IAB-donor-DU is remarkably less than the number of IAB nodes. Therefore, for downstream transmission, the number of bits needed to indicate the source node ID is short, while the destination node address ID will be long. For the upstream transmission, the lengths of the source node info and the destination node address are just opposite to the downstream situation.

However, in last RAN2 #107bis meeting, RAN2 agrees that the DL BAP routing ID is consisted of 10bits BAP address and 3bits BAP path ID. And the network will ensure no path ID confliction. In fact, we consider this means that the network will take some variation of solution 1 and somehow loosen the restriction of sharing path IDs among different path. For the UL, we suggest the BAP path ID should be longer than the DL since more source nodes will result in higher probability of path ID confliction, and the BAP address is shorter than the DL since the number of IAB donor DUs in an IAB donor should be very limited. We believe 3bits is enough to indicate the IAB donor DU, and the rest 10bits is used for UL BAP path ID.

Proposal 10: For UL, the BAP routing ID consists of 3bits BAP address and 10bits BAP path ID.

Issue 6: Next hop link/node for routing

In RAN2 #107 meeting, the next hop link/node ID for UL and DL are discussed, and it has been agreed that “For upstream, Cell group ID is used to identify next hop/egress link. For downstream FFS.”[1].

In the case of DL transmission, for a given current node (e.g. an IAB node, or the IAB donor DU), the next hop node is one of its child nodes. Thus the current node should identify a backhaul link towards its child node when do routing selection. In fact, the child node can be identified by various types of IDs. For example, the C-RNTI allocated by the current node for child node, the allocated DU F1AP UE ID for the child node, the BAP address of the child node, IP address of the child node, etc.

Considering that the DL scheduling and transmission of the DU part will use the child node’s C-RNTI, the current node should be aware of the C-RNTI of child node and the cell in which the child node is located, after the DL routing selection.  Thus, we can deduce that no matter which ID is used to identify the DL next hop link when the BAP layer do routing selection, such ID should be linked to the cell ID (identify the cell that serves the child node) + C-RNTI (identify the child node within the serving cell).

Furthermore, RAN2 has agreed that “the NR DC framework (e.g. MCG SCG related procedures) is used to configure dual radio links used as IAB BH links with two parent nodes.” in RAN2 #105bis meeting[6]. Therefore, there are at most 2 UL next hop link, i.e. two parent nodes, for a given IAB node.

Proposal 11: For upstream, at most two next hop IDs are configured for the same destination, in the routing table of an IAB node.

Considering some special scenario, only one next hop node is connected for some nodes, e.g. parent node only has one child node, or child node only has one parent node. In such scenario, the next hop node is the default one to be chosen, which may not need explicit routing configuration from the IAB donor CU. If no routing configuration is explicitly provided, an IAB node will deliver the received packet to the only next hop node in default way if the BAP address carried in the BAP layer is not the IAB node’s own BAP address. Such way can reduce some routing configuration related signalling.

However, the explicit routing configuration can provide extra benefits in some abnormal case. For example, for an IAB node, if it receives a packet which carries a destination BAP address but it cannot find any routing entry for this destination BAP address, it can think this packet is wrongly forwarded to itself and just discard this packet. In such way, it can prevent some radio link resource wastage caused by forwarding the packet in wrong BH links. But using default forwarding way without explicit routing entry configuration cannot prevent such resource wastage.

Whatever, whether we need such simplification for routing configuration is worth to be discussed for further step.

Proposal 12: RAN2 discusses whether it is allowed not to configure the routing table, if there is only one next hop link at the IAB node.

Issue 7: RLF handling of routing for the IAB node connecting to multiple donor DUs

In RAN2 106 meeting, the following agreements about routing in IAB network was achieved [4].

Each BAP address can have one or multiple entries in the routing table to enable local route selection. Multiple entries is for load balancing, re-routing at RLF. For load balancing still FFS what is decided locally and/or decided by the Donor.

To avoid data loss in IAB network, packets need to be re-routed in the RLF case. As analysed in section 2.2, if multiple entries are configured to a node (either the source node in wireless backhaul link, or intermediate IAB nodes) for a given BAP address, it is possible for this node to choose an alternative routing path if the path indicated by the carried BAP path ID suffers RLF.

As described in section 2.1, we assume the upstream destination BAP address identifies the IAB donor DU instead of IAB donor CU. Therefore, multiple uplink routing entries relates to a given BAP address will provide multiple paths to an IAB-donor-DU. To provide link redundancy in case of BH link RLF, it is possible for an IAB node to connect to multiple IAB-donor-DUs, and have one or more paths towards each IAB-donor-DU.

For example, as shown in Figure 4, IAB node 5 can connect to 3 different IAB-donor-DUs belong to a same IAB donor. There are two different paths between the IAB-donor-DU 3 and the IAB node 5, i.e. IAB-donor-DU 3↔ IAB node 1 ↔ IAB node 3↔ IAB node 5, and IAB-donor-DU 3↔ IAB node 2 ↔ IAB node 4↔ IAB node 5. Only one path exists between the IAB-donor-DU 1 and the IAB node 5, i.e. IAB-donor-DU 1↔ IAB node 1 ↔ IAB node 3↔ IAB node 5. Similarly, there is only one path between the IAB-donor-DU 2 and the IAB node 5, i.e. IAB-donor-DU 2↔ IAB node 2 ↔ IAB node 4↔ IAB node 5.

Figure 4. Scenario of BH link redundancy for IAB networks

Based on the example shown in Figure 4, if some link of one path between the IAB-donor-DU 3 and the IAB node 5 is RLF, another alternative UL path can be selected by the IAB node 5 for re-routing. If some link between IAB node 5 and the IAB-donor-DU 1/2 is failure, no alternative path can be used by IAB node 5 for UL re-routing. In fact, if the IAB-donor-CU is not CP-UP split, each IAB-donor-DU can forward the UL packet to the IAB-donor-CU. Even in the case that the IAB-donor-CU consists of one IAB donor-CU-CP and multiple IAB-donor-CU-UPs, the IAB-donor-CU has connections to each IAB-donor-DU, if we can make the following assumption 1, any IAB-donor-DU of an IAB donor can forward the UL UP packet to the right IAB-donor-CU-UP, and forward the UL CP packet to the IAB-donor-CU-CP.

Assumption 1: In an IAB donor, each IAB-donor-DU can connect to all the IAB-donor-CU-UPs.

With the above assumption, when some link of the preferred path to an IAB-donor-DU is failure, inter IAB-donor-DU re-routing may be used for robust UL transmission.

Proposal 13: RAN2 asks RAN3 to confirm the assumption 1, i.e. in an IAB donor, each IAB-donor-DU can connect to all the IAB-donor-CU-UPs.

If the assumption 1 is confirmed, the upstream data can be transmitted via any IAB-donor-DU to the target CU-UP. If the path to the configured or preferred IAB node DU fails, IAB node can switch to another path to the other alternative IAB donor DU within the same donor CU. The alternative IAB donor DU has different BAP address with the original one in the BAP header. In the above path failure case, IAB node can select any next hop link to perform routing, which is not based on the BAP address in BAP header and routing table. This can be considered as the inter donor DU path switch or path redundancy.

Proposal 14: If assumption 1 is confirmed, IAB node can select any other alternative IAB-donor-DU as the destination, if the path to an IAB-donor-DU identified by the BAP address in BAP header fails.

If the assumption 1 is not valid, RAN2 may need some standard efforts to support the inter IAB-donor-DU path switching or redundant, since which donor DUs are connected to the same CU-UP needs to be known by IAB node. Otherwise, if such inter-donor-DU path switch is not supported, those upstream packets has to be stored at intermediate IAB node until the path to the given destination IAB-donor-DU identified by the BAP address is recovered.

Observation 4: If the assumption 1 is not valid, RAN2 may need some standard efforts to support the inter IAB-donor-DU path switching or redundant in case of BH RLF.

- Conclusion

This paper mainly discusses remain issues about routing for IAB networks, then we draw the following observation and proposals:

Observation 1: If assuming the BAP address in the BAP header is to identify the destination donor CU, BAP PDU with different BAP addresses may be transmitted via one donor DU.

Observation 2: The amount of upstream destinations (i.e. the donor DU) is much smaller than that of downstream destinations (i.e. the access IAB node) in one typical IAB topology.

Observation 3: BAP path ID collision may happen at intermediate IAB nodes, if the path ID is only locally unique within each source node.

Observation 4: If the assumption 1 is not valid, RAN2 may need some standard efforts to support the inter IAB-donor-DU path switching or redundant in case of BH RLF.

Proposal 1: In principle, the BAP parameters for DU part are defined in F1AP ASN.1, and the BAP parameters for MT part are defined in RRC ASN.1.

Proposal 2：For upstream, the BAP address in BAP header should be used to identify the destination IAB donor DU, rather than the IAB donor CU.

Proposal 3: RAN2 sends LS to RAN3 to confirm that the BAP address in BAP header is to identify the destination IAB donor DU for upstream.

Proposal 4: For upstream, the BAP address needs to be configured to the IAB donor DU, to identify the traffic to be delivered to upper layers.

Proposal 5: For upstream, the BAP address to be added in BAP header is configured based on the destination IP address in the BAP SDU.

Proposal 6: For upstream, the path ID to be added in BAP header is configured based on Destination IP address + GTP-TEID or F1-C message type in the BAP SDU, in case multiple paths towards the same destination are configured.

Proposal 7: If the path with highest priority (or indicated by path ID in BAP header) suffers BH RLF, BAP layer selects the other alternative path belonging to the same BAP address by implementation or based on the priority if configured.

Proposal 8: It is up to the IAB donor implementation whether the path selection for local load balancing is allowed, i.e. the donor decides whether to configure the preferred path in the redundant paths case.

Proposal 9: If the path indicated by the path ID in the BAP header is not available (e.g. BH RLF), BAP resets the path ID in BAP header as one of the actual path to be re-routed, or resets the path ID to the default value, e.g. all zero values.

Proposal 10: For UL, the BAP routing ID consists of 3bits BAP address and 10bits BAP path ID.

Proposal 11: For upstream, at most two next hop IDs are configured for the same destination, in the routing table of an IAB node.

Proposal 12: RAN2 discusses whether it is allowed not to configure the routing table, if there is only one next hop link at the IAB node.

Proposal 13: RAN2 asks RAN3 to confirm the assumption 1, i.e. in an IAB donor, each IAB-donor-DU can connect to all the IAB-donor-CU-UPs.

Proposal 14: If assumption 1 is confirmed, IAB node can select any other alternative IAB-donor-DU as the destination, if the path to an IAB-donor-DU identified by the BAP address in BAP header fails.

- Reference

- Report of the 3GPP RAN WG2 #107 meeting.

- Chairman notes of the 3GPP RAN WG3 #105 meeting.

- Chairman notes of the 3GPP RAN WG3 #105bis meeting.

- Report of the 3GPP RAN WG2 #106 meeting.

- R2-1905063, Summary of email discussion [105#46][IAB] Routing, LG Electronics Inc.

- Report of the 3GPP RAN WG2 #105bis meeting.

- Report of the 3GPP RAN WG2 #107bis meeting.