3GPP TSG-RAN WG2 #108	R2-1915603

Reno, Nevada, USA, 18th – 22nd November 2019

Agenda Item:	6.13.2

Source:	Ericsson

Title:	New msgB formats for 2-step RA

Document for:	Discussion, Decision

# 1	Introduction

In current Rel-15, the gNB may signal back off in the msg2 RAR which entails that an UE receiving a RAR with the E/T/R/R/BI mac sub header but no 'MAC subPDU(s) with RAPID and MAC RAR'; with RAPID matching its preamble transmission, the UE will back off for a random time between 0 and a time indicated by the BI field before doing a new preamble transmission attempt, i.e. return to Random Access Resource selection (Section 5.1.2 in 38.321).

This is useful in e.g. load situations when colliding UEs compete for available PRACH resources.

For 2-step RA, the ongoing email discussion on MsgB format design states that

  - The contents of backoff MAC subPDU are not explicitly discussed. However the general understanding is that backoff MAC subPDU includes a MAC subheader and there is no payload as in 4 step RA. MAC subheader at least includes 4 bit BI field.

In this contribution, use of a 2-step specific back off and fallback indicator(s) are discussed.

# 2	Discussion

## 2.1	2-step Random Access

The 2-step RA is assumed to give much shorter latency than the ordinary 4-step RA. In the 2 step RA the preamble (transmitted on PRACH) and a message corresponding to Message 3 (transmitted on PUSCH) in the 4 step RA are transmitted in the same or in two subsequent sub frames. The first message in the 2-step procedure is denoted Message A (msgA). The 2-step procedure is depicted in Figure 1. In the 4-step procedure, the gNB the grant is linked to a particular preamble. The same kind of mapping will be needed in the 2-step procedure. For all different preamble ids that have been configured for the 2-step there must be a mapping to a particular PUSCH resource. Upon successful reception of msgA (i.e. both the preamble and PUSCH), the gNB will respond with a msgB. Details are still discussed but it would typically contain TA (which by assumption should not be needed or just give very minor updates) and a CR id for contention resolution.

Figure 1 Illustration 2-step RA.

In case the UE does not receive a msgB, it would re-try with a new msgA, similar to the action taken by the UE which does not receive a RAR in the 4-step procedure.

It is also possible to do fallback to 4-step by transmitting a new preamble indicating 4-step procedure when the configured maximum number of msgA transmissions has been reached.

Back off signaling using msgB serves the purpose of reducing the load on 2-step resources (both PRACH and PUSCH) by ordering the UEs to retry after a random back off time. Since the 2-step RA will be used by all UEs that fulfill the conditions for using the 2-step RA the load on the 2-step resources can be expected to be high. At the same time, the 2-step resources are expensive to configure which further increases the risk of high load since the 2-step configuration can be expected to be slim not to waste resources.

## 2.2	How to signal back off in 2-step RA

In the 4-step procedure, the gNB may signal back off in the RAR. This is indicated by a MAC subheader with Back off Indicator which consists of five header fields E/T/R/R/BI as described in Figure 6.1.5-1 of 38.321 shown below.

Figure 6.1.5-1: E/T/R/R/BI MAC subheader.

If a UE receives a RAR with the E/T/R/R/BI mac subheader but no 'MAC subPDU(s) with RAPID and MAC RAR' with RAPID matching its preamble transmission, the UE will  for a random time between 0 and a time indicated by the BI field before doing a new preamble transmission attempt, i.e. return to Random Access Resource selection (Section 5.1.2 in 38.321).

In the 2-step procedure, details of back off mechanism is not yet agreed. In the ongoing email discussion [107bis#76] [2-step RACH]: MSG B Format Design, the two solutions preferred by most companies have a 4 bit BI field and 2 or 3 bits to signal formats leaving 1 or 2 R-bits.

The solutions preferred by most companies in the email discussion [107bis#76] [2-step RACH]: MSG B Format Design have at least one unused R-bit.

We also believe that since the load on 2-step RA resources and 4-step RA resources may be very different, back off signaling for 2-step UEs should only apply for the 2-step RA. If e.g. back off has been signaled and a UE reaches the maximum allowed msgA transmissions, the signaled back off does not apply when the UE falls back to 4-step preamble transmission. We propose

Back off signaling received as a response to msgA transmission does not apply if the UE falls back to 4-step preamble transmission.

The unused bits in the msgB BI subheader could be used to indicate a more detailed back off including switch or fallback to 4-step preamble transmission. This could be used to move users between 2-step and 4-step in case the network experiences high load on the 2-step resources while the load on the 4-step resources are low or moderate.

The unused R bits in the msgB BI subheader could be used to indicate different forms of back off for the 2step procedure.

Therefore, we believe that using a reserved bit to indicate either back off and continue with 2-step or back off and fallback to 4-step can be done as follows:

- if the R-bit is set to 0, the 2-step UE should do back off according to the BI value and continue with a msgA transmission.

- if the R-bit is set to 1, the 2-step UE should do back off according to the BI value and continue with a preamble transmission in the 4-step procedure.

We propose

One R-bit in the msgB BI subheader should be used to differentiate if after back off, the UE should continue with msgA transmission or fallback to preamble transmission in the 4-step procedure.

## 2.3	New fields in msgB

The discussions in RAN1 are ongoing of the need for additional information in msgB. Minimizing specification changes and trying to reuse the release 15 PUCCH resource determination should be the goal for 2-step RACH at this stage.

When multiple UEs are multiplexed in msgB, all PUCCH resource and timing related parameters for all UEs should be put in the corresponding msgB successRARs to use a unified msgB format. For the HARQ feedback to the msgB in connected mode where C-RNTI is available the PUCCH resource allocation in DCI is probably fine. An important goal is to avoid two different msgB formats. No matter whether a single UE’s information or multiple UEs’ information are carried in msgB identified by msgB RNTI, the msgB HARQ feedback resource determination should reuse the method for msg4 HARQ feedback. The needed information is TPC command for scheduled PUCCH – 2 bits, PUCCH resource indicator – 3 bits, and PDSCH-to-HARQ_feedback timing indicator – 3 bits. Hence, the number of bits needed is in the order of 8.

PUCCH resource and timing related parameters are put in msgB successRAR.

More details can be found in our RAN1 contribution [1].

## 2.3	HARQ NACK for a MsgA response

In a RAN1 response to LS R2-1911862 (R1-1909955) “LS to RAN1 on RAN2 agreements related to 2-step RACH inform RAN2 of agreements related to MsgB HARQ-ACK:

Agreements:

- A UE shall provide HARQ-ACK feedback if it receives a MsgB that contains a successRAR addressed to this UE.

… Parts omitted …

Agreements:

- For a downlink PDSCH transmission with the corresponding PDCCH addressed to C-RNTI in response to a MsgA with C-RNTI, the HARQ-ACK response to the downlink transmission can include ACK or NACK.

  - In case of ACK, if downlink PDSCH includes TA MAC CE and the time alignment timer is not running, the UE is not expected to transmit the PUCCH before for the TA is applied.

  - A UE can send a NACK only if it time alignment timer is running.

…. Parts omitted …

RAN1 respectfully requests RAN2 to take these agreements in account and to note the following aspects that can impact the MsgB PDSCH design:

- The signalling of the TPC command of each PUCCH resource containing the MsgB’s HARQ-ACK feedback is in the MsgB PDSCH.

- The content of the UL grant in MsgB fallbackRAR follows that of the 4-step RACH RAR.

- Resource and timing signalling for the PUCCH containing the MsgB HARQ-ACK feedback is still under discussion in RAN1.

- The RAN1 design allows the HARQ-ACK to include a NACK for a MsgA response addressed to the C-RNTI when the time alignment timer is running (UE has a valid TA). It is up to RAN2 to consider the usability of such use case in release 16.

…End …

In particular, RAN1 leaves RAN2 to consider the usability of HARQ-ACK to include a NACK for a MsgA response addressed to the C-RNTI when the time alignment timer is running.

According to current Rel-15 behavior, the UE would initiate Random Access in situations for where these conditions are met as a result of either,

1	A pending Scheduling Request with no valid PUCCH resource:

As long as at least one SR is pending, the MAC entity shall for each pending SR:

1>	if the MAC entity has no valid PUCCH resource configured for the pending SR:

2>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR.

2	 As a result of reaching a maximum SR transmission:

2>	if the PUCCH resource for the SR transmission occasion does not overlap with a UL-SCH resource:

3>	if SR_COUNTER < sr-TransMax:

4>	increment SR_COUNTER by 1;

4>	instruct the physical layer to signal the SR on one valid PUCCH resource for SR;

4>	start the sr-ProhibitTimer.

3>	else:

4>	notify RRC to release PUCCH for all Serving Cells;

4>	notify RRC to release SRS for all Serving Cells;

4>	clear any configured downlink assignments and uplink grants;

4>	clear any PUSCH resources for semi-persistent CSI reporting;

4>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel all pending SRs.

3	During Reconfiguration with sync where the UE may perform CFRA or CBRA Random Access depending on if rach-ConfigDedicated is provided by the NW.

The “HARQ-ACK to include a NACK for a MsgA response” in the RAN1 response LS entails that after performing a Random Access including a UE specific RNTI, i.e. C-RNTI, a PDDCH for scheduling msgB (RAR) has been successfully received, but for where the scheduled msgB itself has not been successfully decoded.

If the gNB only operates on HARQ ACK based msgB transmissions and retransmissions, in this scenario upon receiving HARQ ACK, the gNB can deduce whenever a msgB has been successfully decoded, but as a result of an absent ACK (the HARQ resource and timing known), cannot deduce if the UE failed to decode the PDCCH, or successfully decoded the PDCCH but not the RAR in msgB.

In scenarios 1&2 it can be assumed that msgA contain at least the BSR that initially triggered the SR, and hence the gNB should ideally respond with a dynamic grant through a PDCCH scheduled with the C-RNTI. As a result, a msgB payload part (apart from containing an UL grant) would only be transmitted if there would be DL data, e.g. RRC signaling. For the case of no downlink data, which should be most frequent, HARQ NACK would thus not be applicable. The RA procedure should be considered complete at the time of receiving the PDCCH with C-RNTI and UL grant.

If a retransmission of msgB is necessary, another PDCCH for scheduling a repeated msgB RAR is needed and can be sent at earliest in the next PDCCH occasion within the msgB window. The benefit using HARQ NACK in addition to ACK only, would thus only exist if there would be gain in the total procedure delay detecting an absent ACK compared to NACK signaling and if a new transmission of msgB is necessary.

Assuming that msgA PUSCH has been successfully decoded by the gNB, and that a msgB PDSCH is always to be expected, then the msgB PDSCH would include a grant for transmitting the data as indicated by the BSR. By receiving a NACK for this msgB PDSCH, the gNB can also here assume that a retransmission/repeated msgB PDSCH can be substituted by a PDCCH granting the UE for dynamically scheduling the data that triggered the BSR, although this would entail a longer procedure delay.

For case 3, Reconfiguration With Sync, the msgB in response to the RRC connection Setup Request would include the necessary RRC messages to complete the HO procedure. As a result, the msgB content in this case needs to be retransmitted (repeated) if not successfully delivered to the UE. That is, another PDCCH for scheduling a repeated msgB or PDSCH containing the RRC signaling is needed. In this case, it can be assumed that a DL MAC PDU payload part (either as msgB or a DL SCH PDSCH PDU) is required compared to only issuing a grant as in case 1 & 2.

The gNB can in any case schedule the DL at earliest in the next PDCCH occasion within the msgB window for this UE to complete the procedure. It can be assumed that the resource usage for the above variants are equivalent. Also, here the benefit using HARQ NACK in addition to ACK only would thus only exist if there would be gain in the total procedure delay detecting an absent ACK compared to explicit NACK signaling if a new transmission of msgB is necessary.

In any of the above cases, if there is not difference in reliability of detecting DTX vs explicit NACK and if no significant gain in shorter procedure time, an ACK only based mechanism seems sufficient

The time to transmission/grant for a BSR due to SR triggered RA is not necessary to enhance with NACK signaling for msgB in response to MsgA transmissions when the time alignment timer is running.

ACK signaling for a MsgA response addressed to the C-RNTI when the time alignment timer is running is sufficient.

# 3	Conclusion

In section 2 we made the following observations:

Observation 1	The solutions preferred by most companies in the email discussion [107bis#76] [2-step RACH]: MSG B Format Design have at least one unused R-bit.

Observation 2	The unused R bits in the msgB BI subheader could be used to indicate different forms of back off for the 2step procedure.

Observation 3	The time to transmission/grant for a BSR due to SR triggered RA is not necessary to enhance NACK signaling for msgB in response to MsgA transmissions when the time alignment timer is running.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Back off signaling received as a response to msgA transmission does not apply if the UE falls back to 4-step preamble transmission.

Proposal 2	One R-bit in the msgB BI subheader should be used to differentiate if after back off, the UE should continue with msgA transmission or fallback to preamble transmission in the 4-step procedure.

Proposal 3	PUCCH resource and timing related parameters are put in msgB successRAR.

Proposal 4	ACK signaling for a MsgA response addressed to the C-RNTI when the time alignment timer is running is sufficient.

# 4	References

- R1-1912672, Procedure for Two-step RACH, Ericsson, 3GPP TSG-RAN WG1 Meeting #99, Reno, USA, November 18th – 22nd, 2019