3GPP TSG-RAN WG2#108	R2-1915651

Reno, NV, USA, 18th – 22nd November 2019

Agenda Item:	6.8.2.1

Source:	Ericsson, T-Mobile, Nokia, Nokia Shanghai Bell, Deutsche Telekom

Title:	Additional path reporting for downlink NR positioning measurements

Document for:	Discussion, Decision

# Introduction

In RAN1#98, an LS was sent to RAN2 to inform about agreed reference signals, measurements and also some relation to positioning techniques for which the reference signals and associated measurements are intended to facilitate. The positioning techniques are indicated for information purpose as examples and do not intend to influence higher layer signalling to be developed by RAN2. The tables below from the LS provide mappings between reference signals and gNB measurements based on the latest status of the RAN1 discussion.

| DL/UL Reference Signals | gNB Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| Rel.16 SRS for positioning | UL RTOA | UL-TDOA |
| Rel.16 SRS for positioning | UL SRS-RSRP | UL-TDOA, UL-AoA, Multi-RTT |
| Rel.16 SRS for positioning, Rel.16 DL PRS | gNB Rx-Tx time difference | Multi-RTT |
| Rel.16 SRS for positioning, | AoA and ZoA | UL-AoA, Multi-RTT |

| DL/UL Reference Signals | UE Measurements | To facilitate support of the following positioning techniques |
| --- | --- | --- |
| Rel.16 DL PRS | DL RSTD | DL-TDOA |
| Rel.16 DL PRS | DL PRS RSRP | DL-TDOA, DL-AoD, Multi-RTT |
| Rel.16 DL PRS / Rel.16 SRS for positioning | UE Rx-Tx time difference | Multi-RTT |
| Rel. 15 SSB / CSI-RS for RRM | SS-RSRP(RSRP for RRM), SS-RSRQ(for RRM), CSI-RSRP (for RRM), CSI-RSRQ (for RRM), SS-RSRPB (for RRM) | E-CID |

Some of these UE measurements are based on UE time of arrival measurements. As has been discussed during the NR positioning work item [1] and also previously for LTE positioning [2], there are significant benefits from including detected additional paths by the UE. In this contribution, we discuss the necessity to support reporting of additional paths also in NR Rel. 16.

# Discussion

In the Rel. 14 work item on LTE positioning enhancements, rich UE reporting was considered in the scope of RSTD reporting for OTDOA. Significant benefits were observed [2], and additional path reporting was agreed in Rel 14 LPP. Reporting of additional paths has also been investigated during the NR positioning work item [1] and significant benefits have again been observed.

Observation 1	UE reporting of detected additional paths with measurements based on downlink time of arrival significantly improves positioning.

Therefore, it seems obvious that the LTE additional path reporting shall also be supported in NR.

Proposal 1	Support UE reporting of detected additional paths with NR measurements based on downlink time of arrival.

In LTE, the additional paths field is described in the following way, reference cell example:

additionalPathsRef

This field specifies one or more additional detected path timing values for the RSTD reference cell, relative to the path timing used for determining the rstd value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

Hence, absence of additional paths in the reporting means that the UE did not detect any additional paths.

Proposal 2	A UE measurement report not including additional paths for a specific resource is interpreted that the UE did not detect any additional paths.

In LTE, the maximum number of additional paths per cell that could be reported was set to 2. It is reasonable to discuss the maximum number of additional paths to be reported. It could be a fixed number or configurable from the network.

Proposal 3	RAN2 to discuss the maximum additional paths per configured resource the UE supports, and whether the number is fixed, or configurable from the network

# Conclusion

Based on the discussion in section 2 we observe and propose the following:

Observation 1	UE reporting of detected additional paths with measurements based on downlink time of arrival significantly improves positioning.

Proposal 1	Support UE reporting of detected additional paths with NR measurements based on downlink time of arrival.

Proposal 2	A UE measurement report not including additional paths for a specific resource is interpreted that the UE did not detect any additional paths.

Proposal 3	RAN2 to discuss the maximum additional paths per configured resource the UE supports, and whether the number is fixed, or configurable from the network

# References

R1- 1909426 UE and gNB measurements for NR positioning, Ericsson

R1-167663 Multipath RSTD reporting per cell for enhanced indoor positioning, Ericsson