3GPP TSG-RAN WG2#108 meeting	Tdoc R2-1915706

# Reno, USA, 14– 18 November 2019

Agenda Item:		5.4.1.3.11

Souce:	Samsung

Title:	Remaining issues regarding UE assistance information

Document for:		Discussion and decision

# Introduction

RAN2 discussed the following contribution:

R2-1913586	Remaining issues regarding UE assistance information	Samsung, Ericsson	discussion	Rel-15	NR_newRAT-Core

Based on this contribution, RAN2 agreed the following general principles:

At least for NR, Prohibit mechanisms are feature specific, e.g. prohibit timer handling

At least for NR, For UE assistance reporting features use delta as the general approach.

At least for NR, specify that UE repeats UE assistance reported within 1 second prior to change of PCell

For LTE, the intention is to apply this to new features (rel-16 and later)

There was a further offline mainly to discuss from when to apply these general principles and to at least try to agree a CR to NR RRC (LTE CR to be handled at the next meeting). The discussion did however not conclude:

R2-1914211	Report of offline discussion 020 on Correction of UE assistance information	Samsung	report	Rel-15		NR_newRAT-Core

noted

R2-1914161	Correction of UE assistance information	Samsung, Ericsson	draftCR	Rel-15	36.331	15.7.0	F	NR_newRAT-Core

- 	Huawei think there is a compatibility problem. Ericsson think there is no compatibility problem. Nokia also would like to check

- 	Chair; Yes it is ok to be careful

Postpone

# Discussion

After some further discussion with companies that requested more time for checking, we think it should now seems possible to conclude the discussion.

NR RRC

It seems that companies that requested more time for checking are now fine to introduce the general principles from REL-15. There was also some further discussion regarding the UL delay budget reporting and the general view expressed was that:

- UE shall trigger reporting initially and whenever there is a change (i.e. rather than when it wants the DRX to be adjusted)

- UE shall report delay budget information whenever the procedure is triggered for delay budget reporting.

There was also discussion about the need to introduce a means by which UE can indicate it has ‘no preference’, both initially and at a later moment (i.e. reverting back to ‘no prefernce’). Several companies regarded this to be new functionality not explicitly supported and given that it requires signaling changes, something that should not be done within REL-15 anymore.

Altogether we thus think it is now possible to agree the following:

Proposal 1	Use the earlier agreed general UE assistance principles from REL-15 in NR RRC i.e. have a clean start

Proposal 2	For delay budget reporting, at least not within REL-15, do not introduce a means by which UE can indicate it has ‘no preference’

It should noted that the updated CR provided during the previous meeting was already correctly reflecting these proposed agreements.

LTE RRC

One company that requested more time for checking still has concerns to modify UE requirements regarding the start/ restart of prohibit timers. We think existing specification does not reflec the correct/ intended behavior and hence that specification should at least be changed to reflect the correct/ recommended behavior. What might be done additionally is to allow UEs to start/ restart the timers also when UE assistance information is sent for reporting concerning another feature.

Proposal 3	Update LTE RRC to reflect the correct/ recommended handling of prohibit timers. Discuss/ conclude whether to allow in REL-15 that UEs may still start/ restart the timers also when UE assistance information is sent for reporting concerning another feature

The CR RRC was updated to illustrate how we could specif the recommended behaviour while still allowing a UE to support what is currently specified.

# Conclusion & recommendation

This contribution aims to conclude the discussion regarding from when to apply the general principles on UE assistance reporting covering both NR and LTE. The document includes the following proposals that RAN2 is requested to discuss and conclude:

Proposal 1	Use the earlier agreed general UE assistance principles from REL-15 in NR RRC i.e. have a clean start

Proposal 2	For delay budget reporting, at least not within REL-15, do not introduce a means by which UE can indicate it has ‘no preference’

Proposal 3	Update LTE RRC to reflect the correct/ recommended handling of prohibit timers. Discuss/ conclude whether to allow in REL-15 that UEs may still start/ restart the timers also when UE assistance information is sent for reporting concerning another feature

# References

[1] TS 38.331 Radio Resource Control

[2] R2-1915707 Correction of UE assistance information	(CR to 38.331, Samsung)

[2] R2-1915708 Correction of UE assistance information	(CR to 36.331, Samsung)