3GPP TSG-RAN2 Meeting #108	R2-1915714

# Reno, Nevada, 18-22 Nov 2019	Revision of R2-1913393

Agenda item:		6.15.2

Source: 		Qualcomm Incorporated

Title: 		Too strong interference during CLI-RSSI measurement

Document for:	Discussion/Decision

# Introduction

RAN2 was requested by RAN1 in LS R2-1905203 to define CLI-RSSI and SRS-RSRP measurements. Previous RAN2 meetings made significant progress, however some issues are still open.

# Performing CLI-RSSI measurements

In a network with dynamic TDD configuration, strong CLI may occur to a UE if it is close to multiple UEs from another cell that have strong UL transmissions colliding with its DL reception. In this case, the strong interference power may push the UE analog circuit to saturation. For the UE that is configured to measure the CLI-RSSI, in such case the measurement may not be accurate.

If the interference is too strong, the measurement may not be accurate.

RAN2 has previously received LS from RAN4 indicating that too high to measure SRS-RSRP can be included in the measurement report. Similarly, UE should be allowed to indicate in the CLI-RSSI measurement report that the interference is too strong to measure. RAN2 should:

- either agree that too strong interference to measure CLI-RSSI is added to measurement report signalling, or

- ask RAN4 whether this is needed/why this is not needed for CLI-RSSI.

Our preference is, to save on LS traffic and round-trip time, RAN2 should agree that UE should be able to indicate ‘too strong interference to measure’ in CLI-RSSI measurement result.

UE can indicate ‘too strong interference to measure’ in CLI-RSSI measurement results.

# Summary

Based on the above discussion, we have the following observation:

Observation 1.	If the interference is too strong, the measurement may not be accurate.

Based on the discussion and observation, we propose:

Proposal 1.	UE can indicate ‘too strong interference to measure’ in CLI-RSSI measurement results.

# References

- R2-1905203/R1-1903836, RAN1 LS on RRC parameters for NR CLI-handling

- RP-190667, Status Report to TSG, Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR