# 3GPP TSG RAN WG2 Meeting #108      			                           R2-1915746
Reno, USA, 18th – 22nd Nov. 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | xxxx | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.331 on support of F1AP over LTE for IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, KDDI, LG, ZTE, Verizon, KT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  |  | Date: |  | 2019-11-18 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on R2-1913557 Report for [107#51][NR IAB] F1 over LTE (AT&T), RAN2 has decided to specify support for F1AP signalling via LTE for IAB-MT operating in EN-DC mode based on Solution 1a – F1AP interface transported over MT’s RRC. Per Solution 1a, the IAB-MT’s NR RRC acts as a transport layer to deliver the F1AP interface. A new RRC message (IABF1APInformationTransferMRDC) in NR RRC will be defined to encapsulate the IP packet corresponding to the F1AP interface for delivery from NR CU-CP to the IAB-MT via RRC, or vice-versa. When IAB-MT is in EN-DC mode, the NR RRC message will be delivered over X2 interface and to the IAB-MT via LTE RRC. The LTE specifications are agnostic to this delivery of F1 information via NR RRC. Consequently, the only change to 36.331 is to ensure that two-way transfer of NR RRC messages between IAB-MT and NR CU-CP via LTE path is enabled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Per description/discussion in R2-1913557 and RAN2#107bis, the below changes to 36.331 are required: - Define new DLInformationTransferMRDC message for the transfer of a generic NR RRC message via LTE. When IAB-MT is operating in EN-DC mode, this message may be used to transfer the NR RRC F1APInformationTransferMRDC message from CU-CP to IAB-MT. - Add new DL-DCCH-MessageType to DL-DCCH-Message to carry the DLInformationTransferMRDC message. - Enhance field description for existing ULInformationTransferMRDC message to indicate the possible use of ul-DCCH-MessageNR IE to transfer the NR RRC IABF1APInformationTransferMRDC message. Impact Analysis: Impacted functionality: Section 6.2.2 Message definitions: - DL-DCCH-Message - New IABF1APInformationTransferMRDC message - ULInformationTransferMRDC Inter-operability: - If the IAB-MT is implemented according to the CR and the network is not, the network will not be able to utilize the F1AP over LTE feature. - If the network is implemented according to the CR and the IAB-MT is not, the network will not be able to utilize the F1AP over LTE feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | F1AP signalling via LTE for IAB-MT in EN-DC operation can’t be supported in Rel-16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st change

### 6.2.2	Message definitions

. . .

#### –	DL-DCCH-Message

The DL-DCCH-Message class is the set of RRC messages that may be sent from the E-UTRAN to the UE or from the E-UTRAN to the RN on the downlink DCCH logical channel.

-- ASN1START

DL-DCCH-Message ::= SEQUENCE {

message					DL-DCCH-MessageType

}

DL-DCCH-MessageType ::= CHOICE {

c1						CHOICE {

csfbParametersResponseCDMA2000			CSFBParametersResponseCDMA2000,

dlInformationTransfer					DLInformationTransfer,

handoverFromEUTRAPreparationRequest		HandoverFromEUTRAPreparationRequest,

mobilityFromEUTRACommand				MobilityFromEUTRACommand,

rrcConnectionReconfiguration			RRCConnectionReconfiguration,

rrcConnectionRelease					RRCConnectionRelease,

securityModeCommand						SecurityModeCommand,

ueCapabilityEnquiry						UECapabilityEnquiry,

counterCheck							CounterCheck,

ueInformationRequest-r9					UEInformationRequest-r9,

loggedMeasurementConfiguration-r10		LoggedMeasurementConfiguration-r10,

rnReconfiguration-r10					RNReconfiguration-r10,

rrcConnectionResume-r13					RRCConnectionResume-r13,

dlInformationTransferMRDC-r16          	DLInformationTransferMRDC-r16,

spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

End of 1st change

Start of 2nd change

### 6.2.2 Message definitions

. . .

#### –	DLInformationTransferMRDC

The DLInformationTransferMRDC message is used for the downlink transfer of MR DC information.

Signalling radio bearer: SRB2

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

DLInformationTransferMRDC message

-- ASN1START

DLInformationTransferMRDC-r16 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

dlInformationTransferMRDC-r16			DLInformationTransferMRDC-r16-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

DLInformationTransferMRDC-r16-IEs ::=	SEQUENCE {

dl-DCCH-MessageNR-r16			OCTET STRING						OPTIONAL,

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

DLInformationTransferMRDC field descriptions

dl-DCCH-MessageNR Includes the DL-DCCH-Message as defined in TS 38.331 [82]. In case of IAB-MT in EN-DC mode, this field may be used to transfer the NR RRC IABF1APInformationTransferMRDC message.

End of 2nd change

Start of 3rd change

### 6.2.2	Message definitions

. . .

#### –	ULInformationTransferMRDC

The ULInformationTransferMRDC message is used for the uplink transfer of MR DC information (i.e. for the case the SCG employs another RAT e.g. for transferring the NR RRC Measurement Report message).

Signalling radio bearer: SRB1 or SRB2

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

ULInformationTransferMRDC message

-- ASN1START

ULInformationTransferMRDC-r15 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

ulInformationTransferMRDC-r15			ULInformationTransferMRDC-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

ULInformationTransferMRDC-r15-IEs ::=	SEQUENCE {

ul-DCCH-MessageNR-r15			OCTET STRING						OPTIONAL,

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

ULInformationTransferMRDC field descriptions

ul-DCCH-MessageNR Includes the UL-DCCH-Message as defined in TS 38.331 [82]. In this version of the specification, the field is only used to transfer the NR RRC MeasurementReport and the NR RRC FailureInformation messages. In case of IAB-MT in EN-DC mode, this field may be used to transfer the NR RRC IABF1APInformationTransferMRDC message, in which case SRB2 is used.

End of 3rd change