3GPP TSG-RAN WG2 Meeting #108                                    R2-1915906

Reno, NV, USA, November 18 – November 22, 2019

Agenda Item	: 6.7.2.3 (NR_IIOT-Core)

Source	: LG Electronics Inc.

Title	: Discussion on performing ROHC and EHC

Document for	: Discussion and Decision

Introduction

This document discusses which compassion/decompression should be performed first when the EHC and ROHC are simultaneously configured to a PDCP entity.

Discussion

In RAN2#107bis meeting, it was agreed that the EHC and ROHC can be simultaneously configured to a PDCP entity. However, it is still ambiguous which compression should be performed first when the EHC and ROHC are simultaneously configured to a PDCP entity.

In order to correctly perform two compression algorithms in a transmitting PDCP entity, the following options can be considered.

- Option 1 The EHC is performed after performing the ROHC in the transmitting PDCP entity.

- Option 2. The EHC is performed before performing the ROHC in the transmitting PDCP entity.

Figure 1 Example of compression procedure

In figure 1, we illustrate the compression procedure for Option 1 and Option 2. The main difference is that Option 1 performs checking the Ethernet header length before performing the ROHC. Considering that the location of the IP header should be identified to successfully compress the PDCP SDU by ROHC, Option 1 requires checking the Ethernet header length before performing the ROHC. Note that the IP header is located right after at the end of the Ethernet header.

On the contrary, for Option 2, the Ethernet header inspection may not be required because the Ethernet header length can be identified while performing the EHC compression. In other words, the location of the IP header can be identified after performing the EHC compression.

In summary, Option 1 requires the additional complexity to identify the Ethernet header length. Therefore, we propose that the EHC should be performed before performing the ROHC in the transmitting PDCP entity.

Proposal 1. The EHC should be performed before performing the ROHC in a transmitting PDCP entity.

If the proposal 1 is agreeable, RAN2 should discuss which decompression should be performed first when the EHC and ROHC are simultaneously configured to a receiving PDCP entity.

In order to correctly perform two decompression algorithms in a receiving PDCP entity, the following options can be considered.

- Option 1 The EHC is performed after performing the ROHC in a receiving PDCP entity.

- Option 2. The EHC is performed before performing the ROHC in a receiving PDCP entity.

Figure 2 Example of decompression procedure

In figure 2, we illustrate the compression procedure for Option 1 and Option 2. The main difference is that Option 1 performs checking the compressed Ethernet header length before performing the ROHC. This is because the location of the compressed IP header should be identified to successfully decompress the PDCP PDU by ROHC. However, considering that either full header or compressed header can be contained in the compressed Ethernet header, it is hard to identify the compressed Ethernet header length before performing the EHC. Thus, for Option 1, it is needed to know whether the compressed Ethernet header is full header or compressed header without EHC then the compressed Ethernet header length can be identified.

On the contrary, for Option 2, since the EHC is performed before performing the ROHC, there is no reason to check the compressed Ethernet header length. This is because the location of the compressed IP header can be identified after performing the EHC.

Based on the above discussion, we propose that the EHC should be performed before performing the ROHC in a receiving PDCP entity.

Proposal 2. The EHC should be performed before performing the ROHC in a receiving PDCP entity.

With the proposal 1 and proposal 2, the functional view of PDCP can be modified as follows

Figure 4.2.2-1: PDCP layer, functional view

Conclusion

In this contribution, we discuss which compassion/decompression should be performed first when the EHC and ROHC are simultaneously configured to a PDCP entity. Based on the above discussion, we propose followings.

Proposal 1. The EHC should be performed before performing the ROHC in a transmitting PDCP entity.

Proposal 2. The EHC should be performed before performing the ROHC in a receiving PDCP entity.