3GPP TSG-RAN WG2 Meeting #108                                                             	R2-1915944

Reno, USA, 18 – 22 November 2019

Source:	CATT

Title:	Discussion on Configuration of MCG Fast Recovery

Agenda Item:	6.10.5

Document for:	Discussion and Decision

Introduction

In RAN2#107bis meeting, agreement about the configurability of the MCG fast recovery has been made as following:

MCG failure recovery can be configured by the network.

But as for how to configure the MCG failure recovery is not clear, this contribution will discuss the configuration of the MCG fast recovery further.

Discussion

Via system information or dedicated signaling

In order to discuss the configuration of the MCG fast recovery is via system information or dedicated signaling, first we should discuss whether the NW can enable or disable the function of MCG fast recovery for per UE or can only enable or disable the function for the cell.

- Option 1: NW enable/disable the function for per UE

- Option 2: NW enable/disable the function for the cell

As for option 2, the NW will inform UE the cell is enabled or disabled the MCG fast recovery via system information, so if the NW decides to change the function, the NW will update the system information, which will lead in each UE camped on the cell or connected with this cell to perform system information update. However for the UE which is not support the function e.g. for the UE of release 15 or UE configured without DC will update the system information without valid information update. It is a waste for the UE who doesn’t support the MCG fast recovery.

As for option 1, the NW will inform the UE to enable or disable the MCG fast recovery by dedicated signaling. It is flexible from the view point of NW side comparing with option 2. The NW can make the decision based on the type of the service of the UE and the load on the interface or SN, e.g. for the UE with low interruption time requirement who supports the MCG fast recovery, the NW can enable the MCG fast recovery otherwise the NW can disable the MCG fast recovery for the UE to avoid high load on the interface or on the SN.

Observation 1: It is flexible to enable/disable the MCG fast recovery for per UE considering the latency requirement of UE.

As above discussion, Support the MCG fast recovery configured per UE will bring the benefit for NW. the NW can enable or disable the function for one UE by dedicated signaling considering the state of the load and the service of the UE.  It will bring invalid power consumption for the UE who doesn’t support MCG fast recovery or without configuration of DC if the configuration of the function is via system information.

Proposal 1: NW should configure (enable/disable) the MCG fast recovery for per UE via dedicated signaling only.

Impact on inter-node message

The support of the MCG fast recovery is not only related to the MN, but also related to the SN.

If the split SRB1 is configured for the UE, the MN can make the decision whether the MCG fast recovery function can be enabled for the UE because the MCG fast recovery related message can be sent to MN via the SN leg of the split SRB1, the SN doesn’t need to do extra work except for transport the PDCP SDU which include the MCGFailureInformtion to the MN.

But for the case that the UE is configured with SRB3 but without split SRB1, whether the UE can be enabled the function is also related to the ability of the SN. Considering the MCGFailureInformation reported via SRB3 should be encapsulated in the ULInformationTransferMRDC, upon the reception of the ULInformationTransferMRDC, the SN should decode the message to get the container including the MCGFailureInformation and then transfer the container to MN. If the SN can’t decode the message of ULInformationTransferMRDC, the UE can’t recovery the MCG via the SN. So for the case only SRB3 is configured for the UE, whether the MCG fast recovery is supported is based on the capability of both the MN and SN.

As the MN is involved in both the two cases, the decision should be made by MN. But the SN should inform the MN whether the UE can be enabled to perform the MCG fast recovery if SRB3 is configured.

Proposal 2: MN makes the decision and generates the message to enable or disable the MCG fast recovery for a UE.

Proposal 3: SN should inform MN whether the UE can be enabled the MCG fast recovery function if SRB3 is configured.

Unified or separate indication for SRB3 and split SRB1

If the UE is configured with both split SRB1 and SRB3, whether separate indication is needed to inform the UE whether the UE can perform MCG fast recovery via SRB3? i.e. define only one indication to configure the function without the distinguish of split SRB1 or SRB3; define 2 indication to configure the function: one for split SRB1, another for SRB3.

Considering the agreement made last meeting:

Split SRB1 is always used for the transmission of the MCGFailureInformation message. SRB3 is used only if split SRB1 is not configured

That is when the UE is configured with both split SRB1 and SRB3, the UE will perform the MCG fast recovery via split SRB1 not via SRB3, so it is unnecessary to define separate indication for split SRB1 and SRB3. Only one indication is enough.

Proposal 4: Only one indication is needed to inform the UE whether to enable or disable the MCG fast recovery, i.e. separate indications are not needed for SRB3 and split SRB1.

Presence condition

Considering the MCG fast recovery can only be performed when the DC is configured for the UE, so the configuration of the function can only be configured or be valid when the UE is configured with DC. To simplify the specification, the function can only be configured upon or after the DC is configured for the UE.

Proposal 5: The configuration of the MCG fast recovery should only be configured upon or after the configuration of the DC.

Conclusion

In this contribution, we discussed the configuration of the MCG fast recovery, and we propose:

Observation 1: It is flexible to enable/disable the MCG fast recovery for per UE considering the latency requirement of UE.

Proposal 1: NW should configure (enable/disable) the MCG fast recovery for per UE via dedicated signaling only.

Proposal 2: MN makes the decision and generates the message to enable or disable the MCG fast recovery for a UE.

Proposal 3: SN should inform MN whether the UE can be enabled the MCG fast recovery function if SRB3 is configured.

Proposal 4: Only one indication is needed to inform the UE whether to enable or disable the MCG fast recovery, i.e. separate indications are not needed for SRB3 and split SRB1.

Proposal 5: The configuration of the MCG fast recovery should only be configured upon or after the configuration of the DC.

Reference

- RAN2-107bis-Chair-Notes-2019-10-18-1710 EOM;