3GPP TSG-RAN2 Meeting #108	R2-1915955

Reno, Nevada, USA, 18 - 22 November 2019

Agenda Item:	[6.5.3] Segmentation of UE Radio Capabilities

Source: 	Apple Inc

Title:  	UE Radio Capabilities Segmentation Remaining Issues

Document for:	Discussion and Decision

# Introduction

In RAN2#105bis meeting, following agreement were made with regards to the segmentation of UE radio capabilities [1]

Agreements

1	The segmentation mechanism should be defined in ASN.1 in a generic manner so that it could in principle be applied in future to any uplink and downlink RRC message.

2	RAN2 will decides explicitly which RRC messages may be segmented.

2a	Within this WI, segmentation of UE capability information message will be specified.

3	A transmitter (UE or NW) may only use segmentation if it knows that the receiver (NR or UE) comprehends it.

3a	The network indicates in the UECapabilityEnquiry message whether the UE may apply RRC segmentation to its UECapabilityInformation

4	The transmitting entity (UE or NW) uses segmentation only if the unsegmented message exceeds the size limit of PDCP

In RAN2#106 meeting, following agreement were made with regards to the segmentation of UE radio capabilities after offline discussion [2]

Agreements

1    Each uplink message segment carries a segment number

2	Max number segments is 16

3	RAN2 will specify a new UL message type which carries a segment of uplink messages

In RAN2#107 meeting, the followings agreements were reached for segmentation [3].

Agreements

1	The Working Assumption from RAN2#106 is confirmed (i.e. that there will be no interleaving of different RRC messages between segments of the capability information)

Agreements

1	LTE also adopts RRC Segmentation for UE capability information message (following same principles as NR).

# Background

In RAN2#107bis meeting, as per the notes in [4], there were no agreements reached on issues related to RRC processing delay and other aspects related to UE capability segmentation.

In this contribution, we share our views on some of the remaining issues on this topic.

# Discussion

## Maximum time budget for processing RRC message segments

In [6] it was proposed to retain the maximum time budget for processing of RRC message as 80 ms. But this time budget might prove insufficient if we have to consider the worst-case complexity of UE radio capability segmentation, as there are many steps involved while encoding this message. Specifically, it involves

- Determining the raw RF capability (including all UL/DL CA combinations, MIMO capability support etc.)

- Filtering of capability based on the NW request

- Prioritization of Capability

- Finally, UE side segmentation and ASN.1 encoding

Given these steps, and considering the potential maximum size of the UE radio capability message, it is suggested to link the time budget to the size of the UE radio capability message being segmented, and the number of such segments that it is being segmented into.

Observation 1: Maximum time budget for RRC message processing should be a function of the size of the UE radio capability message and hence directly dependent on the number of segments that it is being segmented into.

[5] provided a reasonable time budget for such segmentation.

# Summary

Observation 1: Maximum time budget for RRC message processing should be a function of the size of the UE radio capability message and hence directly dependent on the number of segments that it is being segmented into.

# References

- RAN2#105bis chairman’s notes

- RAN2#106 chairman’s notes

- RAN2#107 chairman’s notes

- RAN2#107bis chairman’s notes

- R2-1912407 - Remaining issues on UE capability segmentation    Huawei, HiSilicon

- R2-1912831 – Remaining issues for RRC segmentation    Ericsson