3GPP TSG-RAN WG2 Meeting#108	R2-1916018

# Reno, USA, 18 – 22 November, 2019

Agenda Item:	6.13.2

Source:	Huawei, HiSilicon

Title:	Further Discussions on Fallback Procedure for 2-step RACH

Document for:	Discussion, Decision

Introduction

In RAN2#107, the following agreement were made [1]:

RAN2#107 Agreements - If the random access procedure is not successfully completed even after transmitting the msgA 'N' times, UE fallbacks to 4 step RACH procedure i.e. UE only transmits the PRACH preamble. - Network can configure the number of times 'N', a UE can attempt to re-transmit msgA during the random access procedure.

There is no further discussion in RAN2#107bis.

In this paper, we further discuss fallback procedure based on the above agreements and provide our considerations.

Fallback procedure

Based on TS 38.321, the random access variables will be initialized when a random access procedure is triggered.

When the Random Access procedure is initiated on a Serving Cell, the MAC entity shall: 1> flush the Msg3 buffer; 1> set the PREAMBLE_TRANSMISSION_COUNTER to 1; 1> set the PREAMBLE_POWER_RAMPING_COUNTER to 1; 1> set the PREAMBLE_BACKOFF to 0 ms;

RAN2 agreed ‘If the random access procedure is not successfully completed even after transmitting the msgA 'N' times, UE fallbacks to 4 step RACH procedure i.e. UE only transmits the PRACH preamble.’

The UE can fallback to 4-step random access if two-step random access failure occurs. When falling back to 4-step random access, the question comes how the RA state variables should be maintained, (e.g. PREAMBLE_TRANSMISSION_COUNTER, PREAMBLE_POWER_RAMPING_COUNTER, PREAMBLE_BACKOFF).

PPREAMBLE_TRANSMISSION_COUNTER

This variable is used to determine whether a random access problem should be indicated to upper layer.

2> if PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax + 1: 3> indicate a Random Access problem to upper layers.

We agreed the UE can switch to four-step random access after the number of 2-step RACH retransmissions exceeds the configured threshold and there is no need to indicate a two-step random access failure to upper layer. The UE continues four-step random access procedure until the whole random access procedure problem (including two-step random access failure and four-step random access failure) occurs. Therefore, the variable counting the number of RACH transmissions should still be maintained when falling back to 4-step random access due to two-step random access failure.

Proposal 1: PREAMBLE_TRANSMISSION_COUNTER is maintained when falling back to 4-step random after the number of 2-step RACH retransmissions exceeds the configured threshold.

PREAMBLE_POWER_RAMPING_COUNTER

This variable is used to determine the transmitting power for preamble transmission.

1> select the value of DELTA_PREAMBLE according to clause 7.3; 1> set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP;

Since the RACH resource (time/frequency/code) for 2-step and 4-step RACH are separately configured, transmission of preamble on different set of RACH resource experience difference level of pathloss, interference etc. Therefore, we think it is reasonable to re-initialize this variable when falling back to 4-step random access due to two-step random access failure.

Proposal 2: PREAMBLE_POWER_RAMPING_COUNTER is initialized when falling back to 4-step random access after the number of 2-step RACH retransmissions exceeds the configured threshold.

PREAMBLE_BACKOFF

This variable is used to determine the delay time to perform RACH resource selection after contention resolution failure or RAR reception failure in four-step random access in R15. After two-step random access failure, the question comes how to proceed the PREAMBLE_BACKOFF.

RAN1 agreed both separated RO and shared RO with different preambles are supported. For both cases, the overload condition between four-step and two-step are separated from the perspective of time/frequnccy/code.

RAN2 also agreed msgB cannot be multiplexed with legacy RAR in four-step random access. Considering the BI indicator in MsgB is used for indicating the level of RACH load in 2-step RACH. Hence, the UE shall ignore the BI indicator in MsgB and initialize this variable when falling back to 4-step random access due to two-step random access failure.

Proposal 3: PREAMBLE_BACKOFF is initialized when falling back to 4-step random access after the number of 2-step RACH retransmissions exceeds the configured threshold.

Conclusion

Based on the discussion we propose the following:

Proposal 1: PREAMBLE_TRANSMISSION_COUNTER is maintained when falling back to 4-step random after the number of 2-step RACH retransmissions exceeds the configured threshold.

Proposal 2: PREAMBLE_POWER_RAMPING_COUNTER is initialized when falling back to 4-step random access after the number of 2-step RACH retransmissions exceeds the configured threshold.

Proposal 3: PREAMBLE_BACKOFF is initialized when falling back to 4-step random access after the number of 2-step RACH retransmissions exceeds the configured threshold.

References

- RAN2#107 chairman note