3GPP TSG-RAN2 Meeting #108	R2-1916080

# Reno, US, 18-22 NOV 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0776 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UE Inactive AS context |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R2 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_5GCN_connect-Core |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Upon cell reselection to another RAT, the UE shall discard the stored UE Inactive AS context and release the rrc-InactiveConfig. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Upon cell reselection to another RAT, specify the UE discard the stored UE Inactive AS context and rrc-InactiveConfig. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: RRC resume functionality Inter-operability: If the network is implemented according to this CR while the UE is not, the UE may use the invalid UE Inactive AS context and rrc-InactiveConfig to perform an RRC resume procedure upon cell reselection back to 5GC from another RAT. If the UE is implemented according to this CR while the network is not, there is no inter-operability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Upon cell relestion to another RAT, the UE does not discard the stored UE Inactive AS context and rrc-InactiveConfig. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.4.11	Cell reselection when storing UE AS context

For UEs storing UE Inactive AS context, rrc-InactiveConfig, UE AS context and resumeIdentity as specified in TS 36.331 [3], upon cell reselection to another RAT, the UE shall discard the stored UE Inactive AS context, rrc-InactiveConfig, UE AS context and resumeIdentity.