3GPP TSG-RAN WG2 # 108	R2-1916083

Reno, USA, November 18th-22nd, 2019

Agenda Item:	6.13.2

Source:	Google

Title:	Discussion on MSGB-RNTI design

Document for:	Discussion, Decision

# Introduction

Agreements: - Introduce preambles group A and B for 2-step RACH. - Apply the same selection formulas to select between 2-step preambles group A and B as specified for 4-step in Rel-15. For the purpose of data threshold, ra-MsgASizeGroupA parameter can be introduced. - Support configuration where fallback from 2-step RA to 4-step RA is not allowed - TB size offered in UL grant in the Msg2 RAR in 4-step RACH shall be the same as the TB size offered for payload transmission in MsgA in 2-step RACH; otherwise, the UE behavior is not defined (i.e. it is up to UE implementation). Rebuilding is not supported in the specification (i.e. it is up to UE implementation). - If switching to 4-step RACH is expected to be supported, then support network configuration where the same TB sizes offered for 2-step RACH preamble groups are the same with those of 4-step RACH preamble groups. - No UE specific RNTI will be designed for 2-step RACH in case CCCH SDU was included in MsgA. - Confirm the Working Assumption: SRB RRC messages of multiple UEs cannot be multiplexed in same MsgB (i.e. same MAC PDU). - RAN2 will work on specifying a new RA-RNTI design for msgB

According to the replay LS from RAN-1 on differentiating MsgB and Msg2, there is no new CORESET configuration for 2-step RACH, thus RAN-2 has agreed to work on specifying a new RA-RNTI design for MsgB.  In this contribution, we discuss the RNTI design for distinguishing msgB and msg2.

# Discussion

## Distinguish between msgB and msg2

Shared RACH occasion for 2-step and 4-step RACH is supported in RAN1. According to the RA-RNTI formula in Rel-15, shared RACH occasion will lead 2-step and 4-step RACH UE to monitor msgB and legacy RAR with the same RA-RNTI. In other words, 2-step RACH UE will have to decode 4-step RACH MAC PDU and 4-step RACH UE will have to decode 2-step RACH MAC PDU. It is redundant because RAN2 has agreed not to multiplex legacy RAR with msgB, furthermore, the UE implementation complexity increases accordingly.

As agreed in RAN1 that all 4-step RACH RO can be shared with 2-step RACH, one feasible solution is to extend the Rel-15 RA-RNTI formula by introducing a parameter ra_id to differentiate the RACH type (2-step or 4-step) as shown in below.

  - RA-RNTI = 1 + s_id + 14×t_id + 14×80×f_id + 14×80×8×ul_carrier_id + 14×80×8×2×ra_id

    - ra_id is the RACH type indicator (0 for 4-step RACH, and 1 for 2-step RACH).

Another way to achieve it is to introduce 2-step RACH specific new RNTI (e.g. MSGB-RNTI) as shown in the Proposal. The formula in the Proposal is identical to the RA-RNTI extension solution above with ra_id equal to 1. The idea for both solutions is to add an offset for the 2-step RACH. Because we allocate the same number of RNTI resources to 2-step and 4-step RACH process, the networks can configure the same range of physical resources (e.g. RACH occasion) to the 2-step and 4-step RACH.

Proposal: Support new RNTI design to distinguish MsgB and Msg2 as

- MSGB-RNTI = 1 + 14×80×8×2 + s_id + 14×t_id + 14×80×f_id + 14×80×8×ul_carrier_id

On the other hand, RAN2 has agreed to use LSB of SFN to solve the response window extension. There is no need to extend RNTI for this reason. Thus, the new MSGB-RNTI along with the legacy RA-RNTI will occupy around half of the RNTI table, which means 30k RNTIs remained in the RNTI table. In our view, for the future enhancement, only IDLE/INACTIVE mode features may use the remaining 30k RNTIs, therefore it is sufficient for the future enhancement.

# Conclusions

Proposal:

Support new RNTI design to distinguish MsgB and Msg2

- MSGB-RNTI = 1 + 14×80×8×2 + s_id + 14×t_id + 14×80×f_id + 14×80×8×ul_carrier_id