3GPP TSG-RAN WG2 Meeting #108			     R2-1916136

Reno, NV, US, 18th - 22th November 2019

Agenda Item:	6.1.3.3 (NR_IAB-Core)

Source: 	LG Electronics Inc.

Title:         	Remaining issues for DL flow control

Document for: 	Discussion and Decision

Introduction

The following agreements for flow control were made in RAN2#107 meeting. This contribution discusses remaining FFSes of DL hop-by-hop flow control for IAB.

The UL end-to-end flow control is not supported in IAB network The DL hop-by-hop flow control is supported in IAB network. One hop DL flow control feedback is considered for DL hop-by-hop flow control, i.e. congested IAB node feedback flow control info to its parent IAB node. DL One-hop flow control feedback should include the IAB node buffer load (details FFS) and flow control granularity info. FFS other information. Per BH RLC channel based flow control feedback can be considered as baseline. FFS on the necessity of other flow control granularity BAP layer supports the DL hop-by-hop flow control and flow control feedback function It is FFS how to trigger the the DL hop-by-hop flow control in IAB network

Discussion

In [1], we discussed the first FFS point which is about other information in flow control feedback.

The next FFS is about the necessity of other flow control granularity, there may be three flow control granularities:

- Per BH RLC channel;

- Per UE radio bearer;

- Per IAB node/BH link.

In per UE radio bearer, it requires that UE bearer ID should be included to each BAP header, but this is not allowed by RAN2 agreements so far and we don’t think this much overhead is really needed. Per IAB node may need the smallest size of flow control feedback, but it may not give enough information about which RLC-channel is actually congested in the IAB node. On the other hand, if per BH RLC channel is used for granularity of flow control feedback, the parent IAB node can recognize which BH RLC channel is congested and it would be helpful to control DL traffic in the parent IAB node. Based on the above explanation, we think the current agreement, i.e., per RLC channel flow control granularity, is sufficient to figure out congestion status and no other granularity is not needed. One clarification on per BH RLC channel option is that per BH RLC channel should be per the ingress BH RLC channel.

Proposal 1. Per BH RLC channel based flow control feedback is sufficient and no other flow control granularity is not needed.

Another discussion point is buffer load. We think that buffer load is not only useful information to indicate seriousness of downlink data congestion to the parent IAB node, but also is used to decide whether to trigger flow control or not. If buffer load exceeds the certain level, the IAB node triggers hop-by-hop flow control feedback to the parent IAB node. For buffer load metric, actual buffer occupancy is sufficient to estimate DL data congestion and other metric is not needed.

Proposal 2. Actual buffer occupancy is used for buffer load metric.

Proposal 3. When the buffer load exceeds the certain level, the DL hop-by-hop flow control feedback should be triggered. No other triggering condition should not be considered in Rel-16 IAB.

Conclusion

In this contribution, we discussed remaining FFSes for DL flow control and present below proposals:

Proposal 1. Per BH RLC channel based flow control feedback is sufficient and no other flow control granularity is not needed.

Proposal 2. Actual buffer occupancy is used for buffer load metric.

Proposal 3. When the buffer load exceeds the certain level, the DL hop-by-hop flow control feedback should be triggered. No other triggering condition should not be considered in Rel-16 IAB.

- References

[1] R2-1916132 Consideration on the postponed decision on DL hop-by-hop flow control	LG Electronics, AT&T, Ericsson, Huawei, Verizon