3GPP TSG-RAN WG2 Meeting #108			 	R2-1916151

Reno, USA, 18th – 22nd November 2019	(Resumbmission of R2-1913919)

Agenda Item	: 6.10.3	 (LTE_NR_DC_CA_enh-Core)

Source	: LG Electronics Inc.

Title	: Further consideration on capability reporting

Document for	: Discussion and Decision

1.	Introduction

In RAN2#107 [1], the following agreements were made.

Agreements Do not support the network provide information on network’s support of CA/DC between frequencies to assist the UE to determine which frequencies to provide NR early measurement in Rel-16.

2.	Discussion

According to [2], due to the lack of network capability whether to support DC and/or CA, it is difficult for a UE (i.e. supporting DC and/or CA with configured frequencies) to determine which frequency for early measurement are to be measured for DC and/or CA with the serving frequency.

For example, a UE may report early measurement results on a frequency based on UE’s CA capability for the frequency combination. Since a network does not know whether the measurement results are intended for CA or for DC, the network may configure DC to the UE, resulting in the incompatible configuration. Furthermore, UE power consumption is wasted for unnecessary measurements.

To solve the problem, we propose that a UE reports the applicable capability information for reported frequencies, whether each reported frequency is applicable for CA or DC with respect to the serving frequency.

Proposal 1: To prevent unnecessary measurement, a UE needs to report applicable capability information.

3.	Conclusion

In this contribution, we provided observations and proposals for capability reporting.

Proposal 1: To prevent unnecessary measurement, a UE needs to report applicable capability information.

4.	References

[1] RAN2#107 chair notes.

[2] R2-1907095, Further consideration on early measurement reporting, ZTE Corporation, Sanechips.