3GPP TSG-RAN2 Meeting # 108	R2-1916309

# Reno, Nevada, USA, 18th – 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0775 | rev | 1 | Current version: | 15.4.0 |  |
| For HELP on using t4is form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on handling of stored AS context for UP optimization and RRC_INACTIVE state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_5GCN_connect-Core, TEI15 |  |  |  |  |  | Date: |  |  | 2019-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Two issues are observed: (1) For RRC Suspend/Resume, in current TS 36.304, the following use case is defined for UE discarding the stored AS context information (see subclause 5.2.4.11): - Inter-RAT cell reselection (from LTE to NR) in RRC_IDLE However, for the following use case, there is currently no corresponding requirement: - The UE is in RRC_IDLE and AS context is stored due to RRC Suspend, and there is a CN type change from EPC to 5GC In this case the UE should also discard the stored AS context information. (2) Currently, the UE behaviour on inter-RAT cell reselection in RRC_INACTIVE state has been captured in TS 38.304, and detailed actions are referring to TS 38.331. However, it is missing in TS 36.304. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are made: (1) Section 5.2.4.11 describes UE behaviour for cell reselection when storing UE AS context, so this section is extended to apply to the following use case: - The UE is in RRC_IDLE and AS context is stored due to RRC Suspend, and there is a CN type change from EPC to 5GC (2) A new section is added in order to describe the UE behaviour on inter-RAT cell reselection or CN type change in RRC_INACTIVE state. For detailed actions, TS 36.331 is referred. Impact analysis Impacted functionality: LTE connected to 5GC, CIoT EPS optimisation Inter-operability: There is no inter-operability issue as this CR is to make alignments between TS 36.331 and 36.304. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may not discard the stored AS context upon CN type change, resulting in potential RRC Resume failure, and the UE behaviour on inter-RAT cell reselection in RRC_INACTIVE state is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.11, 5.2.4.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.2.4.11	Cell reselection or CN type change when storing UE AS context

For UEs storing UE AS context and resumeIdentity as specified in TS 36.331 [3], upon cell reselection to another RAT or upon reselecting to another CN type, the UE shall discard the stored UE AS context and resumeIdentity.

<Next modification>

5.2.4.X	Cell reselection or CN type change in RRC_INACTIVE state

For UE in the RRC_INACTIVE state, upon cell reselection to another RAT or CN type change, UE transitions from RRC_INACTIVE to RRC_IDLE and performs actions as specified in TS 36.331 [3].