3GPP TSG-RAN2 Meeting #108	R2-1916357

# Reno, Nevada, 18-22 November 2019

Title:		[DRAFT] Reply LS on Mobile-terminated Early Data Transmission

Reply to:	Reply LS on Mobile-terminated Early Data Transmission (S2-19108041)

Release:	Release 16

Work Item:	LTE_eMTC5-Core, NB_IOTenh3-Core, 5G_CIoT

Source:	Qualcomm [to be RAN2]

To:		SA2, CT1

Cc:		RAN3, RAN, SA

Contact person:	Umesh Phuyal

uphuyal@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks SA2 for their reply LS on Mobile-terminated Early Data Transmission feature which is being introduced for eMTC and NB-IoT in Rel-16.

SA2 indicated in the LS:

- SA2 has concluded that the UE indicates the capability of supporting MT-EDT via NAS layer. MME can decide whether a PDN connectivity is applied for MT-EDT according to UE capability; SA2 kindly ask RAN2 whether there is concern about this.

RAN2 response:

- From RAN2 point of view, there is no concern to indicate the capability of supporting MT-EDT via NAS layer, and for MME to decide whether a PDN connectivity is applied for MT-EDT according to UE capability.

- It should be possible for the UE to indicate separate capability for CP MT-EDT and UP MT-EDT at NAS level.

- If NAS indicates MT-EDT capability and CN initiates MT-EDT according to the UE capability, there is no need of separate AS capability indication for MT-EDT. Therefore, RAN2 will specify that:

  - CP and/or UP MT-EDT are optional features for UE without AS capability signalling.

  - If the UE supports MT-EDT for the CP solution, it shall also support MO-EDT for the CP solution.

  - If the UE supports MT-EDT for the UP solution, it shall also support MO-EDT for the UP solution.

SA2 also indicated:

- SA2 has assumption that MT-EDT is implemented on top of MO-EDT (i.e. UE will trigger MO-EDT procedure in response to Paging Request with MT-EDT indication).

RAN2 response:

- RAN2 is aiming to specify MT-EDT with minimal changes on top of Rel-15 MO-EDTas follows:

  - For CP case: RAN2 will support ability to include dedicatedInfoNAS in RRCConnectionSetup in response to RRCEarlyDataRequest (i.e. during CP EDT fallback to full RRC connection).

  - For UP case: in response to paging request with MT-EDT indication, in some cases the UE may transmit non-EDT msg1 and receive non-EDT msg2; however, same procedure as in MO-EDT for AS security reactivation and transmission of msg3 is performed for MT-EDT.

2. Actions:

To SA2 and CT1:

ACTION: 	RAN2 respectfully asks SA2 and CT1 to take the above into account and provide feedback/update on MT-EDT, if any.

3. Date of next TSG RAN WG2 meetings:

RAN2#109	24-28 Feb 2020		Athens, Greece

RAN2#109bis	20-24 April 2020	Japan