3GPP TSG RAN WG2 Meeting #108 	R2-1916376

Reno, USA, 18-22 November 2019												revised from R2-1914632

Title:	[107bis#66][NR/NTN]  Running TP (Thales)

Source: 	Thales

Type:	pCR

Document for:	Agreement

Agenda Item:	6.6.1 General

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This document proposes TPs for the TR 38.821 in line with agreements of RAN2#107bis.

[107bis#xx][NR/NTN] Running TP ()

Running TP capturing new agreements in this meeting and removing editor’s notes

Incorporate new pedestrian requirement changes

Intended outcome: Running TP

Deadline:  Thursday 07/11/2019

# Discussion

The running TP is expected to capture

1/ Text proposals from the following TDOC:

R2-1914199	TP on tracking area management in NTN  ZTE, Sanchips, Huawei, Ericsson, LG, CATT, Oppo, Hughes

-	Nokia would like some time to review

=>	The TP will be included in the rapporteur running TP to be reviewed until next meeting

=>	Noted

2/ Incorporate new pedestrian requirement changes:

As part of the NGMN (https://www.ngmn.org/) project on Extreme Long-Range Communications for Deep Rural Coverage, a new pedestrian usage scenario has been identified with increased data rate on the uplink.

Proposal 1: Add a new pedestrian usage scenario with 250 kbps data rate on the uplink

It is suggested to adopt same Overall UE density per km2 for both pedestrian usage scenario

Proposal 2: Consider adopt same Overall UE density per km2 for both pedestrian usage scenario which is 100.

No target peak data rate values have been made available for the study. The experienced data rate are sufficient at this stage.

Proposal 3: Remove the peak data rate columns from the table B.2-1

3/ Proposals to removing editor’s notes

Proposal 4: resolve editors’ notes. See TP.

4/ TPs agreed at RAN2#107bis

The TPs in the following TDOCs: R2-1913969, R2-1914055, R2-1914056, R2-1914069, R2-1914070, R2-1914194, R2-1914196, R2-1914198 were agreed and are incorporated in the draft TR 38.821 v0.9.0 (See draft R3-196330).

5/ Errors in tables 4.2-2 and 7.1-1 regarding max delay difference and max delay variation

Some errors have been discovered in the following tables

- Table 7.1-1: NTN scenarios versus delay constraints, Source [2]

- Table 4.2-2: Reference scenario parameters

Table 7.1-1 defines “Max delay difference within a NTN beam as seen by the UE” while Table 4.2-2 defines “Max differential delay within a cell”. In order to prevent confusion, we propose to suppress the “Max delay difference within a NTN beam as seen by the UE” row in table 7.1-1.

Proposal 5: Remove the “Max delay difference within a NTN beam as seen by the UE” row in table 7.1-1.

Table 4.2-2 defines “Max delay variation within a beam” but the values are in ms instead of a rate like in table 7.1-1 which defines “Maximum Delay variation as seen by the UE”

In order to prevent confusion, we propose to suppress the “Max delay variation within a beam” row in table 4.2-2.

Proposal 6: Remove the “Max delay variation within a beam” row in table 4.2-2

# Conclusion

Proposal 1: Add a new pedestrian usage scenario with 250 kbps data rate on the uplink

Proposal 2: Consider adopt same Overall UE density per km2 for both pedestrian usage scenario which is 100.

Proposal 3: Remove the peak data rate columns from the table B.2-1

Proposal 4: resolve editors’ notes. See TP.

Proposal 5: Remove the “Max delay difference within a NTN beam as seen by the UE” row in table 7.1-1.

Proposal 6: Remove the “Max delay variation within a beam” row in table 4.2-2

Proposal 7: Agree on the text proposals below

# Text Proposal for TR 38.821 v0.9.0

START OF CHANGES

#### 7.3.1.2	Moving Tracking Area for NTN LEO, Scenario C2 and D2 [17]

Moving tracking area means that the tracking area sweeps over the ground as the cells move. Due to high speed movement of satellite, the satellite beam and therefore the cell providing coverage for an earth stationary UE will be changed frequently. As a result, a stationary UE would have to keep performing Registration area update (can also be called as tracking area update, i.e. TAU) in RRC_IDLE state. For each Registration area update, the UE needs to initiate connection with the network. For Rel-15 NR this requires 4-steps of the random access procedure followed by some RRC message exchange over the service link. This can become a non-acceptable overhead if all IDLE mode UEs in the registration area need to perform tracking area update (TAU) frequently as the LEO satellite passes by. If the geographical area covered by the Registration area is large, this issue may become slightly less severe.  However, size of the Registration area and paging capacity forms a trade-off as the UE may need to be paged via all cells belonging to the Registration area and thus the paging capacity may become an issue when network initiated calls arrive.

#### 7.3.1.3	Fixed Tracking Area for NTN LEO, Scenario C2 and D2 [17]

##### 7.3.1.3.1	Approach 1: For the case when UE location information is unavailable

In order not to have TAU performed frequently by the UE triggered by the satellite motion, the tracking area may be designed to be fixed on ground. For NTN LEO, this implies that while the cells sweep on the ground, the tracking area code (i.e. TAC) broadcasted is changed when the cell arrives to the area of next planned earth fixed tracking area location.

The TAC, or a list of TACs, broadcasted by the gNB needs to be updated as the gNB enters to the area of next planned tracking area. When the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the network, a mobility registration update procedure will be triggered.

Figure 7.3.1.3.1-1. An example of updating TAC and PLMN ID in real-time for scenario C2 and D2

As shown in Figure 7.3.1.3.1-1, network update the broadcast TAC in real time according to the ephemeris and confirm the broadcast TAC is associated with the geographical area covered by the satellite beam. UE listens to 
TAI = PLMN ID + TAC and determines to trigger registration area update procedure based on the broadcast TAC and PLMN ID when it moves out of the registration area.

This approach allows to use Rel-15 NR network procedures and can be applied to UE with or without location information.

Two possible options should be studied to update the broadcast TAC:

“hard switch” option: one cell broadcast only one TAC per PLMN. The new TAC replaces the old TAC and there may be some fluctuation at the border area. As shown in Figure 7.3.1.3.1-2, the UE will see its TAC changing like TAC-2-> TAC-1-> TAC-2 from T1 to T3.

Figure 7.3.1.3.1-2. TAC fluctuation at the border area

“soft switch” option: one cell can broadcast more than one TACs per PLMN. The cell adds the new TAC in its system information in addition to the old and removes the old a bit later. If there is a chain of TAs, the TA list adds one TA more and removes one old while the cell sweeps the ground. This also reduces the amount of TAUs for UEs that happen to be located at the border area. However, for the “soft switch” option, the more TACs a cell broadcast, the heavier paging load it experiences, which usually leads to a significant imbalance distribution of paging load among cells. Thus, there is a trade-off between paging load and balancing the fluctuation of actual TA area enabled by the soft switch to be considered in network planning and implementation.

In some area, the gNB may not be able to provide NTN service and thus not broadcast TAC(s).

##### 7.3.1.3.2	Approach 2: For the case when UE location information is available

One possible solution is to divide the earth into a lot of geographical areas and each geographical area is mapped to a certain TAC. During initial registration, UE derives the TAC based on its location information (the mapping rule between the geographical area and the TAC value is kept both on UE side and network side), forms the TAI based on the derived TAC and broadcast PLMN ID and reports the TAI to network via Registration Request message. The AMF confirms the reported TAI and includes a TAI list as a registration area the UE is registered to in the Registration Accept message.

When UE moves to a new geographical area, UE derives the TAC based on the location information and forms the TAI based on the derived TAC and PLMN ID. If UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered to, a mobility registration update procedure will be triggered. UE reports the TAI derived by itself to network via Registration Request message. The AMF confirms the reported TAI and include a new TAI list for the UE in the Registration Accept message. The UE, upon receiving a Registration Accept message, shall delete its old TAI list and store the received TAI list.

##### 7.3.1.3.3	Tracking Area recommendation

Fixed Tracking Area is recommended for the WI phase

END OF CHANGES

START OF CHANGES

# B.2	Performance targets for evaluation purposes

This table includes performance values that may be used for theoretical analysis or simulations.

The values relate to targeted performances, but should not be considered as strict requirements.

Table B.2-1: Non-Terrestrial network target performances per usage scenarios

| Usage scenarios | Peak data rate (note 1) |  | Experience data rate (note 2) |  | Overall UE density per km2 | Activity factor (note 3) | Max UE speed | Environment | UE categories | Sources |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |  |  |  |  |  |  |
| Pedestrian |  |  | 2 Mbps | 60 kbps | 100 | 1,50% | 3 km/h | Extreme coverage | Handheld | Data rate => see 7.10.1 "extreme coverage performance" in [13]; Activity factor: see table 6.1.6-1 "extreme rural" in [13] |
| Pedestrian 2 |  |  | 2 Mbps | 250 kbps | 100 | 1,50% | 3 km/h | Extreme coverage | Handheld | NGMN (https://www.ngmn.org/) project on Extreme Long-Range Communications for Deep Rural Coverage |
| Vehicular connectivity |  |  | 50 Mbps | 25 Mbps | TBD | N.A. | 250 km/h | Along roads in low population density areas | Vehicular mounted | data rate and activity factor in TS 22.261[12] |
| Stationary |  |  | 50 Mbps | 25 Mbps | TBD | N.A. | 0 km/h | Extreme coverage | Building mounted | Data rate =>assuming per end-user 50/25 Mbps data rate and an average of 5 end-user devices per stationary UE) rate and 20% activity factor per end-user device |
| Airplanes connectivity |  |  | 360 Mbps | 180 Mbps | TBD | N.A. | 1 000 km/h | Open area | Airplane mounted | Data rate =>assuming per end-user 15/7.5 Mbps data rate and 20% activity factor per end-user devices (See [12]); number of users per airplane: average aircraft size (assuming 120 users per plane) |
| IoT connectivity (note 4) |  |  | 2 kbps | 10 kbps | 400 | 1,00% | 0 km/h | Extreme coverage | IoT | Device density => [15] ; Data rate and activity factor => derived from [14] annex E.2 "Traffic models for Cellular IoT" |
| Public Safety |  |  | 3.5 Mbps | 3.5 Mbps | TBD | N.A | 100 km/h | Open area | Handheld | Section 7.1 of TS 22.280 (To the extent feasible, it is expected that the end user's experience is similar regardless if the MCX Service is used with a 3GPP network or based on the use of a ProSe direct communication path. Covers pedestrian speed through medium vehicular speeds.) |
| Public Safety |  |  | 3.5 Mbps | 3.5 Mbps | TBD | N.A | 250 km/h | Open area | Vehicle mounted | Section 7.1 of TS 22.280 (To the extent feasible, it is expected that the end user's experience is similar regardless if the MCX Service is used with a 3GPP network or based on the use of a ProSe direct communication path.) |

Note 1: Void

Note 2: As defined in [12]

Note 3: As defined in [12]

Note 4: This refers to low power wide area service capability

Note 5: This does not preclude the definition of performance parameters for other usage scenarios in future stages

Signalling (control plane) and data (User plane) interruption time should be made as much as possible equivalent as in terrestrial systems except in the case of handover between satellite and terrestrial access. In the latter case, this interruption time will depend on the satellite orbit.

END OF CHANGES

START OF CHANGES

# 7	Radio protocol issues and related solutions

END OF CHANGES

START OF CHANGES

## 7.2		User plane enhancements

END OF CHANGES

START OF CHANGES

#### 7.2.1.4	HARQ

END OF CHANGES

START OF CHANGES

### 7.2.2	RLC

All the RLC modes are supported in non-terrestrial networks.

END OF CHANGES

START OF CHANGES

#### 7.2.2.1	Status Reporting

..

No modification of the t-PollRetransmit timer and of the t-statusProhibit timer are needed to support NTN.

#### 7.2.2.2 	RLC Sequence Numbers

END OF CHANGES

START OF CHANGES

#### 7.2.3.1	SDU Discard

…

Modification of the discardTimer can be discussed in the WI phase.

#### 7.2.3.2	Reordering and In-order Delivery

…

Modification of the t-Reordering timer can be discussed in the WI phase.

END OF CHANGES

START OF CHANGES

## 7.3	Control plane enhancements

…

Editor’s note: characteristic of the measurement variations in satellite systems will be added here

Editor’s note: It is agreed that UE location and satellite ephemeris information would be beneficial

END OF CHANGES

START OF CHANGES

#### 7.3.1.6	Using ephemeris information and UE location information

Ephemeris information and UE location information can be used to help UEs perform measurement and cell selection/reselection, in addition to PCI and frequency information included in the broadcast system information. How to deliver and utilize the information can be defined in the WI phase.

END OF CHANGES

START OF CHANGES

### 7.3.2	Connected mode mobility enhancements

For GEO NTN, mobility management procedures require adaptations to accommodate large propagation delay. In particular radio link management may require specific configuration.

For LEO NTN, mobility management procedures should be enhanced to take into account satellite movement related aspects such as measurement validity, UE velocity, movement direction, large and varying propagation delay and dynamic neighbour cell set.

END OF CHANGES

START OF CHANGES

### 7.3.3	Paging issue

END OF CHANGES

START OF CHANGES

END OF CHANGES

START OF CHANGES

#### 8.7.1.1.1	Transparent LEO, Architecture Option 1, different gNBs

…

How to enable cells of two gNB via the transparent LEO satellite can be defined in the WI phase

END OF CHANGES

START OF CHANGES

### 7.1.1	Delay

In order to reduce the standardization work, the table here below identifies the worst case NTN scenarios to be considered for the delay constraint.

Table 7.1-1: NTN scenarios versus delay constraints, Source [2]

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35786 km |  | 600 km |  |  |  |
| Relative speed of Satellite with respect to earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 10° for feeder link |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 3500 km |  | 50 km / 1000 km |  |  |  |
| Maximum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 541.46 ms (Worst case) | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| Minimum propagation delay contribution to the Round Trip Delay on the radio interface between the gNB and the UE | 477.48 ms | 238.74 ms | 8 ms |  | 4 ms |  |
| Maximum Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 40 µs/sec (Worst case) |  | Up to +/- 20 µs/sec |  |
| NOTE 1: The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design, and radio resource management strategy in a given system. NOTE 2: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time when the satellite moves towards/away from the UE. It is expressed in µs/s and is negligible for GEO scenario NOTE 3: Void NOTE 4: Speed of light used for delay calculation is 299792458 m/s. |  |  |  |  |  |  |

END OF CHANGES

START OF CHANGES

## 5.3	Multi connectivity involving NTN-based NG-RAN

### 5.3.1	Overview

END OF CHANGES

START OF CHANGES

## 5.4	Service continuity between NTN and Terrestrial Networks

In 3GPP TS 22.261 (Clause 6.2.3	Service continuity requirements), for a 5G system with satellite access, the following requirements apply:

-	The 5G system shall support service continuity between 5G terrestrial access network and 5G satellite access networks owned by the same operator or owned by 2 different operators having an agreement.

Figure 5.4-1: Typical example of NTN-TN interworking

The NTN and TN could either operate in two different frequency bands (e.g. FR1 vs FR2), or in same frequency band (e.g. FR1 or FR2).

The NTN reference scenarios as listed in chapter 4.2 considers two types of NTN UEs, a) UE with Omni directional antenna, b) UE with directive antenna. For the support of NTN-TN service continuity use cases, assumptions on UE characteristics considering NTN use cases are listed in the below table:

END OF CHANGES

START OF CHANGES

Table 4.2-2: Reference scenario parameters

| Scenarios | GEO based non-terrestrial access network (Scenario A and B) | LEO based non-terrestrial access network (Scenario C & D) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |
| Max channel bandwidth capability (service link) | 30 MHz for band < 6 GHz 1 GHz for band > 6 GHz |  |
| Payload | Scenario A : Transparent (including radio frequency function only) Scenario B: regenerative (including all or part of RAN functions) | Scenario C: Transparent (including radio frequency function only) Scenario D: Regenerative (including all or part of RAN functions) |
| Inter-Satellite link | No | Scenario C: No Scenario D: Yes/No (Both cases are possible.) |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see note 1 Scenario C2: No (the beams move with the satellite) Scenario D 1: Yes (steerable beams), see note 1 Scenario D 2: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (Note 5) | 1000 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) Scenario B: 270.73 ms (service link only) | Scenario C: (transparent payload: service and feeder links) - 25.77 ms (600km) - 41.77 ms (1200km) Scenario D: (regenerative payload: service link only) - 12.89 ms (600km) - 20.89 ms (1200km) |
| Max differential delay within a cell (Note 6) | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27ppm/s (600km) 0.13ppm/s(1200km) |
| User equipment motion on the earth | 1200 km/h (e.g. aircraft) | 500 km/h (e.g. high speed train) Possibly 1200 km/h (e.g. aircraft) |
| User equipment antenna types | Omnidirectional antenna (linear polarisation), assuming 0 dBi Directive antenna (up to 60 cm equivalent aperture diameter in circular polarisation) |  |
| User equipment Tx power | Omnidirectional antenna: UE power class 3 with up to 200 mW Directive antenna: up to 20 W |  |
| User equipment Noise figure | Omnidirectional antenna: 7 dB Directive antenna: 1.2 dB |  |
| Service link | 3GPP defined New Radio |  |
| Feeder link | 3GPP or non-3GPP defined Radio interface | 3GPP or non-3GPP defined Radio interface |

NOTE 1:	Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite

NOTE 2:	Max delay variation within a beam (earth fixed user equipment) is calculated based on Min Elevation angle for both gateway and user equipment

NOTE 3:	Max differential delay within a beam is calculated based on Max beam foot print diameter at nadir

NOTE 4:	Speed of light used for delay calculation is 299792458 m/s.

NOTE 5: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation).

NOTE 6: The maximum differential delay at cell level has been computed considering the one at beam level for largest beam size. It doesn’t preclude that cell may include more than one beam when beam size are small or medium size. However the cumulated differential delay of all beams within a cell will not exceed the maximum differential delay at cell level in the table above.

END OF CHANGES