# 3GPP TSG-RAN2 Meeting #108	R2-1916425

# Reno, Nevada, 18-22 November 2019

Title:		[DRAFT] LS on PUR Fallback

Reply to:	-

Release:	Release 16

Work Item:	LTE_eMTC5-Core, NB_IOTenh3-Core

Source:	Qualcomm [to be RAN2]

To:		RAN1

Cc:		-

Contact person:	Umesh Phuyal

uphuyal@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks RAN1 for their LS on PUR transmission for NB-IoT/eMTC [R1-1911399]. In the LS, RAN1 has indicated the following agreement from RAN1#98bis.

Agreement After data transmission on PUR, if nothing is received by the UE in a PUR search space window, the UE shall fallback to legacy RACH/EDT procedure.

RAN2#108 noted the above RAN1 agreement contradicts with the following RAN2#107 agreement:

Agreements - Fallback after D-PUR transmission is not successful is not specified i.e. it is up to UE implementation to initiate legacy RA, MO-EDT or wait for next D-PUR occasion.

RAN2 would like to respectfully notify RAN1 that RAN2 reconfirms the above RAN2#107 agreement, i.e. if nothing is received by the UE in response to the UL data transmission on PUR, it is left up to UE implementation to initiate legacy RA, MO-EDT or wait for next D-PUR occasion.

2. Actions:

To RAN1:

ACTION: 	RAN2 respectfully asks RAN1 to take the above into account in their further work on PUR.

3. Date of next TSG RAN WG2 meetings:

RAN2#109	24-28 Feb 2020		Athens, Greece

RAN2#109bis	20-24 April 2020	Japan