3GPP TSG-RAN WG2 Meeting #108                                                      R2-1914423

Reno, USA, 18th – 22nd November, 2019

Source:	CATT

Title:	Consideration on msgB-RNTI

Agenda Item:	6.13.2

Document for:	Discussion and Decision

Introduction

In last meeting, msgB reception was discussed in both RAN1 and RAN2. This contribution discusses msgB-RNTI design based on RAN1 and RAN2 agreements.

Discussion

RAN2 considered two options for msgB and Msg2 reception: 1) separate search spaces; 2) separate RNTI values. Based on RAN1 and RAN2 agreements in last meeting, it seems only separate RNTI method left.

In RAN1 LS [1], it is stated that:

Agreements: - For 2-step RACH, no new CORESET for MsgB is defined. - (Working assumption) For the PDCCH associated with MsgB, MsgB is received on the ra-SearchSpace. Agreements: UEs in CONNECTED state use UE specific search space, or common search space to receive the PDCCH associated with MsgA response and with CRC scrambled by the C-RNTI. Therefore, it is up to RAN2 to decide whether or not to use RNTI to differentiate Msg2 vs. MsgB, for which RAN1 respectfully requests RAN2 to inform RAN1 the decision once made.

In RAN2#107bis, the agreements related to msgB reception are:

Agreements:

6. No UE specific RNTI will be designed for 2-step RACH in case CCCH SDU was included in MsgA.

8. RAN2 will work on specifying a new RA-RNTI design for msgB

Agreements:

1. Legacy UEs are not required to decode msgB

So what we need to do is to design msgB-RNTI since both RAN1 and RAN2 consider it is RAN2 work.

To distinguish msgB-RNTI and RA-RNTI, msgB-RNTI value space should be different to RA-RNTI value space. And then legacy UEs don’t need to decode msgB as RAN2 agreement.

The simplest way to get two distinct RNTI value spaces is to deduce msgB-RNTI by adding an offset to the RA-RNTI formula. This solution is adopted in LTE for BL UEs and UEs in enhanced coverage.

msgB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + offset

There are at least two options for the offset value:

1) Offset is equal to the maximum number of RA-RNTI based on the maximum number of parameters in RA-RNTI formula, i.e. 17920 in Rel-15;

2) Offset is equal to the maximum number of RA-RNTI in the cell based on the ROs configured in the cell.

We prefer option 1 because option 1 is simpler, i.e. offset = 17920.

Proposal: RAN2 adopts below formula for msgB-RNTI:

msgB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 17920

Conclusion

This contribution discusses msgB-RNTI design and provide below proposal.

Proposal: RAN2 adopts below formula for msgB-RNTI:

msgB-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 17920

Reference

- R1-1911656, Reply to LS on differentiating between MSG2 and MSGB