3GPP TSG-RAN WG2 Meeting #108	                                                  R2-1914617

Reno, USA, 18th - 22th November 2019

Agenda item:		6.13.2

Source:			Asia Pacific Telecom co. Ltd

Title:		Discussion on 2-step RACH for BFR

Document for:	 	Discussion and decision

# Introduction

At RAN#85, the work item for 2-step RACH was updated to add 2-step CFRA for HO case only, i.e., 2-step CFRA is not supported for BFR. Furthermore, RAN2 confirmed that 2-step CBRA for BFR is supported in last meeting.

RAN2 #107bis Agreement

- 2-step RACH resources can only be configured on SpCell

- The 2-step RACH resources can be configured on a BWP where 4-step CBRA resources are not configured.  In that case we will not have 4-step switch.

- The PDCCH triggered 2-step CFRA RACH will not be supported in Rel-16

- The 2-step CBRA for SpCell BFR is supported in Rel-16.

Based on current running CR [1], when a RA procedure is initiated, the UE will select RA type first before determining the triggering event and performing the corresponding behaviours. However, if the UE selects 2-step RA, the UE will lose the chance to perform CFRA for BFR, i.e. the UE can only perform, 2-step CBRA for BFR. To use dedicated CFRA resource for BFR would be more reliable than using CBRA resource since the NW will have better understanding on candidate beam, and the priority of CFRA is higher than CBRA in R-15.  The performance of beam failure recovery would be degraded if the UE selects 2-step CBRA without determining if CFRA is usable or not. Some criterions are needed to let UE have opportunity to determine if to perform CFRA for BFR before performing 2-step CBRA.

# Discussion

If a UE is configured to perform beam failure detection, the UE would be configured with CFRA resources and candidate beams for beam failure recovery. In R-15, when beam failure is detected, the UE will initiate a RA procedure for BFR. Within RA resource selection, the UE should determine whether to use CFRA for BFR or fallback to perform 4-step CBRA, e.g., based on BFR timer, if CF RA resource for BFR associated with any of the SSB/CSI-RS have been provided, and if any candidate beam is qualified. With the association between the CFRA resource and candidate beam, the NW could have better understanding to change the beam for the UE. Thus, to perform CFRA for BFR is more reliable than CBRA. The 4-step CBRA is more like a fallback mechanism, i.e., the UE only performs 4-step CBRA if CFRA is not available. In other words, the UE should prioritize the CFRA for BFR over 4-step in R-15.

Observation 1: To perform CFRA for BFR is more reliable than CBRA due to the association between the configured candidate beam (SSB/CSI-RS) and the dedicated CFRA resource for BFR

Based on current running CR [1], when a RA procedure is initiated for BFR, the UE will select RA type first before determining whether this is for BFR. Therefore, if the criterions for 2-step RA are satisfied, the UE will select 2-step RACH no matter whether CFRA could be used or not. If the UE selects 2-step RA, the UE will lose the chance to perform CFRA for BFR (since only 2-step CBRA is supported for BFR). The only way for the UE to use CFRA for BFR is to rely on fallback mechanism, i.e., fallback to 4-step RACH. However, to select the 2-step CBRA without any determination for using CFRA would degrade the performance of beam failure recovery. Figure 1 shows the flow chart of current RA procedure for BFR.

Observation 2: To perform 2-step CBRA without any determination for using CFRA would degrade the performance of BFR and waste the CFRA resource for BFR

Figure 1: Current RA procedure for BFR

Considering the performance of BFR, we should stick to the original intention of R-15 BFR procedure, i.e., the priority of CFRA is higher than CBRA. In light of this, option 1 is to further add the criterions of CFRA for BFR before the RA type selection, e.g., within RA procedure initiation section. For example, if a RA is triggered by BFR event, the UE should determine if the criterions of CFRA for BFR are satisfied (e.g., if CFRA resources for BFR have been configured, and if any candidate beam associated with the dedicated CFRA resource is qualified, etc.) before selecting the RA type. The flow chart for the option 1 is shown in Figure 2. When beam failure is detected, the UE will initiate a RA procedure. If the RA procedure is initiated for BFR and the criterions of CFRA for BFR are satisfied, the UE should perform CFRA for BFR. However, if one of the criterions is not satisfied, the UE could perform RA type selection (i.e., to select the RA type based on RSRP of the DL pathloss reference).

Figure 2: Option 1 - UE determines if performing CFRA for BFR before RA type selection

On the other hand, option 2 is to further add the criterions of CFRA for BFR before performing the RA resource selection procedure for 2-step CBRA. The flow chart for the option 2 is shown in Figure 3. When beam failure is detected, the UE initiates a RA procedure and performs RA type selection. No matter whether the UE selects 2-step RA or 4-step RA, the UE should check if the criterions of CFRA for BFR are satisfied (e.g., if CFRA resources for BFR have been configured, and if any candidate beam associated with the dedicated CFRA resource is qualified, etc.). If the UE selects 2-step CBRA and the criterions of CFRA for BFR are satisfied, the UE will switch to perform CFRA for BFR, otherwise the UE will perform 2-step CBRA.

Figure 3: Option 2 - UE determines if performing CFRA for BFR after RA type selection but before performing 2-step CBRA

Proposal 1: If a RA procedure is initiated for BFR, down select one of the following options:

- Option 1 - UE determines if performing CFRA for BFR before RA type selection

- Option 2 - UE determines if performing CFRA for BFR after RA type selection but before performing 2-step CBRA

# Conclusion

Based on the analysis shown aforementioned, we have the following observations and proposal.

Observation 1: To perform CFRA for BFR is more reliable than CBRA due to the association between the configured candidate beam (SSB/CSI-RS) and the dedicated CFRA resource for BFR

Observation 2: To perform 2-step CBRA without any determination for using CFRA would degrade the performance of BFR and waste the CFRA resource for BFR

Proposal 1: If a RA procedure is initiated for BFR, down select one of the following options:

- Option 1 - UE determines if performing CFRA for BFR before RA type selection

- Option 2 - UE determines if performing CFRA for BFR after RA type selection but before performing 2-step CBRA

# Reference

[1] R2-1914798 [107bis#75][2step RACH] Running MAC CR for 2-step RACH (ZTE)