3GPP TSG-RAN WG2 Meeting #107bis	R2-1915025

# Chongqing, China, 14th - 18th October 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0179 | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NR suitable cell for NR re-establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spreadtrum Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 11/5/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In clause 9.2.7, it is specified that UE selects a suitable cell and initiates the RRC Re-establishment Procedure after RLF is declared. But in TS 38.331 clause 5.3.7.3, it is specified that only when UE select a suitable NR cell, the UE will continue to intial the RRC Re-establishment Procedure, and if a inter-RAT cell is selected, the UE will go to RRC_IDLE mode. And in TS 38.304 clause 3, the suitable cell is defined as follow: Suitable Cell: This is a cell on which a UE may camp. For NR cell, the criteria are defined in clause 4.5, for E-UTRA cell in TS 36.304 [7]. So the description of stage 2 is not aligned with stage 3, and this might lead to mis-understanding of the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify and correct that the suitable cell mentioned in clause 9.2.7 is suitable NR cell. Impact analysis Impacted 5G architecture options: Standalone, NR-DC, NE-DC Impacted functionality: Radio Link Failure Inter-operability: If the UE implements this CR and the NW does not, there is no inter-operability issue. If the NW implements this CR and the UE does not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 2 description is not alligned with stage 3, which may lead to confusion and un-proper implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

9.2.7	Radio Link Failure

In RRC_CONNECTED, the UE performs Radio Link Monitoring (RLM) in the active BWP based on reference signals (SSB/CSI-RS) and signal quality thresholds configured by the network. SSB-based RLM is based on the SSB associated to the initial DL BWP and can only be configured for the initial DL BWP and for DL BWPs containing the SSB associated to the initial DL BWP. For other DL BWPs, RLM can only be performed based on CSI-RS.

The UE declares Radio Link Failure (RLF) when one of the following criteria are met:

-	Expiry of a timer started after indication of radio problems from the physical layer (if radio problems are recovered before the timer is expired, the UE stops the timer); or

-	Random access procedure failure; or

-	RLC failure.

After RLF is declared, the UE:

-	stays in RRC_CONNECTED;

-	selects a suitable NR cell and then initiates RRC re-establishment;

-	enters RRC_IDLE if a suitable NR cell was not found within a certain time after RLF was declared.