3GPP RAN WG2 Meeting #108		R2-1919446

Reno, Nevada, USA, 18 - 22 November 2019

Agenda item:	5.4.4

Source:	Apple

Title:	Handling of Fallbacks for Contiguous and Non-contiguous CA in FR2

Document for:	Discussion

# 1	Introduction

During the RAN4#92 meeting, an issue was raised in [1] that there are many combinations of intra-band contiguous and non-contiguous CA configurations, especially in FR2. Generating all possible fallbacks for these combinations adds a huge amount of new combinations, and many fallbacks for the existing combinations are missing in the spec. Thus, it was agreed further by RAN WG4 to add the corresponding clarification to Rel-15 TS 38.101-2 [2] and TS 38.101-3 [3] that "a UE is not required to support all possible fallback combinations but can directly fall back to a single FR2 carrier". In addition to it, RAN WG4 sent an LS [4] asking RAN WG2 to check signalling to avoid mandating support of all possible fallback combinations.

# 2	Fallbacks of contiguous/non-contiguous CA in FR2

## 2.1	Background

In the E-UTRA specs it was introduced that a UE needs to support all fallbacks for the supported CA combinations. The same approach has been generally used also for NR, however, there are many more complicated combinations in NR especially within FR2. Fallback groups have been defined to mitigate this a bit by not mandating fallbacks outside the fallback group. However, there are certain CA combinations in in FR2, which are a combination of contiguous and non-contiguous CA; and these CA combinations can also be part of EN-DC/NE-DC combinations. Examples are all the combinations which have dashes within the bandwidth class part or have multiple blocks of contiguous carriers like CA_n260(2O), CA_n260(H-I), CA_n260(2A-2O-Q). While we have fallback groups for contiguous CA, fallback groups or other limitations of fallbacks are missing for combinations of contiguous and non-contiguous fallbacks. Currently this sentence on fallbacks is in 38.101-3:

“A terminal which supports an inter-band EN-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.”

This sentence does not clearly say how the fallbacks are generated and which combination is considered a fallback. Therefore, it is unclear which fallbacks are to be supported and which not. Also, assuming all combinations as fallbacks that result from removing one carrier after the next results in a huge number of additional combinations, that would need to be supported in the spec and in the UE. In the current specs many of these fallbacks are missing.

One relatively simple example of such a combination is DC_2A_n261(H-I). However, already this simple example generates a fallback tree with 12 fallbacks, as shown in figure 1:

Figure 1: Fallback tree for DC_2A_n261(H-I)

There are much more complicated CA combinations that will create many more combinations like CA_n260(2A-2O-Q) and there are many of these combinations. For CA_n260(2A-2O-Q) figure 2 shows the fallback tree with 46 unique fallback combinations (all duplicates already removed). This combination is already in 38.101, however, most of these  fallbacks are missing. Figure 3 shows a visualization of what CA_n260(2A-2O-Q) means.

Figure 2: Fallback tree for CA_n260(2A-2O-Q)

Figure 3: Visualization of CA combination CA_n260(2A-2O-Q)

Usually, an operator requesting a particular band combination is just interested in the original combination. Of course, there should be a possibility not to use some of the carriers, and this can always be done by deactivating one or more carriers of a combination. Nevertheless, it seems that supporting all the fallback combinations as stand-alone combinations is not needed in real network deployments.

Observation 1a:	Especially FR2 combinations with combined contiguous and non-contiguous intra-band CA generate a huge amount of fallback combinations.

Observation 1b:	Each single of the additional fallback combinations add to the already huge number of CA/DC combinations in 3GPP adding a lot of work for the rapporteurs and needs to be implemented separately in the UE as well as tested for the UE type approval.

Observation 1c:	Fallback FR2 combinations with combined contiguous and non-contiguous intra-band CA seem to be not needed, as long as it is possible to deactivate some of the carriers.

Due to these observations, RAN WG4 discussed and agreed to modify fallback requirements in the specification in such a way that fallbacks of combined contiguous and non-contiguous FR2 intra-band combinations are allowed to directly fallback to bandwidth class "A". As said above, this does not mean that some carriers of the original combination cannot be deactivated, but the fallbacks cannot be used as a stand-alone combination. The corresponding CR were agreed in [2] and [3], in which the following clarification was captured.

"A terminal which supports CA or DC configurations, which include FR2 intra-band CA combinations with multiple subblocks, where at least one of the subblocks consists of a contiguous CA combination is not required to support all fallback combinations but can directly fall back to a single FR2 carrier."

With such change, for FR2 intra-band contiguous and non-contiguous band combinations, gNB should consider the UE only support the reported CA band combination and the one single carrier. In details, gNB can only configure the UE with the reported FR2 intra-band CA combination in RRCReconfiguration message. However, gNB can still perform Scell(s) deactivation to achieve lower order CA after the reconfiguration. It is also worth noting that the corresponding clarification was agreed to both Rel-15 and Rel-16.

Observation 2:	RAN WG4 agreement applies to both Rel-15 and Rel-16.

## 2.2	RAN WG2 specification impact

From the email discussion [5], 5 potential solutions were discussed.

- Option 1: UE reports all the fallbacks that it does not support (i.e. incapability indication)

- Option 2: UE reports the fallbacks it does not support outside the current capability framework (e.g. new procedure or new UE “incapabilities”)

- Option 3: Specific criteria sent with the problematic band combination allowing network to figure out the unsupported cases (the simplest being a 1 bit indicator for a given combination)

- Option 4: RAN4 defines new BW classes for the fallbacks which means that the “NR CA bandwidth class” adds new types (e.g. R, S, T…) to indicate those bandwidth classes which suffer from the restriction that only direct fallback to A is allowed.

- Option 5: Up to NW implementation (e.g. use of masked IMEI-SV)

Regarding Option 1 and Option 2, considering each contiguous and non-contiguous combination can have up to 50 fallbacks which might not be supported, signalling the non-supported fallbacks would increase the amount of fallbacks significantly.

For Option 3, it’s not crystal clear what it is and how it is used. It could be interpreted into: 1) One bit indicating UE does not support all the fallback band combination; or 2) UE to report explicitly all the supported CA band combination for one band combination. Our preference would be interpretation 1) that the UE signals it doesn’t support all fallbacks (bit existence) and can only directly fall back to single carrier.

For Option 4, since the current CA bandwidth class is for one single subblock (i.e., contiguous CA), how to explore it to non-contiguous CA is not clear.

For Option 5, it’s not clear how the NW side can understand that UE does not support certain fallback CA band combinations if no explicit indication is reported. It seems like the NW side can maintain a blacklist of UE(s) not supporting all the fallbacks.

Among these options, we prefer to support interpretation 1) of Option 3 since it is simple to implement and has little impact to RRC spec.

Proposal 1: UE explicitly indicates for each FR2 band combination it doesn’t support all fallbacks and can only directly fall back to single carrier.

Proposal 2: RAN2 to agree on the companion CR(s) to TS38.331 [6] and TS38.306 [7].

# 3	Conclusions

In this contribution, we provide the background of the RAN4 discussion and agreement on handling of fallbacks for combined contiguous and non-contiguous CA in FR2.

Observation 1a:	Especially FR2 combinations with combined contiguous and non-contiguous intra-band CA generate a huge amount of fallback combinations.

Observation 1b:	Each single of the additional fallback combinations add to the already huge number of CA/DC combinations in 3GPP adding a lot of work for the rapporteurs and needs to be implemented separately in the UE as well as tested for the UE type approval.

Observation 1c:	Fallback FR2 combinations with combined contiguous and non-contiguous intra-band CA seem to be not needed, as long as it is possible to deactivate some of the carriers.

Observation 2:	RAN WG4 agreement applies to both Rel-15 and Rel-16.

Proposal 1: UE explicitly indicates for each FR2 band combination it doesn’t support all fallbacks and can only directly fall back to single carrier.

Proposal 2: RAN2 to agree on the companion CR(s) to TS38.331 [6] and TS38.306 [7].

# 4	References

- R4-1908027, "Handling of fallbacks for combined contiguous and non-contiguous CA in FR2", Apple Inc.

- R4-1908028, "CR for Handling of fallbacks for combined contiguous and non-contiguous CA in FR2", Apple Inc., CR TS 38.101-3

- R4-1910238, "CR for Handling of fallbacks for combined contiguous and non-contiguous CA in FR2", Apple Inc., CR TS 38.101-2

- R4-1910239, "LS on handling of fallbacks for combined contiguous and non-contiguous CA in FR2", Apple Inc.

- [107bis#36][NR R15] FR2 CA Fallbacks (Nokia)

- R2-1915447, “CR to TS38.331 on FR2 CA fallback”, Apple

- R2-1915448, “CR to TS38.306 on FR2 CA fallback”, Apple