3GPP TSG-RAN WG2 Meeting #108	R2-1915549

Reno, USA, 18th – 22nd November 2019

Agenda Item:	6.11.1

Source:	MediaTek Inc. (rapporteur)

Title:	Open issues related to the running 38.331 CR

Document for:	Discussion and decision

# 1 Introduction

This document captures the open issues identified from email discussion 107bis#70 on the running RRC CR for power savings.

# 2 Discussion

The following open issues have been identified for resolution:

- Can a UE provide assistance information for cDRX without having a cDRX configuration?

- Are the assistance parameters for cDRX from a configured set of values provided by the NW, or can the UE indicate any value?

- Can the UE indicate that it has no preference for power savings assistance information?

- Can the UE indicate that it does not want to be released, having once sent the release request?

- If the UE has indicated a preferred RRC state in an earlier request, and it does not include a preferred RRC state in a new release request, how does the NW interpret the new message? Does it mean that:

  - the UE has no preference, or

  - the same preference as earlier, or

  - the UE would like to stay in connected mode?

- When referring to the reduced BW (as in overheating), what is it reduced from: a) UE capability or b) current configuration?

# 3 Conclusion

We suggest discussing the following open issues at this meeting:

Proposal 1: RAN2 to decide if the UE can provide cDRX assistance information without a cDRX configuration

Proposal 2: RAN2 to decide if the UE can only indicate cDRX parameters from a preconfigured set of values by the NW, or if the UE can provide any value.

Proposal 3: RAN2 to decide if the UE can provide ‘no preference’ for any of the power saving parameters (cDRX, number of SCells, aggregated BW or number of MIMO layers)

Proposal 4: RAN2 to decide if the UE can indicate that it no longer prefers to leave RRC connected state

Proposal 5: RAN2 to decide on the interpretation of a release request sent without a preferred state: a) no preference, b) same preference as earlier indicated value or c) to stay in connected mode

Proposal 6: RAN2 to clarify what reduced BW (i.e. from overheating) refers to: a) reduced from UE capability or b) reduced from current configuration?