# 3GPP TSG RAN WG2 Meeting #108      			                           R2-1915747
Reno, USA, 18th – 22nd Nov. 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | xxxx | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.331 on support of F1AP over LTE for IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T, KDDI, LG, ZTE, Verizon, KT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  |  | Date: |  | 2019-11-18 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on R2-1913557 Report for [107#51][NR IAB] F1 over LTE (AT&T), RAN2 has decided to specify support for F1AP signalling via LTE for IAB-MT operating in EN-DC mode based on Solution 1a – F1AP interface transported over MT’s RRC. Per Solution 1a, the IAB-MT’s NR RRC acts as a transport layer to deliver the F1AP interface. A new RRC message (IABF1APInformationTransferMRDC) in NR RRC is used to encapsulate the IP packet corresponding to the F1AP interface for delivery from NR CU-CP to the IAB-MT via RRC, or vice-versa. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Per description/discussion in R2-1913557 and RAN2#107bis, the below changes to 38.331 are required: - Define new IABF1APInformationTransferMRDC message for the transfer of F1AP information from the IAB-MT to NR CU-CP, or vice-versa, when IAB-MT is operating in EN-DC mode. - Add new DL-DCCH-MessageType to DL-DCCH-Message to carry the IABF1APInformationTransferMRDC message. - Add new UL-DCCH-MessageType to UL-DCCH-Message to carry the IABF1APInformationTransferMRDC message. Impact Analysis: Impacted functionality: Section 6.2.1 General message structure: - DL-DCCH-Message - UL-DCCH-Message Section 6.2.2 Message definitions: - New IABF1APInformationTransferMRDC message Inter-operability: - If the IAB-MT is implemented according to the CR and the network is not, the network will not be able to utilize the F1AP over LTE feature. - If the network is implemented according to the CR and the IAB-MT is not, the network will not be able to utilize the F1AP over LTE feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | F1AP signalling via LTE for IAB-MT in EN-DC operation can’t be supported in Rel-16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 6.2.1, Section 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st change

### 6.2.1	General message structure

. . .

#### –	DL-DCCH-Message

The DL-DCCH-Message class is the set of RRC messages that may be sent from the network to the UE on the downlink DCCH logical channel.

-- ASN1START

-- TAG-DL-DCCH-MESSAGE-START

DL-DCCH-Message ::=              SEQUENCE {

message                         DL-DCCH-MessageType

}

DL-DCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

rrcReconfiguration              RRCReconfiguration,

rrcResume                       RRCResume,

rrcRelease                      RRCRelease,

rrcReestablishment              RRCReestablishment,

securityModeCommand             SecurityModeCommand,

dlInformationTransfer           DLInformationTransfer,

ueCapabilityEnquiry             UECapabilityEnquiry,

counterCheck                    CounterCheck,

mobilityFromNRCommand           MobilityFromNRCommand,

iabF1apInformationTransferMRDC	IABF1APInformationTransferMRDC,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension   SEQUENCE {}

}

-- TAG-DL-DCCH-MESSAGE-STOP

-- ASN1STOP

End of 1st change

Start of 2nd change

### 6.2.1	General message structure

. . .

#### –	UL-DCCH-Message

The UL-DCCH-Message class is the set of RRC messages that may be sent from the UE to the network on the uplink DCCH logical channel.

-- ASN1START

-- TAG-UL-DCCH-MESSAGE-START

UL-DCCH-Message ::=             SEQUENCE {

message                         UL-DCCH-MessageType

}

UL-DCCH-MessageType ::=         CHOICE {

c1                              CHOICE {

measurementReport               MeasurementReport,

rrcReconfigurationComplete      RRCReconfigurationComplete,

rrcSetupComplete                RRCSetupComplete,

rrcReestablishmentComplete      RRCReestablishmentComplete,

rrcResumeComplete               RRCResumeComplete,

securityModeComplete            SecurityModeComplete,

securityModeFailure             SecurityModeFailure,

ulInformationTransfer           ULInformationTransfer,

locationMeasurementIndication   LocationMeasurementIndication,

ueCapabilityInformation         UECapabilityInformation,

counterCheckResponse            CounterCheckResponse,

ueAssistanceInformation         UEAssistanceInformation,

failureInformation              FailureInformation,

ulInformationTransferMRDC       ULInformationTransferMRDC,

scgFailureInformation           SCGFailureInformation,

scgFailureInformationEUTRA      SCGFailureInformationEUTRA

},

messageClassExtension           SEQUENCE {

c2							CHOICE {

iabF1apInformationTransferMRDC	IABF1APInformationTransferMRDC,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtensionFuture-r16		SEQUENCE {}

}

}

-- TAG-UL-DCCH-MESSAGE-STOP

-- ASN1STOP

End of 2nd change

Start of 3rd change

### 6.2.2	Message definitions

. . .

#### –	IABF1APInformationTransferMRDC

The IABF1APInformationTransferMRDC message is used for the transfer of F1AP information from the IAB-MT to NR CU-CP or vice-versa when IAB-MT is operating in EN-DC mode.

Signalling radio bearer: SRB2

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network and Network to UE

IABF1APInformationTransferMRDC message

-- ASN1START

-- TAG-IABF1APINFORMATIONTRANSFERMRDC-START

IABF1APInformationTransferMRDC ::=               SEQUENCE {

criticalExtensions                          CHOICE {

c1                                          CHOICE {

iabF1APInformationTransferMRDC                   IABF1APInformationTransferMRDC-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

IABF1APInformationTransferMRDC-IEs::=           SEQUENCE {

iabF1APContainer                           	OCTET STRING                    OPTIONAL,

lateNonCriticalExtension                    OCTET STRING                    OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                     OPTIONAL

}

-- TAG-IABF1APINFORMATIONTRANSFERMRDC-STOP

-- ASN1STOP

IABF1APInformationTransferMRDC field descriptions

iabF1APContainer For IAB-MT operating in EN-DC mode, this field contains an IP packet corresponding to the IP layer carrying the F1-C signalling bearer as described in 38.472.

End of 3rd change