3GPP TSG-RAN WG2 Meeting #108	R2-1916094

Reno,  USA, 18 – 22 November 2019

Agenda item:	6.2.2.2

Source:	Charter Communications

Title:	Remaining Issues on Consistent LBT Failure Detection in NRU

WID/SID:	NR-U WID - Release 16

Document for:	Discussion and Decision

# 1	Introduction

Operation in unlicensed spectrum requires conformance to regulatory rules. Particularly, listen-before-talk (LBT) rule ensures that operating nodes do not transmit over each other, hence avoiding collision in a shared spectrum. While LBT is important for operation in unlicensed bands, it affects operation of some of the NR baseline procedures such that it calls for additional attention in case of repetitive LBT failures. In NR-Unlicensed WID [1], the implications that LBT brings has been captured in a few of the objectives.

In RAN2 #105bis meeting [2], the following agreements related to the impact of UL LBT success/fail were reached:

- Adopt a mechanism in MAC spec to handle the UL LBT failure, where “consistent” UL LBT failures (at least for UL transmissions of SR, RACH, PUSCH) are used for problem detection

In RAN2 #107 meeting [3], the following agreements related to consistent UL LBT failure were reached:

- L2 LBT failure mechanism take into account any LBT failure regardless UL transmission type.

- The UL LBT failure mechanism will have the same recovery mechanism for all failures regardless UL transmission type

Baseline Mechanism, further enhancements not precluded:

- A “threshold” for the maximum number of LBT failures which triggers the “consistent” LBT failure event will be used.

- Both a timer and a counter are introduced, the counter is reset when timer expires and incremented when UL LBT failure happens

- The timer is started/restarted when UL LBT failure occur.

- UL LBT failures are detected per BWP

- The UE will report the occurrence of consistent UL LBT failures on PSCell and SCells. The assumption is to reuse SCell failure reporting for BF

In RAN2#107bis meeting [4], RAN2 agreed on the regarding UL LBT failure handling:

- MAC relies on reception of a notification of UL LBT failure from the physical layer to detect a consistent UL LBT failure.

- The UE switches to another BWP and initiates RACH upon declaration of consistent LBT failure on PCell or PSCell if there is another BWP with configured RACH resources.

- The UE shall perform RLF recovery if the consistent UL LBT failure was detected on the PCell and UL LBT failure was detected on “N” possible BWP.   “

- When consistent uplink LBT failures are detected on the PSCell, the UE informs MN via the SCG failure information procedure after detecting a consistent UL LBT failure on “N” BWPs.

- “N” is the number of configured BWPs with configured PRACH resources.   If N is larger than one it is up to the UE implementation which BWP the UE selects.

- When consistent uplink LBT failures are detected on an SCell, a new MAC CE to report this to the node where SCell belongs to is used.  FFS whether the MAC CE can be used to report failure on PCell

In this contribution, we express our views regarding the need for accurate enumeration of LBT failures such that correlated LBT failure events are counted once, hence avoiding early LBT failure declaration.

# 2	Avoiding Correlated LBT failures

Any transmission by an NR-U device requires channel access based on LBT procedure, whose outcome may be LBT success or failure. LBT failure happens when another device, whether an NR-U or another RAT device, engages in transmission. The duration of transmission of the competing devices depend on the type of RAT, and it could be as short as a few NR OFDM symbols or as long as several NR slots. Thanks to the regulatory limitation on maximum channel occupancy time (MCOT), the busy channel duration may not be more than about 5-10 NR slots (in FR1).

To detect a consistent LBT failure event, each occurrence of LBT failure/success may be considered a sample of the channel status, where based on a set of such samples a UE attempts to decide whether a consistent LBT failure event has occurred or not. Therefore, detection of a consistent LBT failure event depends on the following factors:

- How often the LBT failure/success occurrences are observed, in other words how often a UE attempts to perform channel access.

  - Less often sampling may cause a delayed detection of a consistent LBT failure event, if the channel is truly congested.

  - More often sampling may cause detection of a consistent LBT failure event during consecutive transmission by a few neighbour competing devices which may be a transitory event and not a true consistent LBT failure event.

- How correlated the gathered LBT failure/success samples are, in other words whether a significant portion of samples are taken during a single or consecutive channel access by a competing device.

  - More correlated samples may cause a false early detection if most of the samples are taken during an interval where a few neighbouring devices transmit consecutively.

For a more accurate evaluation of the channel status, it is best if the LBT success/failure occurrences are less correlated and obtained as uniformly as possible during a sufficient interval, e.g. during several MCOTs.

Observation 1: For a more accurate evaluation of the channel status, it is best if the LBT success/failure occurrences are less correlated and obtained as uniformly as possible during a sufficient interval, e.g. during several MCOTs.

Given above criteria, the associated timer and counter for consistent LBT failure detection procedure should be configured such that:

- The consistent LBT failure timer is long enough that spans several MCOT durations.

  - The setting of the consistent LBT failure timer may seem to be straightforward, as long as it is configured with a value of several MCOTs. However, depending on how congested the BWP/sub-band(s) are, the consistent LBT failure timer can be adjusted. For instance, in a congested BWP/sub-band(s) a longer consistent LBT failure timer may be more reasonable, to avoid the UE to declare a consistent LBT failure event (and taking the subsequent actions). Note that a congested channel situation should be reflected in channel occupancy (CO) reporting upon which the gNB may decide to initiate a BWP switching for the UE.

- The consistent LBT failure counter does not reach its threshold during the COT(s) of a few neighbouring devices.

  - The setting of the consistent LBT failure counter is less straightforward since some of the UE channel access occurrences are managed by the gNB, some of them are initiated by the UE.

Observation 2: The threshold of the consistent LBT failure counter should be configured by the gNB based on the congestion level of the BWP/sub-band(s).

For a less correlated enumeration of LBT failure occurrences, the events that have occurred during a correlated set of transmissions should be counted as one event. While it may not be straightforward to define a correlated set of transmissions, it is fair to say that a single transmission by a device or a set of transmissions within a COT (as is defined LAA and NRU spec) or TXOP (as is defined in 802.11 spec) can be considered a correlated set of transmissions. Therefore, consistent LBT failure detection becomes more accurate if all LBT failure occurrences during such interval are counted as one event. In order to achieve this, an auxiliary timer may be used to aid MAC to count the LBT failure occurrences properly. Therefore, we suggest to introduce a new timer, with value may be set to the maximum COT duration, which is specified within a regulatory domain, where all LBT failure events before expiration of this timer is counted as one LBT failure event. This timer starts after an LBT failure event, and is not restarted during subsequent LBT failure events unless it has expired.

Observation 3: For a less correlated enumeration of LBT failure occurrences, the events that have occurred during a correlated set of transmissions should be counted as one event. Transmissions within a COT/TXOP can be considered a correlated set of transmissions.

Proposal 1: Introduce a new timer, e.g. lbt-FailurePauseTimer, whose value may be set to the maximum COT duration, where all LBT failure events before expiration of this timer is counted as one LBT failure event. This timer starts after an LBT failure event, and is not restarted during subsequent LBT failure events unless it has expired.

Proposal 2: Adopt the proposed TP to introduce lbt-FailurePauseTimer.

# Proposed TP:

### NRU-UplinkLbtFailureConfig [38.331]

#### –	NRU-UplinkLbtFailureConfig

The IE NRU-UplinkLbtFailureConfig-r16 is used to configure the parameters used for detection of consistent uplink LBT failures on unlicensed spectrum, as specified in TS 38.321 [3].

NRU-UplinkFailureConfig information element

-- ASN1START

-- TAG-NRU-UPLINKLBTFAILURECONFIG-START

NRU-UplinkLbtFailureConfig-r16 ::=                     SEQUENCE {

lbt-FailureInstanceMaxCount                         ENUMERATED {FFSvalue}                              OPTIONAL,   -- Need R

lbt-FailureDetectionTimer                           ENUMERATED {FFSvalue}

OPTIONAL   -- Need R

lbt-FailurePauseTimer                           	ENUMERATED {FFSvalue}

OPTIONAL   -- Need R

}

-- TAG-NRU-UPLINKLBTFAILURECONFIG-STOP

-- ASN1STOP

NRU-UplinkLbtFailureConfig field descriptions

lbt-FailureInstanceMaxCount This field determines after how many consistent uplink LBT failure events the UE triggers uplink LBT failure recovery (see TS 38.321 [3]).

lbt-FailureDetectionTimer Timer for uplink LBT failure detection (se TS 38.321 [3]).

lbt-FailurePauseTimer Timer for avoiding frequent uplink LBT failures during LBT UL failure detection (see TS 38.321 [3]).

### 5.X.2 LBT failure detection and recovery procedure [38.321]

For each activated Serving Cell configured with lbt-FailureRecoveryConfig, the MAC entity shall:

1>	if LBT failure indication has been received from lower layers and if lbt-FailurePauseTimer is not running:

2>	start or restart the lbt-FailureDetectionTimer;

2>	start the lbt-FailurePauseTimer;

2>	increment LBT_COUNTER by 1;

2>	if LBT_COUNTER >= lbt-FailureInstanceMaxCount:

3>	if this Serving Cell is an SCell:

4>	declare consistent LBT failure for the active UL BWP;

4>	indicate to the Multiplexing and assembly entity to include an LBT failure MAC CE in the subsequent uplink transmission.

…

# 3	Conclusions

In this contribution, we highlighted our concerns regarding consequences that LBT could bring in several MAC procedures. We have proposed the following:

Observation 1: For a more accurate evaluation of the channel status, it is best if the LBT success/failure occurrences are less correlated and obtained as uniformly as possible during a sufficient interval, e.g. during several MCOTs.

Observation 2: The threshold of the consistent LBT failure counter should be configured by the gNB based on the congestion level of the BWP/sub-band(s).

Observation 3: For a less correlated enumeration of LBT failure occurrences, the events that have occurred during a correlated set of transmissions should be counted as one event. Transmissions within a COT/TXOP can be considered a correlated set of transmissions.

Proposal 1: Introduce a new timer, whose value may be set to the maximum COT duration, where all LBT failure events before expiration of this timer is counted as one LBT failure event. This timer starts after an LBT failure event, and is not restarted during subsequent LBT failure events unless it has expired.

Proposal 2: Adopt the proposed TP to introduce lbt-FailurePauseTimer.

# References

RP-182878, “New WID on NR-based Access to Unlicensed Spectrum,” Qualcomm, RAN#82, December 2018

RAN2 #105b Chairman notes, Xi’an, China, RAN2#105b, April 2019

RAN2 #107 Chairman notes, Prague, Czech Republic, RAN2#107, August 2019

RAN2 #107b Chairman notes, Chongqing, China, RAN2#107b, October 2019