3GPP TSG-RAN WG2 Meeting #108			     R2-1916134

Reno, NV, US, 18th - 22th November 2019 			    revised of R2-1913818

Agenda Item:	6.1.4.1 (NR_IAB-Core)

Source: 	LG Electronics Inc.

Title:         	BSR MAC CE format for pre-BSR

Document for: 	Discussion and Decision

Introduction

In RAN2#107bis meeting, the way forward for Low-latency scheduling was suggested by chair as shown in below box, but the final decision was postponed. If this way forward is agreed, RAN2 should define a BSR MAC CE format for pre-BSR to achieve the yellow highlight in the box. This contribution discusses BSR MAC CE format for pre-BSR to differentiate between already buffered data and data expected to arrive.

Chair summary, possible compromise way forward: - We will differentiate in BSR available data (as today) and expected data. - Associating a LCH with pre-emptive BSR is left to implementation, unless issues are identified requiring normative solutions. - FFS if SR and BSR generated by a MAC entity need or can only be reported to the parent node where the peer of that MAC entity resides. - On Triggering of pre-emptive BSR, can capture some text similar to the current agreements, in stage-3/2. - Exact timing etc is up to implementation.

Discussion

If the chair’s way forward is agreed, the next step is how to differentiate between already buffered data and data expected to arrive. We think that the easiest way is to introduce a new BSR MAC CE for pre-BSR. More specifically, the existing BSR MAC CE format is reused as much as possible and a new BSR MAC CE can be introduced with small specification work. For example, the following figure is completely same as the existing long BSR MAC CE format except for BS field, i.e., the BS field for pre-BSR contains the amount of expected data. The only remaining work to introduce the new BSR MAC CE for pre-BSR is to assign new LCID value for pre-BSR. Thus, RAN2 doesn’t need to discuss too much time to define the new BSR MAC CE format for pre-BSR and it can be introduced easily as shown in below figure.

Proposal 1. A new BSR MAC CE for pre-BSR should be introduced.

Proposal 2. A new BSR MAC CE format for pre-BSR is completely same as the existing BSR MAC CE format except for the definition of BS field and a new LCID value is assigned.

Conclusion

In this contribution, we discussed BSR MAC CE format for pre-BSR and suggested following proposals:

Proposal 1. A new BSR MAC CE for pre-BSR should be introduced.

Proposal 2. A new BSR MAC CE format for pre-BSR is completely same as the existing BSR MAC CE format except for the definition of BS field and a new LCID value is assigned.