3GPP TSG-RAN WG2 Meeting #108                              R2-1916214

Reno, Nevada, USA, 18th - 22th November 2019

Agenda item:	6.4.5

Source:	Samsung

Title:	MAC PDU handling for Sidelink UE capabilities and Sidelink RRC reconfiguration

Document for:	Discussion & Decision

# Introduction

At RAN2#105-bis meeting, the following agreements were made:

Agreements on PC5-RRC signalling flow:

1: 	Separate RRC messages are defined capability transfer and for AS-layer configuration. FFS on whether the two messages can be transmitted together in the same MAC PDU.

2:	Set the following 2a, 2b and 2c as RAN2 working assumption:

2a:	Do not encapsulate PC5-S message related to link setup into PC5-RRC message for AS-layer configuration.

2b:	PC5-RRC message for AS-layer configuration is not to be sent unprotected, so is not to be sent together with PC5-S messages like Direct Communication Request.

2c:	Do not encapsulate PC5-S message related to link setup into PC5-RRC message for capability information.

This contribution discusses the FFS.

# Discussion

In Uu, two separate RRC messages are allowed to be transmitted in the same MAC PDU. It means that one RRC procedure can be performed along with the other RRC procedure. In this sense, it needs to be clarified whether the sidelink UE capability exchange can be performed along with sidelink RRC reconfiguration procedure for SL unicast. We think there seems no use case to allow such operation. Even if it is allowed, we further think it is just a minor signaling optimization.

Proposal: The RRC message for sidelink UE capabilities and the RRC message for sidelink RRC reconfiguration are transmitted in the different MAC PDU.

# Conclusion

Based on the above, RAN2 is requested to discuss and if possible agree on the following proposal:

Proposal: The RRC message for sidelink UE capabilities and the RRC message for sidelink RRC reconfiguration are transmitted in the different MAC PDU.

# References