3GPP TSG-RAN WG2 Meeting #108                              R2-1916217

Reno, Nevada, USA, 18th - 22th November 2019

Agenda item:	6.4.5

Source:	Samsung

Title:	Discussion on SL-RSRP reporting

Document for:	Discussion & Decision

# Introduction

At RAN1#98-bis meeting, the following agreements were made:

Agreements:

- L3-filtered sidelink RSRP reporting (from RX UE to TX UE) for open-loop power control for PSCCH/PSSCH uses higher layer signaling.

  - Details (e.g., reporting layer, triggering condition, etc.) are up to RAN2.

  - FFS: Other details

Agreements:

- For SL-RSRP measurement for SL open-loop power control, PSSCH DMRS is used

This contribution discusses how to report L3-filtered sidelink RSRP for open-loop power control in SL unicast.

# Discussion

According to RAN1 agreement, Rx UE will apply L3 filtering for SL-RSRP measurements. As LTE V2X, it seems good to define SL-RSRP configuration (i.e. SL-filterCoefficient) separately from Uu measurement configuration.

Proposal 1: Introduce new SL-RSRP configuration (i.e. SL-filterCoefficient) for SL unicast.

For open-loop power control in SL unicast, Tx UE estimates the pathloss based on the SL-RSRP measurements reported by Rx Ux. It means that from a Tx point of view, to set transmit power appropriately, it is important to know difference between previously reported SL-RSRP measurement result and currently reported SL-RSRP measurement result

Proposal 2: Introduce a new event for SL-RSRP reporting i.e. RX UE sends SL-RSRP measurement result to Tx UE if the difference between the current L3-filtered SL-RSRP measurement result and the one in the last SL-RSRP reporting is more than a configured threshold.

Similar to Uu, we think periodic timer-based triggering can be also applied for SL-RSRP reporting.

Proposal 3: Introduce periodic timer based triggering for SL-RSRP reporting i.e. RX UE sends SL-RSRP measurement result to Tx UE upon expiry of the periodic reporting timer for SL unicast.

# Conclusion

Based on the above, RAN2 is requested to discuss and if possible agree on the following proposal:

Proposal 1: Introduce new SL-RSRP configuration (i.e. SL-filterCoefficient) for SL unicast.

Proposal 2: Introduce a new event for SL-RSRP reporting i.e. RX UE sends SL-RSRP measurement result to Tx UE if the difference between the current L3-filtered SL-RSRP measurement result and the one in the last SL-RSRP reporting is more than a configured threshold.

Proposal 3: Introduce periodic timer based triggering for SL-RSRP reporting i.e. RX UE sends SL-RSRP measurement result to Tx UE upon expiry of the periodic reporting timer for SL unicast.

# References