3GPP TSG-RAN WG2 Meeting #108		R2-1916375

Reno, NV, USA, 18th – 22nd November 2019

Title:	LS on RRM Measurement Relaxation for UE Power Saving in NR

Response to:

Release:	Rel-16

Work Item:	NR_UE_pow_sav-Core

Source:	RAN2

To:	RAN4

Contact Person:

#### Name:	Li-Chuan TSENG

E-mail Address:	li-chuan.tseng@mediatek.com

- Overall Description

When designing the RRM measurement relaxation for UE power saving in NR, RAN2 made the following agreements in recent meetings:

Agreements in R2#105bis

1.	Measurement relaxation criteria can consider both low mobility and UE location in the cell (e.g. whether the UE is in cell-edge).

2.	UE may activate relaxed measurement criteria if at least any of the following conditions are met:

a) Serving Cell measurement does not change more than a relative threshold during a time period

-  LTE relaxed monitoring criteria in 36.304 is considered as a baseline.  Additional enhancements to address aspects that are specific to NR can be considered.

b) UE is not a cell edge, meaning that serving cell/beam RSRP/RSRQ/SINR is above a threshold

FFS: Whether neighbour cell RSRP should also be considered.

Agreements in R2#107bis

1. Network can configure the triggering criteria independently (i.e. either cell-edge or low mobility or both)

2. Cell-edge criteria will not consider neighbour cell measurements.

Agreements in R2#108

- Measurement relaxation criteria is evaluated using cell quality only, and we do not define beam-specific conditions for RRM measurement relaxation.

2	For not-at-cell-edge scenario, the thresholds can be based on RSRP and/or RSRQ and is configurable by the network. For low-mobility scenario, the thresholds of the delta are only in relation to RSRP.

3	Whether higher priority frequencies can be relaxed is up to network configuration.  FFS on how the configuration is done.

4	Network configures RRM measurement relaxation via broadcast only; dedicated control is not supported

5	For modifications of low-mobility scenario, TSearchDeltaP less than 5 minutes is configurable in NR.

RAN2 will continue to agree on further stage-3 details for “low mobility” and “not-at-cell-edge”.

- Actions:

To RAN4:

RAN2 respectfully asks RAN4 to take the above agreements and Ran2 progress into consideration.

- Date of next RAN2 Meetings

TSG-RAN WG2 Meeting #109	24 – 28 February 2020	Athens, GR

TSG-RAN WG2 Meeting #109bis			 20 – 24 April 2020	JP