3GPP TSG-RAN2 Meeting #108	R2-1916597

Reno, USA, 18-22 November 2019

Title:	LS on secondary DRX group

Release:	Rel-16

Work Item:	TEI16

Source:	3GPP RAN WG2

To:	3GPP RAN WG1, 3GPP RAN WG4

Contact Person:

#### Name:	Martin van der Zee

Tel. Number:	+46 107151936

E-mail Address:	martin.van.der.zee@ericsson.com

1. Overall Description:

RAN2 discussed a power saving enhancement when both FR1 and FR2 cells are configured via Carrier Aggregation. With this enhancement the FR2 cells can be configured with a separate (and shorter) drx-InactivityTimer and drx-OnDurationTimer compared to the FR1 cells. The length of Long DRX cycle and Short DRX cycle, if configured, are common for both FR1 and FR2. This enhancement enables the FR2 cells to go to sleep more quickly and thereby reduce power consumption as depicted below:

RAN2 reached the following agreements:

Conditional on R1 acceptance:

A separate drx-InactivityTimer and drx-onDurationTimer can be configured for the secondary DRX group. R2 understands that this has zero or almost zero impact in R1 and R4

The combination of cross-carrier scheduling and secondary DRX group is not supported

FFS if timers for FR2 DRX configuration are shorter than timers for FR1 DRX configuration.

The intention is to apply secondary DRX configuration to FR2 and existing DRX configuration to FR1

RAN2 has the intention to introduce this enhancement under TEI16 and would like to check that this has zero or very little impacts to RAN1 and RAN4.

2. Actions:

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to confirm there is no RAN1 impact, or little impact that is acceptable.

To RAN4

ACTION: 	RAN2 respectfully asks RAN4 to confirm there is no RAN4 impact or little impact that is acceptable.

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #109		24 - 28 February 2020			Athens, Greece

TSG RAN2 Meeting #109bis	20 – 24 April 2020			TBD