3GPP TSG-RAN WG2 Meeting #109 electronic	R2-2000315

24 Feb – 6 Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | <CR#> | rev | - | Current version: | -.-.- |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of dormant BWP operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2020-01-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To enable the fast SCell activation, the dormant BWP and related opertaion is introduced in R16 in CA/DC enhacment work item. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add dormant BWP and related opertaion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | the dormant BWP and related opertaion are missing in stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Based on 38.300 runing CR R2-1916519, see the highlight in green. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

The first of change

10	Scheduling

10.6	Activation/Deactivation Mechanism

To enable reasonable UE battery consumption when CA is configured, an activation/deactivation mechanism of Cells is supported. When an SCell is deactivated, the UE does not need to receive the corresponding PDCCH or PDSCH, cannot transmit in the corresponding uplink, nor is it required to perform CQI measurements. Conversely, when an SCell is active, the UE shall receive PDSCH and PDCCH (if the UE is configured to monitor PDCCH from this SCell) and is expected to be able to perform CQI measurements. When an SCell is active and in dormancy, the UE stops monitoring PDCCH on SCell but continue performing CSI measurements, AGC and beam management, if configured. NG-RAN ensures that while PUCCH SCell (a Secondary Cell configured with PUCCH) is deactivated, SCells of secondary PUCCH group (a group of SCells whose PUCCH signalling is associated with the PUCCH on the PUCCH SCell) should not be activated. NG-RAN ensures that SCells mapped to PUCCH SCell are deactivated before the PUCCH SCell is changed or removed.

When reconfiguring the set of serving cells:

-	SCells added to the set are initially activated or deactivated;

-	SCells which remain in the set (either unchanged or reconfigured) do not change their activation status (activated or deactivated).

At handover or connection resume from RRC_INACTIVE:

-	SCells are activated or deactivated.

To enable reasonable UE battery consumption when BA is configured, only one UL BWP for each uplink carrier and one DL BWP or only one DL/UL BWP pair can be active at a time in an active serving cell, all other BWPs that the UE is configured with being deactivated. On deactivated BWPs, the UE does not monitor the PDCCH, does not transmit on PUCCH, PRACH and UL-SCH. In addition, for each SCell, one DL BWP can be configured as a dormant BWP (i.e. a BWP without PDCCH configuration).

To enable fast SCell activation when CA is configured, one dormant BWP can be configured for a SCell. If the active BWP of the activated SCell is dormant BWP, the UE stops monitoring PDCCH on SCell but continue performing CSI measurements, AGC and beam management, if configured. The DCI is used to control entering/leaving dormant BWP for one or more SCell group(s), or one or more SCell(s).

The dormant BWP is one of UE dedicated BWPs configured by netowork via dedicated RRC signalling.

The SpCell and PUCCH SCell can not be configured with dormant BWP.

Editor's Note: Dormant UL BWP is FFS.

Editor's Note: Details of UE behavior regarding dormant BWP is FFS.

The end of change