3GPP TSG-RAN WG2 Meeting #109	R2-2000353

# Athens, Greece, 24 February – 6 March 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1440 | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the PLMN-IdentityInfoList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-RAN-Core |  |  |  |  |  | Date: |  |  | 2020-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS36.331, it has been specified that one PLMN can be included in only one entry of the cellAccessRelatedInfoList or cellAccessRelatedInfoList-5GC. -------------------------------------TS36.331-start--------------------------------------------- SystemInformationBlockType1-v1430-IEs ::= SEQUENCE { Partlyomitted cellAccessRelatedInfoList-r14 SEQUENCE (SIZE (1..maxPLMN-1-r14)) OF CellAccessRelatedInfo-r14 OPTIONAL, -- Need OR Partlyomitted } SystemInformationBlockType1-v1530-IEs ::= SEQUENCE { Partlyomitted cellAccessRelatedInfo-5GC-r15 SEQUENCE { cellBarred-5GC-r15 ENUMERATED {barred, notBarred}, cellBarred-5GC-CRS-r15 ENUMERATED {barred, notBarred}, cellAccessRelatedInfoList-5GC-r15 SEQUENCE (SIZE (1..maxPLMN-r11)) OF CellAccessRelatedInfo-5GC-r15 } OPTIONAL, -- Need OP Partlyomitted } Partlyomitted CellAccessRelatedInfo-r14 ::= SEQUENCE { plmn-IdentityList-r14 PLMN-IdentityList, trackingAreaCode-r14 TrackingAreaCode, cellIdentity-r14 CellIdentity } CellAccessRelatedInfo-5GC-r15 ::= SEQUENCE { plmn-IdentityList-r15 PLMN-IdentityList-r15, ran-AreaCode-r15 RAN-AreaCode-r15 OPTIONAL, -- Need OR trackingAreaCode-5GC-r15 TrackingAreaCode-5GC-r15, cellIdentity-5GC-r15 CellIdentity-5GC-r15 } -------------------------------------TS36.331-end---------------------------------------------- In TS38.331, the PLMN-IdentityInfoList is introduced, which contains a PLMN identity list and the corresponding TAC, ranac, cell identity and the cellReservedForOperatorUse indicator. However, we do not have the limitation that one PLMN can be included in only one entry of the PLMN-IdentityInfoList and thus it is not clear whether it is allowed for the network to include one PLMN in more than one entry of the PLMN-IdentityInfoList. As specified in TS38.300, “NR access provides only one TAC and one Cell Identity per cell per PLMN” and there is no clear use case to configure more than one ranac or cellReservedForOperatorUse for one PLMN. Having one PLMN in more than one entry of the PLMN-IdentityInfoList will cause duplication and should be avoided. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the limitation that “one PLMN can be included in only one entry of the PLMN-IdentityInfoList”. Impact Analysis Impacted 5G architecture options: SA, NE-DC, NR-DC Impacted functionality: System information Inter-operability: 1. If the network is implemented according to the CR and the UE is not, no inter-operability issue is foreseen. 2. If the UE is implemented according to the CR and the network is not, no inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear whether it is allowed for the network to include one PLMN in more than one entry of the PLMN-IdentityInfoList. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 6.3.2	Radio resource control information elements

=== Unmodified sections omitted ===

#### –	CellAccessRelatedInfo

The IE CellAccessRelatedInfo indicates cell access related information for this cell.

CellAccessRelatedInfo information element

-- ASN1START

-- TAG-CELLACCESSRELATEDINFO-START

CellAccessRelatedInfo   ::=         SEQUENCE {

plmn-IdentityList                   PLMN-IdentityInfoList,

cellReservedForOtherUse             ENUMERATED {true}  OPTIONAL,            -- Need R

...

}

-- TAG-CELLACCESSRELATEDINFO-STOP

-- ASN1STOP

CellAccessRelatedInfo field descriptions

cellReservedForOtherUse Indicates whether the cell is reserved, as defined in 38.304 [20]. The field is applicable to all PLMNs.

plmn-IdentityList The plmn-IdentityList is used to configure a set of PLMN-IdentityInfoList elements. Each of those elements contains a list of one or more PLMN Identities and additional information associated with those PLMNs. The total number of PLMNs in the PLMN-IdentityInfoList does not exceed 12. One PLMN can be included in only one entry of the PLMN-IdentityInfoList. The PLMN index is defined as b1+b2+…+b(n-1)+i for the PLMN included at the n-th entry of PLMN-IdentityInfoList and the i-th entry of its corresponding PLMN-IdentityInfo, where b(j) is the number of PLMN-Identity entries in each PLMN-IdentityInfo, respectively.

End of change